3GPP TSG-RAN WG4 Meeting #116	R4-2510963

Bengaluru, IN, August 25th – 29th 2025

Agenda Item:	7.21.3.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD.

Discussion on existing BS RF RX requirements for SBFD, new requirements for SBFD, and SBFD general aspects are detailed in separate submissions [1-3].

# 2	Discussion

Test model

In RAN4#113, it was agreed to reuse the existing FRCs for conformance testing of the SBFD UL subband. While reusing the existing FRCs for UL subbands is a logical starting point, DL test models must be designed for scalability. Current DL test models (e.g., as shown in Table 4.9.2.2-1 TS 38.104 [4]) do not consider flexible UL subband configurations, which are essential for SBFD operation. Therefore, a scalable test model for the DL subband, which is parameterized by the UL subband bandwidth, UL subband center frequency or its relative offset from the DL, etc., is needed to accurately evaluate SBFD performance under scalable conditions.

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | Value |  |  |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 | 5 | 5 |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

- The test model for SBFD DL subbband needs to be scalable.

- New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

In addition, an important technical concern is the impact of scalable test models on signal statistics. As DL subbands are scaled (e.g., by varying bandwidth or power), their signal statistics may change, potentially affecting the PA testing. To ensure accurate and comprehensive testing, quantitative assessment of signal statistics variations under different subband configurations/DL subband sizes and power levels.

- Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The test model for SBFD DL subbband needs to be scalable.

Observation 2	New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

Observation 3	Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed.

# References

R4-2510964, Impact on SBFD BS RF RX requirements, Ericsson

R4-2510962, New SBFD BS RF requirements, Ericsson

R4-2510961, SBFD general aspects, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP