3GPP TSG-RAN WG4 Meeting #116	R4-2510967

Bengaluru, IN, August 25th – 29th 2025

Agenda Item:	7.9.4.2.1

Source:	Ericsson

Title:	OTA test scope reduction

Document for:	Approval

# 1	Introduction

In Rel-19, there are two requirements identified as the most time consuming and should be prioritized, i.e., TX IM and RX out-of-band blocking, on which our view focusing on FR1 BS type 1-O is shared in the following section.

# 2	Discussion

The transmitter intermodulation and receiver out-of-band blocking requirements are the most time-consuming tests due to their extensive frequency range. These tests involve numerous measurement points and test cases, leading to prolonged testing durations and increased costs. The following text provides further analysis on the open issues listed in the WF [1].

Issue 1-1-1: test scope reduction for OBUE - Agreement: - Further discuss how to reduce the test simplification for OBUE limits. - Option 1: 1xCBW offset - Option 2: 3xCBW offset - Option 3: 3xCBW offset if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands and 1xCBW and 2xCBW if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE Issue 1-1-2: test scope reduction for spurious emission - Agreement: - Option 1: 3xCBW offset Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz - Option 3: mixed EMC and option 1 approach, FFS on the details. - Option 4: To use 2/3 of the test signal configuration channel bandwidths - Option 5: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz. - Other options are not precluded

2.1	Transmitter Intermodulation (TX IM)

OBUE

The OTA operating band unwanted emission requirement for BS type 1-O is that for each applicable basic limit for BS type 1-H in clause 6.6.4.2, the power of any unwanted emission shall not exceed an OTA limit specified as the basic limit + X, where X = 9 dB.

Unlike ACLR and transmitter spurious emissions, OBUE is more complex and has varying limit levels. During RAN4#115, three reduction options were discussed for the OBUE test case, and we intend to support Option 3. However, we have concerns that in some situations, the proposed reduction method may not be applicable.

For instance, referring to Table 6.6.4.2.1-2 and Table 6.6.4.2.2.1-1 in TS 38.104 [2], we can observe that the basic limits differ across frequency ranges. As a result, it is difficult to determine which interferer frequency offset represents the worst-case scenario. From a performance criteria perspective, the lowest limit for Category A occurs in the second segment, while for Category B, it is in the third segment. It is unclear whether the 3rd frequency offset can be considered the worst-case scenario. Therefore, in this case, the test reduction method described in Option 3 should not be applied.

Table 6.6.4.2.1-2 in TS 38.104 [2]: Wide Area BS operating band unwanted emission limits 
(NR bands above 1 GHz) for Category A

| Frequency offset of measurement filter -3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 5 MHz | 0.05 MHz £ f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz £ Df < min(10 MHz, Dfmax) | 5.05 MHz £ f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz £ Df £ Dfmax | 10.5 MHz £ f_offset < f_offsetmax | -13 dBm (Note 3) | 1MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is Df ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be 13 dBm/1 MHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when Dfmax < 10 MHz. |  |  |  |

Table 6.6.4.2.2.1-1 in TS 38.104 [2]: Wide Area BS operating band unwanted emission limits for above 3 MHz channel bandwidth(NR bands below 1 GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band, the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 10MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -16 dBm/100 kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. NOTE 3: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

For OBUE, from a performance criteria perspective, the lowest limit for Category A is found in the second segment, while for Category B, it is in the third segment. In the case of Category A, it is difficult to consider the 3rd frequency offset as the worst-case scenario.

Apply Option 3 for the TX IM test reduction of the OBUE test case, but do not apply it when the third segment is not the lowest limit, such as in the case of Category A WA BS.

2.2	Receiver Out-of-Band Blocking

The out-of-band blocking test is conducted across a wide frequency range 30 – 12750 MHz for FR1, with a 1 MHz step size. This small step size results in a large number of measurement points and prolonged testing durations and increased costs. Therefore, it is essential to find an approach, e.g., increasing the measurement step size, to reduce the excessive testing meanwhile maintain the requirement coverage and ensure the receiver’s ability is well examined, i.e., the increased step size needs to provide sufficient granularity.

From testing perspective, OTA receiver out-of-band blocking is similar to EMC radiated immunity. A summary of EMC radiated immunity requirement captured in TS 38.113 [3] is as follows: the test frequency range is 80 – 6000 MHz with the exception of the operating band for receivers, the test level is 3 V/m, 80% amplitude modulation with 1 kHz. The stepped frequency increments shall be 1% of the momentary measurement frequency, which refers to clause 8.4 in IEC 61000-4-3 [4].

The momentary frequency refers to the current frequency value being used at a specific step during the test.

Example:

If the current test frequency is 100 MHz,

- step size: 1% of 100 MHz = 1 MHz, so the next measuring frequency will be 101 MHz, and then

- next step size: 1% of 101 MHz = 1.01 MHz, so the next measuring frequency will be 102.01 MHz.

- ...

This results in logarithmic spacing rather than linear spacing, which helps in more consistent coverage across a wide frequency range and ensures a uniform testing for all BSs.

Using 1% of the momentary measurement frequency as measurement step size ensures a uniform testing for all BSs.

Conducted receiver out-of-band blocking test is different from EMC radiated immunity test, because the inference is injected via the connector for OOBB test while the enclosure of BS is exposed to the external inference for EMC radiated immunity test. However, for AAS when the antenna is integrated to the radio, it is hard to differentiate the two tests except that the EMC test level is much higher. The exact comparison is as shown in Table 1. Considering the resemblance of the two tests, and the current testing of OTA receiver out-of-band blocking is over 30 – 12750 MHz for FR1, with a 1 MHz step size, we recommend using 1% of the momentary frequency as the testing step size for frequencies between 100 MHz and 6000 MHz. For frequencies higher than 6000 MHz, use a fixed step size of 60 MHz. For frequencies lower than 100 MHz, use a fixed step size of 1 MHz.

Table 1 Comparison between FR1 OTA Receiver Out-of-Band Blocking and EMC Radiated Immunity

| Test | OTA Receiver Out-of-Band Blocking | EMC Radiated Immunity |
| --- | --- | --- |
| Test Frequency Range | 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz | 80 – 6000 MHz (excluding receiver operating band) |
| Test Level | 0.36 V/m | 3 V/m |
| Modulation | CW | 80% AM with 1 kHz tone |
| Frequency Step | 1 MHz | 1% of momentary measurement frequency |
| Performance Criteria | Throughput shall be ≥ 95% of the maximum throughput | Throughput > 95 %, no loss of service |
| Standard Reference | TS 38.104 [2] | TS 38.113 [3] and IEC 61000-4-3 [4] |

From testing perspective, OTA receiver out-of-band blocking is similar to EMC radiated immunity.

There is concern that the step size might be too large, causing the frequency of the unwanted mixing product ±3·fCW±n·finternal might not be able to fall within receiver bandwidth. Therefore, the step size for the interfering signal should be less than 1/3 of the BW. While this approach makes sense to some extent, it is important to consider that, given the starting point of 30 MHz, many test points become irrelevant when the frequencies are far from the operating band, especially for mid-band base stations.

As a result, we support a mixed approach combining Option 1 and Option 5, i.e., use 1% of the momentary frequency as the testing step size for frequencies between 100 MHz and 6000 MHz. For frequencies above 6000 MHz, use a fixed step size of 60 MHz. For frequencies between 30 MHz and 100 MHz, use a fixed step size of 1 MHz. On top of that, apply 1/3 of the minimum supported bandwidth in the following frequency ranges: 100% of operating bandwidth below the lower edge of the operating band (extending downwards from the lower edge of the band), 100% of operating bandwidth above the upper edge of the operating band (extending upwards from the upper edge of the band), and between 1/3 of the frequency at the lower edge and 1/3 of the frequency at the upper edge of the operating band.

For FR2 out-of-band blocking test ranging from 30 – 6000 MHz, the step size is also 1 MHz. Similar test reduction work would be needed for FR2 as well.

When the frequencies are closer to the operating band, a denser set of test points may be required.

For FR2 out-of-band blocking test ranging from 30 – 6000 MHz, the step size is also 1 MHz. Similar test reduction work would be needed for FR2.

Use 1% of the momentary frequency as the testing step size for frequencies between 100 MHz and 6000 MHz. For frequencies above 6000 MHz, use a fixed step size of 60 MHz. For frequencies between 30 MHz and 100 MHz, use a fixed step size of 1 MHz. On top of that, apply 1/3 of the minimum supported bandwidth in the following frequency ranges: 100% of operating bandwidth below the lower edge of the operating band (extending downwards from the lower edge of the band), 100% of operating bandwidth above the upper edge of the operating band (extending upwards from the upper edge of the band), and between 1/3 of the frequency at the lower edge and 1/3 of the frequency at the upper edge of the operating band.

# Conclusion

In this contribution, views, and proposals on OTA test scope reduction are presented.

Observation 1	For OBUE, from a performance criteria perspective, the lowest limit for Category A is found in the second segment, while for Category B, it is in the third segment. In the case of Category A, it is difficult to consider the 3rd frequency offset as the worst-case scenario.

Observation 2	Using 1% of the momentary measurement frequency as measurement step size ensures a uniform testing for all BSs.

Observation 3	From testing perspective, OTA receiver out-of-band blocking is similar to EMC radiated immunity.

Observation 4	When the frequencies are closer to the operating band, a denser set of test points may be required.

Observation 5	For FR2 out-of-band blocking test ranging from 30 – 6000 MHz, the step size is also 1 MHz. Similar test reduction work would be needed for FR2.

Proposal 1	Apply Option 3 for the TX IM test reduction of the OBUE test case, but do not apply it when the third segment is not the lowest limit, such as in the case of Category A WA BS.

Proposal 2	Use 1% of the momentary frequency as the testing step size for frequencies between 100 MHz and 6000 MHz. For frequencies above 6000 MHz, use a fixed step size of 60 MHz. For frequencies between 30 MHz and 100 MHz, use a fixed step size of 1 MHz. On top of that, apply 1/3 of the minimum supported bandwidth in the following frequency ranges: 100% of operating bandwidth below the lower edge of the operating band (extending downwards from the lower edge of the band), 100% of operating bandwidth above the upper edge of the operating band (extending upwards from the upper edge of the band), and between 1/3 of the frequency at the lower edge and 1/3 of the frequency at the upper edge of the operating band.

# References

R4-2508658, Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

TS 38.113, NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)

IEC 61000-4-3, Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test