3GPP TSG-RAN WG4 Meeting #116	R4-2510969

# Bengaluru, India, August 25th – 29th, 2025

Title:	Benefits of Nominated BW for 249

Source:	Qualcomm Incorporated

Agenda item:	7.29.5

Release:	Rel-19

SI: 	IoT_NTN_TDD-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4 has received an LS from ETSI TC SES [14] where ETSI asks 3GPP feedback on the usefulness of the concept of nominated bandwidth. They request feedback by the SESSCN#53e which is on 25th September 2025. In this paper we discuss the usefulness of this concept and propose a reply to the LS. We also provide a companion paper with the draft LS accordingly [15].

# 2. 	Discussion

## 2.1	Concept of nominated bandwidth or additional guard band

Regulatory requirements for bands 254 and 249 have been captured based on ETSI EN 301 441 and ITU-R M.1343-1 in to TS 36.102 under NS flags.

Band 254 shares the UL frequency range with the new band 249. There is an ongoing discussion how to define the AMPR for that band since the current specification has TBD for it.

We proposed the use of nominated bandwidth in [13] and for FDD band in [11] by defining a nominated bandwidth for 249 that is larger than the channel BW and this way the problematic additional spectral emission mask requirements defined under NS_04N and NS_05N originating from ETSI EN 301 441 would be pushed out from the transmission edge and the requirements could be met without any AMPR.

In the discussions for 254, only fully allocated waveform AMPR has been agreed but the proposals for a single tone are quite far away from each other and all of the are non-zero. Having all the possible power available from a UE that is trying to close UL with a satellite, is beneficial.

Observation 1: Nominated bandwidth concept or equivalent additional guard band to emissions is useful for satellite UEs since it enables UE to transmit more power.

Rel-20 WI has been agreed to study output power for NTN IoT systems up to 37 dBm. The emission requirements are of absolute value i.e. same emission level need to be met regardless of the transmit power. It is expected that without nominated bandwidth concept or additional guard band, benefit of higher power classes would be greatly diminished for NTN IoT.

Proposal 1: Use nominated bandwidth concept or guard band for band 249 and 254.

Finally, there have been different understanding on nominated bandwidth and how it relates to ETSI harmonized standards. To avoid confusion, in 3GPP context the additional frequency offset to emission limits could be just named guard band.

RAN4 has received an LS from ETSI which asks about usefulness of the nominated bandwidth. In response, RAN4 should respond that nominated bandwidth is useful as it allows UE to transmit higher power in some operating bands like 249 and 254, and recommend ETSI to include these bands

Proposal 2: Reply to ETSI that nominated bandwidth is useful as it allows UE to transmit higher power in some operating bands, e.g. 249 and 254.

Proposal 3: In case it is agreed to introduce nominated bandwidth or additional guard band in 3GPP, reply to ETSI they should include requirements for these bands with the guard band in place.

Reply to the LS [14] should be drafted accordingly.

## 2.2	How to define the nominated bandwidth in 3GPP specifications

How to exactly use the concept needs further discussion. ETSI EN 301 441 allows for flexible, manufacturer dependent nominated bandwidth. This would mean a definition of a new capability in 3GPP domain. However, if there would be different UEs with different nominated bandwidths, the network would need to manage these UEs differently either by placing the transmission bandwidth in different frequency locations or using maximum of all the nominated bandwidths.

There is a further complexity with flexible nominated bandwidth and especially if network is given a possibility to configure the UE with a nominated bandwidth rather than just accepting what UE declares. In the case when network configures UE with a smaller nominated bandwidth than what UE has declared, and assuming it is declared with zero AMPR condition, UE would need to apply some AMPR and 3GPP would need to define nominated bandwidth dependent AMPR for example as a function of ratio of zero AMPR declared nominated bandwidth and configured nominated bandwidth.

There is also a disadvantage with flexible nominated bandwidth in initial access. Network would not be able to know how to define RACH resources if it does not know what is the UEs nominated bandwidth. This means that nominated bandwidth dependent RACH resources would be needed in the specifications which would limit number of different RACH resources and therefore system capacity. Note that ETSI document also allows for non-symmetrical nominated bandwidth.

Observation 2: Flexibility in additional guard band will create complexity.

It is more straightforward to define one nominated bandwidth or additional guard band in the 3GPP specifications to be one fixed value and associated with the corresponding additional requirements under NS values.

Proposal 4: Define one value of additional guard band larger than channel bandwidth for 249 in the context of satisfying requirements in ETSI EN 301 441 and ITU-R M.1343-1.

## 2.3 	Suitable value for the nominated bandwidth

Channel raster for NTN NB IoT is 100 kHz. Simple choice would be to define the value of even 100 kHz. Our analysis shows that 300 kHz is not wide enough and some AMPR would still be needed so next value is 400 kHz.

Figure 1. Emissions from single tone transmission and ETSI mask

This value would also help to mitigate concerns of UE meeting emissions under Doppler shifted transmissions if RAN5 starts to test the RF requirements in with realistic models.

Proposal 5:  Use 400 kHz as nominated bandwidth for 249, i.e. apply 100 kHz guard band on both sides.

While the 400 kHz may look like it is over estimated and the spectral efficiency is halved, network it has means to recover it with planned deployment. The spacing of the channels within the frequency allocation can be closer than the nominated bandwidth since spectrum holder only need to care about the emissions outside his frequency block.

400 kHz would also support zero AMPR for PC1 and zero AMPR for fully allocated waveform, as shown in Appendix.

## 2.4 	Required A-MPR without guard band

Given the differences in earlier A-MPR results, measurement and simulation analysis was done to check needed A-MPR. While there are no commonly agreed calibration points for NB-IoT, PA 0 dB MPR point was approximately 1 dB higher than achieved for NR DFT-s-OFDM signal, with post-PA loss applied to set output power at measurement point to 23 dBm with MPR = 0 dB. Waveforms used met the general SEM, spurious and ACLR requirements with using only MPR, guaranteeing that the emission performance is representative of a device meeting 3GPP minimum requirements.

Transceiver impairments were aligned with minimum requirements, with -25 dBc IQ-image and DC-leakage and CIM3 applied with -60 dBc level.

Analysis was performed for NS_04N and NS_05N, both for 15 kHz and 3.75 kHz SCS. The results are summarized in Table 1 to Table 4.

Table 1: A-MPR for NS_04N, 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤11 |  | ≤5.5 | ≤3.5 | ≤2 | ≤0.5 | ≤0.5 | ≤2 |  | ≤3.5 | ≤5.5 | ≤ 11 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |  |
| A-MPR (dB) | ≤ 8.5 |  |  |  | ≤ 2.5 dB |  |  |  | ≤ 8.5 |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 5.5 |  |  |  |  |  | ≤ 5.5 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 2 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: A-MPR for NS_04N, 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-9 | 10-38 | 39-41 | 42-44 | 45-47 |
| A-MPR (dB) | ≤ 7.5 | ≤ 3 | ≤ 0.5 | ≤ 0 | ≤ 0.5 | ≤ 3 | ≤ 7.5 |

Table 3: A-MPR for NS_05N, 15 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR (dB) | ≤ 14 |  | ≤7 | ≤5 | ≤3.5 | ≤2 | ≤2 | ≤3.5 |  | ≤5 | ≤7 | ≤ 14 |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  | 9-11 |  |  |  |  |
| A-MPR (dB) | ≤ 10 |  |  |  | ≤ 4 |  |  |  | ≤ 10 |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 8 |  |  |  |  |  | ≤ 8 |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR (dB) | ≤ 3.5 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for NS_05N, 3.75 kHz SCS

| Modulation | QPSK |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for 3.75 kHz single tone allocation | 0-3 | 4-6 | 6-10 | 11-37 | 38-41 | 42-44 | 45-47 |
| A-MPR (dB) | 10.5 | 5.5 | 2.5 | 0 | 2.5 | 5.5 | 10.5 |

In previous meeting, also NS_11N and NS_12N were introduced to band 254. NS_11N and NS_12N are otherwise equal to NS_04N and NS_05N, but they capture also a more stringent in-channel PSD limit. Therefore, for NS_11N and NS_12N A-MPR needs to be the maximum of A-MPR required due to in-channel PSD and spectrum emission mask. Table 5 captures the A-MPR due to in-channel PSD limit of 15 dBm/4kHz.

As this in-band PSD limit is defined as a peak limit, the PAPR of the signal needs to be taken into account. The value below assumes 7 dB PAPR and 3 dB margin to the limit.

Table 5: A-MPR due to in-channel PSD limit of 15 dBm/4kHz for NS_11N and NS_12N

| SCS and number of tones | 3.75kHz | 15 kHz 1-tone | 15 kHz 3-tone | 15 kHz 6-tone | 15 kHz 12-tone |
| --- | --- | --- | --- | --- | --- |
| A-MPR (dB) | 18.0 | 14.5 | 10 | 7 | 4 |

Proposal 6: A-MPR in Table 1 to Table 4 is used for NS_04N and NS_05N for PC3, if no guard band is specified.

Proposal 7: Max A-MPR between Table 1, Table 2 and Table 5 is used for NS_11N. Max A-MPR between Table 3, Table 4 and Table 5 is used for NS_12N for PC3. A-MPR is required if large enough guard band is not specified.

Proposal 8: A-MPR is Table 5 is specified for NS_11N and NS_12N also with guard band.

## 2.5 	Requirement updates

To take guard band into use, related emission requirements need to be adjusted

Proposal 9: Adjust NS_04N and NS_05N as well as NS_11N and NS_12N additional in-band emission masks to start to apply +/- 200 kHz from carrier frequency, i.e. outside nominated bandwidth of 400 kHz.

As an alternative, it could be also possible to define requirements for two options:

- Requirements without using nominated bandwidth

- Requirements using 400 kHz nominated bandwidth

This is a more complicated approach, as e.g. in initial access there has been no information exchange between UE and network on which nominated bandwidth is assumed and which A-MPR UE should therefore apply. This could be solved by adding related information to system broadcast information.

Proposal 10: Also consider the option to define requirements both with and without nominated bandwidth.

Emission requirements are the same for bands 249 and 254, so it would be logical to apply same solution to both bands.

Proposal 11: Apply the same solution to bands 254 and 249.

Companion CRs showing both alternatives are available in [15] and [16].

# Conclusion

We discussed and observed:

Observation 1: Nominated bandwidth concept or equivalent additional guard band to emissions is useful for satellite UEs since it enables UE to transmit more power.

Observation 2: Flexibility in additional guard band will create complexity.

And proposed:

Proposal 1: Use nominated bandwidth concept or guard band for band 249 and 254.

Proposal 2: Reply to ETSI that nominated bandwidth is useful as it allows UE to transmit higher power in some operating bands, e.g. 249 and 254.

Proposal 3: In case it is agreed to introduce nominated bandwidth or additional guard band in 3GPP, reply to ETSI they should include requirements for these bands with the guard band in place.

Proposal 4: Define one value of additional guard band larger than channel bandwidth for 249 in the context of satisfying requirements in ETSI EN 301 441 and ITU-R M.1343-1.

Proposal 5:  Use 400 kHz as nominated bandwidth for 249, i.e. apply 100 kHz guard band on both sides.

Proposal 6: A-MPR in Table 1 to Table 4 is used for NS_04N and NS_05N for PC3, if no guard band is specified.

Proposal 7: Max A-MPR between Table 1, Table 2 and Table 5 is used for NS_11N. Max A-MPR between Table 3, Table 4 and Table 5 is used for NS_12N for PC3. A-MPR is required if large enough guard band is not specified.

Proposal 8: A-MPR is Table 5 is specified for NS_11N and NS_12N also with guard band.

Proposal 9: Adjust NS_04N and NS_05N as well as NS_11N and NS_12N additional in-band emission masks to start to apply +/- 200 kHz from carrier frequency, i.e. outside nominated bandwidth of 400 kHz

Proposal 10: Also consider the option to define requirements both with and without nominated bandwidth.

Proposal 11: Apply the same solution to bands 254 and 249.

# Reference

[1]	RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2419795, “Way Forward for [113][317] IoT_NTN_TDD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-1711536, “Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs”, RAN4 (Ericsson), 3GPP TSG RAN WG2 Meeting #84bis, Dubrovnik, Croatia, 9th – 13th October 2017

[4] R1-2500003, Report of RAN1#119 meeting

[5]	R4-2502292, “Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500491, “Initial views on RF impact for NTN IoT TDD”, Qualcomm, 3GPP TSG-RAN4 Meeting #114 Athens, Greece,  16th – 21st February, 2025

[7]	R4-2501153, “AMPR value for NB-IoT in IoT NTN band b254”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #114, Athens, GR, 17th – 21st February, 2025

[8]	R4-2504719, “Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF”, Thales, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505311, “LS on precompensation for NB-IoT NTN TDD mode”, RAN1, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[10]	R4-2503564, “Frequency error and UE regulatory requirements”, Qualcomm, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11] R4-2506174, “On completing band 254 requirements”, Qualcomm,  3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[12]	R4-2507733, “LS Reply on precompensation for NB-IoT NTN TDD mode”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[13]	R4-2506164, “Nominated BW use for 249 and freq error”, Qualcomm,  3GPP TSG-RAN WG4, Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[14]	R4-2509023 (SES(25)000071)) , “LS on Harmonised Standard for NTN capable UE”, ETSI TC SES, 3GPP TSG-RAN WG4 Meeting #116, Aug 2025

[15]	R4-250, “LS Reply on LS on Harmonised Standard for NTN capable UE”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #116, Aug 2025

[16] R4-2510970, “Draft CR 38101-1 on Flexible guard band for 249”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #116, Aug 2025

[17] R4-2510971, “Draft CR 38101-1 on Guard band for 249”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #116, Aug 2025

# Appendix: Other emissions cases