# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510977

# Bangalore, India, 25th – 29th August 2025

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.13.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial discussions on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation and intra-cell inter-user interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

  - Sub-topic 1-1 General

Definition of MMSE-IRC receiver capability for 8Rx UE

Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

Regarding the above definition of MMSE-IRC receiver, the definition at TS 38.306[3] and 38.101-4[4] are currently as follows.

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Currently, RAN4 considered the inter-cell and intra-cell inter-user interference scenarios in Rel-19 MMSE-IRC receiver. As discussed before, the simplified receiver with 4T8R and rank 2+2 test cases are only applicable for MU-MIMO scenario. From this point of view, the current description of TS 38.306 and 38.101-4 need to be redefined. Therefore, the above option 4 and option 5 are no longer applicable to the receiver description of Rel-19. On the other side, the motivation of defining baseline and simplified receivers in RAN4 is to distinguish the different baseband processing in Rel-18. The initial purpose is to reduce the computational complexity of the covariance matrix in the presence of a large number of receiver antennas and MIMO layers. We think it is better to clearly description in the specification according to the receivers, because RAN4 defines a variety of different receiver types.

From Rel-15 to Rel-19, RAN4 defined different receivers to support different scenarios. In Rel-15, RAN4 defined demodulation requirements for MMSE-IRC and R-ML receiver with 2Rx and 4Rx under SU-MIMO scenario. In Rel-17, RAN4 defined inter-cell interference and intra-cell inter-user interference demodulation requirements for MMSE-IRC receiver with2Rx and 4Rx, and IRC receiver as mandatory feature from this release. In Rel-18, RAN4 defined R-ML demodulation requirement for intra-cell inter-user interference scenario with 2Rx and 4Rx , and introduced 8Rx MMSE-IRC receiver for SU-MIMO based on baseline receiver and simplified receiver. In Rel-19, inter-cell interference and intra-cell inter-user interference with 8Rx are defined. Firstly , SU-MIMO in the description should be removed, because MU-MIMO scenario is defined for baseline and simplified 8Rx receiver.

Currently, only MMSE-IRC receiver can support 8Rx up to 8layers. If other advanced receiver is introduced in the feature release, it is difficult to explain whether different detection methods are supported. If not, we think the current naming is acceptable. Otherwise, more description is needed in the specification. To avoid some confusion, redefine the receiver definitions also can be acceptable.

Observation 1. Baseline receiver and simplified receiver are only applicable for 8Rx for SU-MIMO transmission with support of up to 8 layers.

Observation 2. Currently, only MMSE-IRC receiver can support 8Rx from specification.

Observation 3. The wording of SU-MIMO should be removed.

Proposal 1. An explicit description is needed if RAN4 introduces another 8Rx receiver in the feature.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for UE demodulation performance requirements for 8Rx with MMSE-IRC , The conclusions are:

Observation 1. Baseline receiver and simplified receiver are only applicable for 8Rx for SU-MIMO transmission with support of up to 8 layers.

Observation 2. Currently, only MMSE-IRC receiver can support 8Rx from specification.

Observation 3. The wording of SU-MIMO should be removed.

Proposal 1. An explicit description is needed if RAN4 introduces another 8Rx receiver in the feature.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- R4-2508620, Way forward for [115][323] NR_demod_Ph5_Part2_UE.

- 3GPP TS 38.306 V18.6.0.

- 3GPP TS 38.101-4 V18.8.0.