# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510980

# Bangalore, India, 25th – 29th August 2025

# Title: 	Discussion on Rel-19 ATG demodulation requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.7

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID of enhancements for Air-to-ground network for NR was approved in [1], the objectives related to enhanced ATG are listed :

The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

In Rel-18, RAN4 defined demodulation requirements for PDSCH under AWGN channel model on FR1 single scenario. However, considering that ATG has the advantage of high throughput, low propagation delay, and low cost application, some operators and the aircraft industry have a strong request for the deployment of ATG. In R19, the following two aspects are considered:

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

In this contribution, we give some initial discussion on the enhanced ATG demodulation requirements according to the last meeting agreements[2].

- 2 Discussion

- DL CA requirements

TDD pattern

- Option 1: Use the legacy TDD pattern as the starting point - Option 2: Define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirement

Currently, legacy CA demodulation requirements also follows single CC requirement. From this point of view, we think ATG feature shall follow this rule.

- Proposal 1. Propose to define TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirements.

Rank

- Option 1: For CA requirement, use Rank 2 as the starting point - Option 2: Only consider Rank 1 for DL CA demodulation requirements

- In legacy CA demodulation requirements, RAN4 only reuses the test parameters from single CC requirements to define CA requirements. Hence, we propose to only consider Rank 1 for DL CA demodulation requirements.

- Proposal 2. Propose to only consider Rank 1 for DL CA demodulation requirements.

DL MIMO requirements

TDD pattern

- Option 1: Use legacy TDD pattern 7DS2U as the starting point.

Currently, two TDD pattern demodulation requirements are defined in Rel-18. There is reasonable to define two requirements for different patterns.

- Proposal 3. Propose to define TDD pattern 30D4S6U demodulation requirements for DL MIMO.

- MCS

- Option 1: consider MCS 13 and MCS 22 - Option 2: Consider 64QAM - Other Options are not precluded

- We evaluated 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 with 2T2R, the SNR point of 70% maximum throughput and curves can be found in the following. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher. Therefore, we prefer to consider MCS 13 or MCS 22 to define requirements.

- Table 1. Simulation results for 2T2R Rank 2

| Propogation condition | Antenna configuration | MCS | Layer | SNR(70% Max TP) |
| --- | --- | --- | --- | --- |
| AWGN+220Hz | 2T2R | 16QAM Table 1 MCS13 | Rank 2 | 7.41 |
|  |  | 64QAM Table 1 MCS 22 | Rank 2 | 14.87 |
|  |  | 256QAM Table 2 MCS 24 | Rank 2 | 22.76 |

- Table 2.  Throughput curves for 2T2R Rank 2

| - Fig 1. FDD, MCS13, Rank 1 and Rank 2 | - Fig 2. FDD, MCS22, Rank 1 and Rank 2 |
| --- | --- |
| - Fig 3. FDD, MCS24, Rank 1 and Rank 2 |  |

- Observation 1. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

- Proposal 4. Propose to consider MCS 13 or MCS 22 defining demodulation requirements.

UL MIMO requirements

Antenna configuration for incremental test

- Option 1: 2T2R - Option 2: 2T2R, 2T4R and 2T8R with X-pol low correlation as the starting point - Option 3: 2T2R with low correlation

Since the BS is to support ATG feature, it should pass the Rel-15 normal demodulation requirements and dedicated requirements. In order to reduce the incremental test cases, we propose to define one dedicated requirement with antenna configuration.

Proposal 5. Propose to consider 2T2R antenna configuration to define incremental test.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of ATG UE demodulation requirements, The conclusions are:

- Observation 1. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

- Proposal 1. Propose to define TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirements.

- Proposal 2. Propose to only consider Rank 1 for DL CA demodulation requirements.

- Proposal 3. Propose to define TDD pattern 30D4S6U demodulation requirements for DL MIMO.

- Proposal 4. Propose to consider MCS 13 or MCS 22 defining demodulation requirements.

Proposal 5. Propose to consider 2T2R antenna configuration to define incremental test.

# 4	Reference

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR. CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom.

- R4-2508724, Way Forward for [115][328] NR_ATG_enh_demod, CMCC.