# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510981

# Bangalore, India, 25th – 29th August 2025

# Title: 	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.4.6

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, a new WID on 7 MHz Channel Bandwidth for n26 and n5 was approved in [1], wherein the objectives related to 7MHz are listed as below:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR)

In the following section, we will provide the detailed discussions for 7MHz UE and BS demodulation requirements according to the last meeting agreements[2].

# 2 Discussion

With the introduction of 5GNR, the channel bandwidths were limited to increments of 5 MHz. Now that the 3 MHz channel bandwidth has been added, there is the possibility of coverage of additional operator’s spectrum including 8 and 13 MHz via CA, but there is still not a good way to cover other channel bandwidths that have been requested by operators including 6, 7, 11, 12 and 13 MHz. Given the large investment in spectrum, especially low band spectrum, it is critical that operators have a way to efficiently utilize all their spectrum. While several approaches were initially proposed for this WID, the agreement was to include only native 7 MHz channel bandwidths for n5 and n26 in Rel-19.

UE demodulation requirements

Whether to define PDSCH CA requirments for 7MHz bandwidth

Agreements: - Define normal PDSCH type A CA requirements for 7MHz - Not to consider HST-SFN and HST-DPS scenario. - Interested companies are welcome to bring results for 7MHz. - FFS how to define requirements: - Option 1: Define new requirements for 7MHz CC based on the simulations - Option 2: Reuse existing 5MHz/10MHz requirements

Regarding the 7MHz demodulation requirements, we listed the current 5MHz and 10MHz requirements as below. It is obviously observe that the requirements between 5MHz and 10MHz are the same.

Observation 1. Currently, the 5MHz and 10MHz demodulation requirements are the same value in 38.101-4.

Table 5.2A.2.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.6 |

Table 5.2A.3.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-9.1 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |
| 10 | R.PDSCH.1-2.2 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.5 |

Also, we evaluated the performance between 5MHz, 7MHz, and 10MHz based on the last meeting simulation assumption. The simulation results could be found in the following part.

Table 1. FDD 15kHz 2T2R, 5MHz, 7MHz, and 10Mhz

| Case Number | Duplex Mode and SCS | CHBW | Rank | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz | 5MHz | Rank 2 | 2T2R | Random | TDLA30-10 | ULA Low | MCS13 | 11.0 |
| 2 |  | 7MHz |  |  |  |  |  |  | 11.6 |
| 3 |  | 10MHz |  |  |  |  |  |  | 11.6 |

Table 2. FDD 15kHz 2T4R, 5MHz, 7MHz, and 10Mhz

| Case Number | Duplex Mode and SCS | CHBW | Rank | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | FDD 15kHz | 5MHz | Rank 2 | 2T4R | Random | TDLA30-10 | ULA Low | MCS13 | 6.0 |
| 5 |  | 7MHz |  |  |  |  |  |  | 6.1 |
| 6 |  | 10MHz |  |  |  |  |  |  | 6.1 |

| Figure 1. FDD 15kHz 2T2R MCS 13 TDLA 30-10 | Figure 2. FDD 15kHz 2T4R MCS 13 TDLA 30-10 |
| --- | --- |

Observation 2. From our simulation results, the SNR point of 5MHz, 7MHz, and 10MHz are basically the same.

Proposal 1. Propose to reuse current 5MHz and 10MHz requirements for 7MHz.

Release independence for 7MHz bandwidth requirements

- Proposals - Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

In the last meeting, RF session reached a following conclusion for release independent with 7MHz bandwidth requirements.

- Agreement: - Support of 7 MHz channel BW could be Release independent from Rel-15 - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz. - Support of 7MHz is optional in Rel-19 and in the previous releases.

Proposal 2. Demodulation requirements could be released independently from Rel-15.

# 3 Conclusion

In this contribution, we give some discussions on the UE and BS demodulation requirements for 7MHz channel bandwidth, The conclusions are:

Observation 1. Currently, the 5MHz and 10MHz demodulation requirements are the same value in 38.101-4.

Observation 2. From our simulation results, the SNR point of 5MHz, 7MHz, and 10MHz are basically the same.

Proposal 1. Propose to reuse current 5MHz and 10MHz requirements for 7MHz.

Proposal 2. Demodulation requirements could be released independently from Rel-15.

# 4 References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA.

- R4-2508757, WF on demodulation and CSI requirements for 7MHz, Huawei, HiSilicon.