# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510986

# Bangalore, India, 25th – 29th August 2025

# Title: 	Discussion on UE demodulation performance requirements for LP-WUS

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.24.7

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved in [1], wherein the objectives related to LP-WUS are listed as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the following section, we will provide the detailed discussions for LU-WUS demodulation performance requirements according ti the last meeting agreements[2].

# 2 Discussion

Testability Aspects

Whether to define a test mode for LP-WUS/WUR

- Agreement - Take RF approach as a starting point to test MDR for demodulation requirements. RF agreement Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - FFS whether test mode is recommended for MDR testing.

In last meeting, RAN4 had a command understanding that RAN4 shall discussed the testability first, if the testability of LP-WUS signal can’t be verified or RAN4 can’t find a approach o test it, RAN4 believes there s no need to define requirements. However, a positive agreement was achieved in RF session which 1% MDR of LP-WUS can be tested basd on UE’s response to the TE upon successfully detection the the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods). From this point of view, the testability issues has been verified in RF session that the LP-WUS signal can be tested in RAN5. Therefore, the demodulation requirements can be introduced for demodulation requirements according to current conclusions.

Based on this approach, the testability issues don’t need more discussion and RAN4 should discuss the related issues of test mode and test duration during testing. Currently, according to the design of LP-WUS signal, which can work on IDLE and CONNECTED modes. In IDLE mode, UE needs to monitor the LP-WUS signal before waking up the MR to monitor the PO. And in CONNECTED mode, the LP-WUS signal is used to wake up MR to monitor PDCCH resource. Therefore, from the perspective of testing, two test modes could be considered. Regarding how to chose the test mode, maybe depends on UE how to support this power saving feature, UE may only support this feature in IDLE mode or CONNECTED mode, or both of them. Currently, it is hard to say how UE will support this feature. From our understanding, it’s better to consider two test modes for MDR testing.

Observation1.Currently, how the UE supports this feature depends on UE implementation.

Proposal 1. Propose to consider two test modes for MDR testing.

Whether to introduce FAR test

- WF - Option 1: Yes. - Option 2: No.

In wireless communications, if a BS mistakenly sends a wake-up signal, the UE will wake up from a low power state (such as IDLE/INACTIVE) and perform operations such as channel estimation and synchronization, consuming additional power. False alarm can cause the UE to misjudge the channel state, triggering unnecessary measurement reporting, handover processes, or retransmission mechanisms, increasing baseband and radio frequency power consumption. A low false alarm rate means that the UE is awakened only when truly needed (such as when data is being transmitted), avoiding frequent and ineffective state transitions that extend sleep time.  Excessively reducing the false alarm rate can lead to miss detection, causing the UE to miss the actual wake-up signal. Therefore, it is necessary to introduce false alarm test to save energy for UE.

Proposal 2. Propose to introduce FAR test requirements.

Test metric for FAR test (if introduced)

- WF - Option 1: 1% FAR - Option 2: 0.1% FAR

Regarding test metric, considering that the LP-WUS signal is designed to save power and reduce power consumption, the FAR means that the terminal wakes up unnecessarily, increasing power consumption (for example, the receiving chain is triggered by mistake when it should be asleep). MDR means the probability that the terminal fails to correctly detect a valid wake up signal (actually there is a signal), and it is necessary to increase uplink retransmission or delay, e.g., the based station needs to send LP-WUS repeatedly.

Proposal 3. Propose to consider both 0.1% FAR as test metric.

Frequency range

- WF - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2.

Currently, the frequency of FR1 is more typical than FR2, we would prefer to prioritize FR1.

Proposal 4. Propose to prioritize FR1 frequency range.

SCS

- WF - Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1. - Option 2: 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2

Because we prioritize FR1, the related SCS should be 15kHz and 30kHz.

Proposal 4. Propose to only consider 15kHz for FDD and 30kHz for TDD.

Bandwidth

- WF - Option 1: For the channel bandwidth and SCS, take below assumptions for the starting points - 10MHz CHBW with SCS 15KHz for FR1 FDD - 40MHz CHBW with SCS 30KHz for FR1 TDD - 100MHz CHBW with SCS 120KHz for FR2

We prefer to consider typical channel bandwidth for FDD and TDD of FR1.

Proposal 5. Propose to consider 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD.

Whether to define performance requirements for LP-SS

- WF - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

Currently, LP-SS signal is used to provide time/frequency synchronization to ensure that the UE can accurately demodulate subsequent wake-up signals or data. Compared to traditional synchronization signals (such as PSS/SSS), LP-SS can have a longer transmission period and a simpler structure to reduce device power consumption. We don’t think that define performance requirements for LP-SS is necessary.

Observation 2. LP-SS signal is used to provide time/frequency synchronization to ensure that the UE can accurately demodulate subsequent wake-up signals or data.

Proposal 6. Propose to not define performance requirements for LP-SS.

# 3 Conclusion

In this contribution, we give some discussions on the LP-WUS demodulation requirements, The conclusions are:

Observation1.Currently, how the UE supports this feature depends on UE implementation.

Observation 2. LP-SS signal is used to provide time/frequency synchronization to ensure that the UE can accurately demodulate subsequent wake-up signals or data.

Proposal 1. Propose to consider two test modes for MDR testing.

Proposal 2. Propose to introduce FAR test requirements.

Proposal 3. Propose to consider both 0.1% FAR as test metric.

Proposal 4. Propose to only consider 15kHz for FDD and 30kHz for TDD.

Proposal 5. Propose to consider 10MHz CHBW with SCS 15KHz for FR1 FDD and 40MHz CHBW with SCS 30KHz for FR1 TDD.

Proposal 6. Propose to not define performance requirements for LP-SS.

# 4 References

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Qualcomm.