# 3GPP TSG-RAN WG4 Meeting #116                                R4-2510988

# Bangalore, India, 25th – 29th August 2025

# Title: 	TP for 38.753 introduction of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.12.3

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model parts.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.1  SU-MIMO (PMI Aspects)

This section provides comparison results for different spatial channel modes with simulation assumptions captured in Table 6.1-2.

Table 6.1-2: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8TX | 32TX |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | FDD |  |
| TDD DL-UL configurations |  |  |  | N/A |  |
| MCS |  |  |  | 13 |  |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) | 32 x 4 (N1,N2) = (8,2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 8 | 32 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8,(4,6) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8,(5) | Row 17,(5, 12) |
|  | CSI-RS interval and offset |  | slot | 10, 0 |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |  |
| Sub-band Size |  |  | RB | 8 |  |
| csi-ReportingBand |  |  |  | 11111111111111 |  |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |  |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |  |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) | (8,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) | (4,4) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |  |
| Physical channel for CSI report |  |  |  | Perfect |  |
| CQI/RI/PMI delay |  |  | ms | 7 |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |

Table 6.1-3: Simulation assumptions for CDL channel

| Parameter |  | Value |
| --- | --- | --- |
| FR / Carrier frequency |  | FR1,3.5GHz |
| UE speed and movement direction |  | 3km/h, (65°,90°) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

Two CDL channel based models and two TDL based models are captured for comparison simualtion.

- CDL option 1: The model is based on Table 7.7.1-3 in TR 38.901 without truncation and quantization.

- CDL option 2: The model is based on Table 7.7.1-3 in TR 38.901 with truncation and quantization. And the mdoel profile is in Table 5.1.4.3-1.

- Enhanced TDL option 1: The model description is in Annex C.1.1.1.

- Enhanced TDL option 2: The model description is in Annex C.1.1.2.

Table 6.1-3: AAV configurations

| Options | Description |
| --- | --- |
| AAV 1Y | (16,8,2,8,1) for SU-MIMO eTypeII, for 32 Tx |
|  | (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx |
| AAV 3 | (M,N,P,Ms,Ns) = (2,8,2,1,1) for 32Tx |
|  | (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx |

The following comparison test cases are included this chapter:

- FR1 SU-MIMO PMI 32Tx4Rx 4 layers with Type I (PMI Follow and PMI Random) and eType II (PMI Follow) CodeBook.

- FR1 SU-MIMO PMI 8Tx4Rx 4 layers with Type I (PMI Follow and PMI Random) and eType II (PMI Follow) CodeBook.

- FR1 SU-MIMO PMI 32Tx4Rx 2 layers with Type I (PMI Follow and PMI Random) and eType II (PMI Follow) CodeBook.

- FR1 SU-MIMO PMI 8Tx4Rx 2 layers with Type I (PMI Follow and PMI Random) and eType II (PMI Follow) CodeBook.

More specific simulation results are captured in [R4-xxxxxxx].

Observation: TBD

# 3 Conclusion

This draft TP captures comparison of PMI performance under SU-MIMO scenario for different spatial channel model part.