3GPP TSG-RAN WG4 Meeting #116	R4-2511002

Bangalore, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-07-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction blocking requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add blocking requirements, including IBB, OBB and NBB for LP-WUR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUR blocking requirements are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6M.1, 7.6M.2, 7.6M.3, 7.6M.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

7.6M	Blocking characteristics for LP-WUR

7.6M.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

7.6M.2	In-band blocking for LP-WUR

The UE shall fulfil the minimum requirements specified in Table7.6M.2-1 and Table 7.6M.2-2 for LP-WUR type 1 UE and LP-WUR type 2 UE respectively. The test parameters for the interferer are chosen as in Table 7.6M.2-3 for verification of the requirements specified in Table 7.6M.2-1 and Table 7.6M.2-2. For these test parameters, the Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1.

Table 7.6M.2-1: In-band blocking parameters for LP-WUR type 1 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 15 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-2: In-band blocking parameters for LP-WUR type 2 UE

| RX parameter | Units | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS bandwidth | dBm | REFSENS + 9 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 |  |  |

Table 7.6M.2-3: In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset) | MHz | -BW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -BW/2 – FIoffset, case 2 and ≥ BW/2 + FIoffset, case 2 |

7.6M.3	Out-of-band blocking for LP-WUR

The out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band. The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1 with parameters for the wanted signal as specified in Table 7.6M.3-1 and Table 7.6M.3-2 for LP-WUR type 1 and type 2 UE respectively, and for the interferer as specified in Table 7.6M.3-3.

Table 7.6M.3-1: Out-of-band blocking parameters LP-WUR type 1 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS bandwidth [dBm] |
| --- | --- |
| 11 | REFSENS + 15 dB |

Table 7.6M.3-2: Out-of-band blocking parameters LP-WUR type 2 UE

| LP-WUS bandwidth (RBs) | Power in LP-WUS bandwidth [dBm] |
| --- | --- |
| 11 | REFSENS + 9 dB |

Table 7.6M.3-3: Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6M.3-3, a maximum of

max24,6⋅n⋅NRB/6minn⋅NRB/10,5

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of 5 MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration of NR signal, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2 for SCS = 15, 30 kHz, respectively. For these exceptions, the requirements in clause 7.7M apply.

7.6M.4	Narrow band blocking for LP-WUR

This requirement is measure of a receiver's ability to receive a LP-WUS signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The Miss Detection Rate (MDR) for LP-WUS signal shall be ≤ 1 % with the reference measurement channels as specified in Annexes A.3M.1 with parameters specified in Table 7.6M.4-1 and Table 7.6M.4-2 for LP-WUR type 1 and type 2 UE respectively.

Table 7.6M.4-1: Narrow Band Blocking for LP-WUR type 1 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 25 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.6M.4-2: Narrow Band Blocking for LP-WUR type 2 UE

| Parameter | Unit | LP-WUS bandwidth (RBs) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + 19 |
| Puw (CW) | dBm | -55 |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier, with the minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

<<End of change>>