# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511020

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for Ambient-IoT

Agenda item:	7.22.4

Document for:	Approval

Introduction

At the RAN#106 meeting, the revised WID on Ambient-IoT [1] has been approved. In this meeting, we will discuss the scenarios and provide our initial views on potential RRM impact. We could see the following description of the latest WID:

Ambient-IoT RAN4 scope: - RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

Discussion

RAN1 and RAN2 have already discussed Ambient-IoT with several meetings. Compared to legacy NR functionalities, the supported and not supported features could be seen as below:

As to the A-IoT required functionalities, the following functionalities are supported: - A-IoT paging (see clause 6.3.3) - A-IoT random access procedure (see clause 6.3.4) - A-IoT data transmission (see clause 6.3.5) As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): - AS security (The AS layer design assumes no support of AS security, unless the study in [152] further concludes differently.) - RRC states - RRC connection management - RRM L3 measurement reporting - Mobility - ASN.1 encoding/decoding - Periodical system information and MIB - Tracking/RAN area update procedure - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. - Multiple A-IoT logical channels for upper layer data - NR SR, as in TS 38.300 [153] - NR BSR, as in TS 38.300 [153]

- In this paper, we would like to mainly study the RRM impact on A-IoT timing issues base on RAN1’s latest agreements.

- For timing relationship, RAN4 had some discussion and reached the agreement as follows:

Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any. - Way forward: - FFS: device receive timing

- For defining D2R transmit timing error requirements, RAN4 has already defined the concrete Te value. However, for MSG1 X=2, RAN1 has the latest agreements as below:

Agreement For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, Toffset1 is not smaller than 30 us for CBRA. Agreement For Toffset1, where Toffset1 is the time interval from the end of the R2D transmission triggering random access to the starting time of the first Msg1 time domain resource for X=1 or X=2, from the device perspective: - Toffset1 is from a set of predefined values. - FFS: the predefined values - E.g. values within the range [2666.8, 1333.4, 666.6, 333.4, 222.2, 166.6, 133.34, 111.2, 83.4, 55.6, 44.44, 41.6, 30] us - FFS: which values of Toffset1 correspond to which values of the following factors (to be selected with future agreement): - R2D chip length, D2R chip length, padding length, whether FEC is applied Agreement Toffset2=αToffset1+βTmsg1, where - Tmsg1 is the duration of the 1st Msg1 time domain resource - α=0.25 and β=1.25

- Based on above agreement, the illustration could be seen as below:

- The starting time of Toffset2 is the starting time of first Msg1, we know that X=2 represents two different Msg1 transmission occasion, that is, Msg1 can be transmitted by TDM method. Toffset1 means TR2D_min and Toffset2 means offset of second Msg1 transmission. From the above figure, we believe that the same principle could apply and the same definition could be considered for X=2. Note that Toffset1 and Toffset2 are two dependents offsets for first Msg1 and second Msg1 transmission, the Te will be calculated by different Msg1 transmission.

- Observation 1: Msg1 can be transmitted by TDM method. Toffset1 means TR2D_min and Toffset2 means offset of second Msg1 transmission.

- Observation 2: Note that Toffset1 and Toffset2 are two dependents offsets for first Msg1 and second Msg1 transmission, the Te will be calculated by different Msg1 transmission.

Proposal 1: For MSG1 X=2, the reference point for the device transmission timing is Toffset 2 after the starting time of first Msg1 transmission occasion. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding first Msg1 transmission starting occasion to the starting time of second MSG1.

For TD2R_min, RAN1 also has the conclusion on it as follows:

Agreement It is up to RAN4 to define the value(s) for TD2R_min. Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

First of all, for this timing gap the illustration could be seen as below:

We observe that the TD2R_min may comprise several components. From device perspective, this timing gap contains like Tx-Rx switching time. From NW perspective, this timing gap contains the time for D2R data reading.

By the way, the TD2R_min is similar to T2 compared to RFID spec as below:

We found that the T2 is directly defined the concrete values. Whether this value could be reference shall have some further consideration.

Overall, we deem that RRM session shall not define the range of TD2R_min .

Proposal 2: RRM session shall not define the range of TD2R_min.

Conclusion

In this paper we provided our views on RRM impact on R19 Ambient-IoT.

- Observation 1: Msg1 can be transmitted by TDM method. Toffset1 means TR2D_min and Toffset2 means offset of second Msg1 transmission.

- Observation 2: Note that Toffset1 and Toffset2 are two dependents offsets for first Msg1 and second Msg1 transmission, the Te will be calculated by different Msg1 transmission.

Proposal 1: For MSG1 X=2, the reference point for the device transmission timing is Toffset 2 after the starting time of first Msg1 transmission occasion. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding first Msg1 transmission starting occasion to the starting time of second MSG1.

- Proposal 2: RRM session shall not define the range of TD2R_min including Tx-Rx switching time or the time for reading D2R data.

- 4 References

- RP-243326, New WID: Solutions for Ambient IoT (Internet of Things) in NR.

- TR 38.769

- TS 38.133

- TR 38.848