# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511021

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for downlink coverage enhancement

Agenda item:	7.27.5.2

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved. RAN4 shall define the RRM requirements for downlink coverage enhancement based on the latest revised WID as below:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

Discussion

In past meetings, RAN4 has mainly discussed the RRM requirements on SSB periodicity larger than 20ms. There are still some issues to be discussed in this meeting.

Downlink coverage enhancement is a new feature for R19 UE, and RAN4 replied LS to RAN2 on SMTC enhancements in the last meeting towards two issues as follows:

- support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

For the first bullet, we think it is feasible for downlink coverage enhancement. The case is downlink coverage enhancement from the same satellite, and a new default SSB periodicity was introduced by RAN1 which is 160ms. To our understanding, it is not enough to capture all neighbouring cell SSBs if we still use one SMTC periodicity with different offsets. Thus, we deem that it is reasonable to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbour cell measurements. The SMTC periodicity is changed and there is RRM requirements impact.

- For the second bulllet, we observe that UEs conduct measurement based on previous capabilities which are mandatorily support 2 parallel SMTC measurements and optionally 4 parallel SMTC measurements. NTN cells cover a relatively larger area, UE only need to measure the SMTCs of nearby cells within a certain period. For instance, if a UE is located at one corner of a cell, it may not need to measure the cell at the opposite corner. From our perspective, the UE capability is not changed and it is up to UE implementation to choose the SMTCs to be measured.

In this meeting, the concrete RRM impact shall be considered due to SMTC enhancements. First of all, we would like to list the potential impact RRM requirements due to different SMTC periodicities:

| Cell selection | M1 |
| --- | --- |
| Cell re-selection | Kmulti_SMTC, Tdetect,NR intra_(enh), Tmeasure, NR intra_(enh), Tevaluate, NR intra_(enh) , Ttarget_cell_SMTC_period |
| Handover | Trs |
| Connected measurement | Kmulti_SMTC |

- For cell selection, M1 impacts the measurement and evaluation of serving cell. In current spec, the number of SMTC configured by SAN is only 4. If the number of SSB is extented to 6, the M1 will be modified as below:

- M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE, - M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC 4, - M1=[x] if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC 6 - otherwise M1=1. Where, NSMTC is the number of SMTCs configured by SAN.

- Proposal 1: For cell selection, the M1 value shall be changed as :

- M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE, - M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC 4, - M1=[x] if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC 6 - otherwise M1=1. Where, NSMTC is the number of SMTCs configured by SAN.

- For cell re-selection, we find that M2 values will impact the cell re-selection requirements. The same manner as M1, we would like to modify M2 values as below:

- For normal measurement: - M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 6 upon more than 1 SMTC configured at the UE; - M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; - otherwise M2=1.

- For enhanced measurement, we believe that the current requirements shall not be modified since the conditions are proper for enhanced SMTC periodicities.

- Kmulti_SMTC shall also be considered changing. In legacy, the collision of SMTC mainly caused by different offsets. In this case, the different SMTC periodicities are introduced. The collision will be more diverse. RAN4 shall consider one case that two SMTCs with different periodicities are collided.

- Proposal 2: For cell re-selection, M2 values shall be considered:

- For normal measurement: - M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 6 upon more than 1 SMTC configured at the UE; - M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; - otherwise M2=1.

- For enhanced measurement, we believe that the current requirements shall not be modified since the conditions are proper for enhanced SMTC periodicities.

- Proposal 3: RAN4 shall consider one case that two SMTCs with different periodicities are collided in defining new Kmulti_SMTC .

- For handover, the current definition of Trs is SMTC periodicity of the target cell if the UE has been provided with an SMTC. However, two SMTC with different periodicities and different offsets will be configured for DL CE feature. Thus the definition shall be changed as :

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with two kinds of SMTC configurations for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- Proposal 4: For handover, Trs shall be modified as:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with two kinds of SMTC configurations for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- One thing shall be noted that if SSB periodicity is larger than SMTC periodicity, the UE consumption will increase and the measurement is meaningless from UE perspective. Thus, the correct case is SMTC periodicity is the same as SSB periodicity or larger than SSB periodicity.

- Proposal 5: The correct case is SMTC periodicity is the same as SSB periodicity or larger than SSB periodicity.

- From our perspective, we shall not force the default SMTC periodicity is the largest one since the measurement delay will be totally extended. The larger SMTC periodicity is the correct case but it does not mean the SMTC periodicity same as SSB periodicity is not proper or mean that the larger SMTC periodicity is mandated.

- Proposal 6: RAN4 shall not force the default SMTC periodicity is the largest one since the measurement delay will be totally extended.

- For measurement in connected state, the Kstaellite and Kmulti_SMTC shall also be changed based on different SMTC periodicities like in RRC IDLE state.

- Proposal 7: The Kstaellite and Kmulti_SMTC shall also be changed based on different SMTC periodicities like in RRC IDLE state.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN downlink coverage enhancement.

- Proposal 1: For cell selection, the M1 value shall be changed as :

- M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE, - M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC 4, - M1=[x] if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 4<NSMTC 6 - otherwise M1=1. Where, NSMTC is the number of SMTCs configured by SAN.

- Proposal 2: For cell re-selection, M2 values shall be considered:

- For normal measurement: - M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 6 upon more than 1 SMTC configured at the UE; - M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; - otherwise M2=1.

- For enhanced measurement, we believe that the current requirements shall not be modified since the conditions are proper for enhanced SMTC periodicities.

- Proposal 3: RAN4 shall consider one case that two SMTCs with different periodicities are collided in defining new Kmulti_SMTC .

- Proposal 4: For handover, Trs shall be modified as:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with two kinds of SMTC configurations for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- Proposal 5: The correct case is SMTC periodicity is the same as SSB periodicity or larger than SSB periodicity.

- Proposal 6: RAN4 shall not force the default SMTC periodicity is the largest one since the measurement delay will be totally extended.

- Proposal 7: The Kstaellite and Kmulti_SMTC shall also be changed based on different SMTC periodicities like in RRC IDLE state.

- 4References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331