# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511022

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for RedCap NTN enhancement

Agenda item:	7.27.5.1

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

RAN4 shall define the RRM requirements based on the latest revised WID as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Discussion

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective.

  - 2.1 What RRM requirements are defined for RedCap UE with FR1-NTN

In above WID description, we know that RAN4 shall combine with R17/R18 RedCap WI and R17/R18 NR NTN WI to discuss this objective. Thus, we shall study the features introduced in RedCap which will impact the definition of requirements.

Issue 1-1-2: MDT requirements

For the discussion of MDT requirements, the way forward can be seen as below:

<Way Forward>: Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

We see majority companies would not like to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN, meanwhile MDT requirements have already been defined in legacy NTN. Accordingly, MDT shall be defined.

Several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs.

To my understanding, for logged MDT requirements, RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirement, among them the relative time stamp has different time duration which is values in seconds.

For measurement requirements:

The requirements specified in this clause apply for the following measurements performed and logged by the UE for MDT in RRC_IDLE: - SS-RSRP per cell, - SS-RSRQ per cell, - SS-RSRP per SSB index of the serving cell, - SS-RSRQ per SSB index of the serving cell, - best SSB index of the serving cell, - the number of SSBs with different SSB index which are above the threshold absThreshSS-BlocksConsolidation for all detected cells whose cell-ranking criterion R value is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell. The requirements apply for the measurements included in logged MDT reports and RRC connection establishment failure reports. The measurement values that are used to meet serving cell and reselection requirements as specified in clauses 4.2C.2.24.2C.2.9 shall also apply to values logged for MDT measurements in RRC_IDLE state.

We deem that the legacy measurement requirements shall be reused.

The concrete accuracy requirements (relative time stamp) can be seen as below:

For logged measurement: The relative time stamp for a logged measurement is defined as the time from the moment the MDT configuration was received at the UE until the measurement was logged. The accuracy of the relative time stamping is such that the drift of the time stamping shall be not more than ± 2 seconds per hour For RRC connection establishment failure log reporting: Relative time stamp for RRC connection establishment failure log reporting is defined as the time elapsed from the last RRC connection establishment failure to the time when the log is included in the report. The accuracy of the relative time stamping for RRC connection establishment failure log reporting is such that the drift of the time stamping shall not be larger than ± 0.72 seconds per hour and ± 10 seconds over 48 hours. For radio link failure and handover failure log reporting: Relative time stamp accuracy requirements for timeSinceFailure reported for MDT in a radio link failure or handover failure log are specified. timeSinceFailure determines the time elapsed from the last radio link failure or handover failure in NR to the time when the log is included in the report. The accuracy of the relative time stamping for timeSinceFailure is such that the drift of the time stamping shall not be larger than ± 0.72 seconds per hour and ± 10 seconds over 48 hours.

To my understanding, RAN4 shall define the related timing accuracy requirements. However, the requirements will not be impacted by the reduced UE receiving branches since the accuracy of the relative time stamping is the drift of the time stamping. Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied. Since the differences between 1Rx UE and 2Rx UE in terms of timestamp recording accuracy are primarily determined by their internal hardware configurations, not directly by the number of antennas.

Observation 1: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 2: Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied.

Proposal 1: RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

Proposal 2: For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 3: For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

  - 2.2 SMTCs and in parallel related features

RAN4 has spent several meetings in discussing the reduction in the number of UE Rx branches. Some issues are still under discussion and we would like to provide our views on the following issues.

Issue 1-2-2: SMTC adjustment in idle/inactive state

For this issue, RAN4 has to specify whether the SMTC adjustment is mandatory for RedCap UE. The clear way forward could be seen as below:

<Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

We believe that the UE behaviour on SMTC adjustment for normal UE is same as RedCap UE in NTN. UE can adjust the actual offset based on the actual propagation delay difference. Some UE vendors deem that the the current spec says “UE can...” which means this feature doesn’t clarify whether it is mandatory or optional clearly. Nonetheless, RAN2 has agreed the CR (R2-2504769) on capability for SMTC adjustment in RRC_IDLE and RRC_INACTIVE, the modification is:

SMTC adjustment for RRC_IDLE/RRC_INACTIVE It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

This CR clarifies that SMTC adjustment is optional for the UE in IDLE/INACTIVE state. So we suggest to align with RAN2’s spec, that is, SMTC adjustment in idle/inactive is kept optional for redcap UEs with NTN. From the other side, network will not control SMTC adjustment in idle/inactive state, only UE autonomously adjust SMTC based on propagation delay difference. If this feature is mandatory for redcap UE, maybe measurement is more accuracy and it has little impact to network. On the contrary, network controlled SMTC adjustment is up to network decision.

Proposal 4: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

  - 2.3 The impact of HD-FDD

Compared to FD-FDD UE, HD-FDD operates by using separate frequency bands for uplink and downlink communication, but with a UE only being able to either transmit or receive at any given time – not both simultaneously. In RedCap WI, they discussed the several requirements for HD-FDD UE and define lots of applicable conditions not the new requirements.

Issue 1-3-5: The impact on NR handover due to HD-FDD

In last meeting, the basic way forward could be seen as below:

- <Way Forward>: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU Option 2: No spec impact - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

First of all, we shall combine RAN1’s agreement to analyze this issue, RAN1’s agreement could be seen as below:

RAN1#116: Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching RAN1#117: Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

For option 1, we think the legacy applicability can be reused for RACH-based/RACH-less HO, Time/location-based CHO and Satellite switching with re-synchronization. In addition, since there is no Tsearch in CHO with L3 measurement in existing requirements.

The proponent for option 2 means if availability is defined, it will cause mis-interpreted as SSB may be dropped due to UL, which is not the intention.

To sum up, option 1 and option 2 are all for supporting SSB is available and will not be dropped due to collision with UL transmission. From our perspective, option 1 is fine for us. We have two points below:

- Since it is similar to the impacts of HD-FDD in TN, and we do not see any issue to re-use them for mobility procedures in NTN.

- In legacy TN scenario, the collision cases were also discussed including case 5. However, the principle of collision situations in TN does not affect the requirements applicability in RAN4. Meanwhile, we believe that the collision principle and applicability are double guarantee for SSB/requirements availability for HD-FDD NTN UE.

In conclusion, we strongly support option 1, that means the same manner shall be followed. Otherwise, we think there is no requirements for HD-FDD NTN UE.

Observation 3:

- Since option 1 is similar to the impacts of HD-FDD in TN, and we do not see any issue to re-use them for mobility procedures in NTN.

- In legacy TN scenario, the collision cases were also discussed including case 5. However, the principle of collision situations in TN does not affect the requirements applicability in RAN4. Meanwhile, we believe that the collision principle and applicability are double guarantee for SSB/requirements availability for HD-FDD NTN UE.

Proposal 5: The requirements applicability (option 1) of legacy HD-FDD TN UE shall be reused for HD-FDD NTN UE. Otherwise, there is no requirement for HD-FDD NTN UE.

  - 2.4 UE Capability

In the last meeting, we discussed the UE capability for redcap UE, the way forward as follows:

<Way Forward>: Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) FFS: The Rel-19 requirement can be release independent from Rel-17. Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

For the RedCap UE with FR-NTN bands capability, RAN1 has also discussed the in previous meeting as below:

We observed that RAN1 is still under discussion on UE capability. The prerequisite feature groups include feature 26 ( R17 NR_NTN_solutions) , feature 28 (R17 NR_RedCap) and feature 48 (R18 NR_redcap_enh). Hence, option 1 seems aligned with RAN1’s consideration. But option 3 is more rigorous. From our perspective, we could wait for RAN1’s final decision on defining UE capability though option 1 is aligned with RAN1.

Proposal 6: RAN4 shall wait for RAN1 final decision on defining UE capability though option 1 is aligned with RAN1.

To my understanding, if the UE supports both RedCap feature and FR1-NTN feature, the requirements defined in R19 can be used. We observed that the latest WID has a description ”Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands”. The UE is R17 and R18 RedCap UE with NR FR1-NTN bands. Thus, we could say R17/R18 UE supporting RedCap and NTN could meet the requirements defined in R19.

Proposal 7: R17/R18 UE supporting RedCap and NTN could meet the requirements defined in R19.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR RedCap NTN .

Observation 1: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 2: Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied.

Proposal 1: RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements.

Proposal 2: For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 3: For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

Proposal 4: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

Observation 3:

- Since option 1 is similar to the impacts of HD-FDD in TN, and we do not see any issue to re-use them for mobility procedures in NTN.

- In legacy TN scenario, the collision cases were also discussed including case 5. However, the principle of collision situations in TN does not affect the requirements applicability in RAN4. Meanwhile, we believe that the collision principle and applicability are double guarantee for SSB/requirements availability for HD-FDD NTN UE.

Proposal 5: The requirements applicability (option 1) of legacy HD-FDD TN UE shall be reused for HD-FDD NTN UE. Otherwise, there is no requirement for HD-FDD NTN UE.

Proposal 6: RAN4 shall wait for RAN1 final decision on defining UE capability though option 1 is aligned with RAN1.

Proposal 7: R17/R18 UE supporting RedCap and NTN could meet the requirements defined in R19.

- References

[1] R4-2508287，WF on RRM requirements for NR_NTN_Ph3_Part1.

[2] TS38.133

[3] TS38.304

[4] TS38.331