# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511025

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for less than 5MHz in NTN enhancement

Agenda item:	7.6.4.5

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. We can see the main task on RRM performance part for less than 5MHz NTN as follows:

Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this paper we will provide our views on RRM performance requirements on less than 5MHz NTN.

Discussion

In last meeting, we have already confirmed the measurement accuracy requirements and common testing configuration. For test cases, there are still some remaining issues to be discussed in following table.

| Test case | Requirements | Configurations | Core requirements to be defined |
| --- | --- | --- | --- |
| Common configuration |  | 15KHz, 3MHz BW, 12 PRB SSB, GSO/NGSO |  |
| HO-1: | 6.1C.1.2 NR SAN FR1 – NR SAN FR1 Handover | unknown cell | Define HO delay requirements, which are taken extension of delay for TN less than 5MHz as baseline. TΔ is extended to [3Trs] |
|  |  | Intra-frequency |  |
|  |  | Same testing procedure as NTN can be reused |  |
|  |  | FFS: Other aspects |  |
| CHO-1 | 6.1C.2.2 NR SAN FR1 – NR SAN FR1 conditional handover or 6.1C.2.3 NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurements | FFS: With L3 measurement/ without L3 measurements | - for CHO with L3 measurements: For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to 2 Trs. - for CHO without L3 measurements: T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] |
|  |  | Time based |  |
|  |  | Unknown target cell |  |
|  |  | Intra-frequency |  |
|  |  | Other aspects |  |
| SatSwitch-1 | 6.1C.3 NR SAN Satellite switching with re-synchronization | Hard switch | Same as HO requirements |
|  |  | Unknown target cell |  |
|  |  | FFS: Other aspects: e.g. RACH based |  |
| RLM-1 | 8.1C Radio Link Monitoring for Satellite Access | RLM OOS | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | FFS: non-DRX/DRX |  |
|  |  | FFS: Other aspects |  |
| RLM-2 | 8.1C Radio Link Monitoring for Satellite Access | RLM IS | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | FFS: non-DRX/DRX |  |
| BFR-1 | 8.5C Link Recovery Procedures for Satellite Access | SSB based BFD | TN less than 5MHz is reused |
|  |  | FR1 PCell only |  |
|  |  | FFS: non-DRX/DRX |  |
|  |  | FFS: Other aspects |  |
| MR-1 | 9.2C NR intra-frequency measurements for SAN | with SSB index detection | The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended by 4 and 3 for intra-frequency (Es/Iot≥-6 dB) and inter-frequency (Es/Iot≥-4 dB) respectively. Same as legacy. |
|  |  | FFS: Non-DRX/DRX |  |
|  |  | FFS: No gap |  |
|  |  | FFS: Other aspects |  |
| MR-2 | 9.3C NR inter-frequency measurements for SAN | With SSB index detection | The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended by 4 and 3 for intra-frequency (Es/Iot≥-6 dB) and inter-frequency (Es/Iot≥-4 dB) respectively. Same as legacy. |
|  |  | FFS: Non-DRX/DRX |  |
|  |  | FFS: No gap |  |
|  |  | FFS: Other aspects |  |
| IDLE-1 | 4.2C Cell Re-selection for NR UE for Satellite Access | FFS | No RRM requirements impact |

In this paper, we would like to provide the views from our side to remaining issues based on the above table.

Topic 1 Test cases to verify the core requirements reusing NTN

In the last meeting, we have agreed that the new test cases of cell re-selection for NTN less than 5MHz could be reused. But how to capture the detailed testing configuration and requirements shall be also discussed in this meeting, the following agreements can be seen:

- Option 1: captured in the new sub clause in NTN clause - Option 2: an additional test configuration in the exsiting sub clause for NTN

Actually, we agree with option 1, which the details shall be captured in the new sub clause in NTN clause. we have two points to support option 1:

1.﷒Based on the existing test cases on “Cell re-selection to FR1 intra-frequency NR case” and “Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements” in clause A.14, the difference is the test requirement, which depends on whether the  UE is configured with “enhancedMeasurementNGSO-r17”. We observed that the test procedure is totally same including test parameters, but RAN4 defined the different clauses. If we follow the same logic, the new sub clause in NTN clause shall be applied since TSI-NR in test requirement is different from legacy.

2.  If option 1 applies, the different feature has clear clarification on test cases. The same method as RedCap NTN in WI [NTN Phase 3], which has also introduced a new clause.

Above all, we think option1 is more proper for new test cases of cell re-selection for NTN less than 5MHz.

Observation 1: The existing test cases on “Cell re-selection tp FR1 intra-frequency NR case” and “Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements” have different sub clauses in NTN clause.

Observation 2: If option 1 applies, the different feature has clear clarification on test cases. The same method as RedCap NTN in WI [NTN Phase 3], which has also introduced a new clause.

Proposal 1: The detailed testing configuration and requirements of cell re-selection shall be captured in the new sub clause in NTN clause.

Topic 2 Test cases configurations

we have agreed the part of test cases configuration, the remaining configuration shall be discussed in this meeting based on above table.

For Handover, we have RACH-based handover and RACH-less handover, we suggest not to test every case since we know that the Tsearch and Tdelta is the same defined in core requirements. Thus , we shall only consider RACH-based case when defining test cases on handover.

Observation 3: Tsearch and Tdelta are the same between RACH-based and RACH-less andover defined in core requirements.

Proposal 2: Only RACH-based handover test configuration shall be defined.

For Conditional Handover, we have already agreed with defining time-based, intra-frequency unknown target cell. In core parts, CHO with L3 measurement and without L3 measurement shall be both considered. Since the core requirements are totally different. We observed that Tmeasure only exists in L3 measurement, Tdelta has different values. To our opinion, the test configuration for CHO with L3 measurement and CHO without L3 measurement shall be both considered.

Observation 4:  The core requirements are totally different between CHO with L3 measurement and CHO without L3 measurement.

Proposal 3: The test configuration for CHO with L3 measurement and CHO without L3 measurement shall be both studied.

For satellite switch in NTN, test cases are defined for hard satellite switch and soft satellite switch. The legacy UE capability has been defined as below:

softSatelliteSwitchResyncNTN-r18 Indicates whether UE supports soft satellite switch with re-sync, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate support of hardSatelliteSwitchResyncNTN-r18.

Based on above UE capability, UE supporting soft switch will also support hard satellite switch. Consequently, we prefer to consider soft satellite switch which can cover hard satellite switch. If only hard switch is tested, how do we test the duration t-service-t-seviceStart. From our perspective, soft satellite shall be tested.

Observation 5:  UE supporting soft switch will also support hard satellite switch.

Proposal 4: For satellite switch in NTN, soft satellite switch shall be tested.

For Radio Link Monitoring, RAN4 has already agreed that the PDCCH transmission bandwidth is 15PRBs and 12PRBs for SSB/PBCH assumption. In current spec, it says “the requirements in clause 8.1C apply for radio link monitoring on PCell and the UE is configured with only PCell”, the same situation for NTN less than 5MHz.

The last aspect is whether the DRX is needed or not, we found the test cases for less than 5MHz in TN as below:

A.6.5.1.13 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW A.6.5.1.14 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW

We strongly suggest that the same manner on DRX mode or non-DRX mode shall be followed as above.

Observation 6: RAN4 has already agreed that the PDCCH transmission bandwidth is 15PRBs and 12PRBs for SSB/PBCH assumption.

Observation 7: In current spec, it says “the requirements in clause 8.1C apply for radio link monitoring on PCell and the UE is configured with only PCell”.

Observation 8: For less than 5MHz in TN, RLM OOS used the DRX mode and IS used the non-DRX mode.

Proposal 5: For RLM, we raise the following test configurations:

RLM-1: FR1 PCell only, in DRX mode, 15PRB PDCCH and 12 PRB SSB RLM-2: FR1 PCell only, in non-DRX mode, 15PRB PDCCH and 12 PRB SSB

For BFR, the same situation in RLM. Consequently, we propose the following:

Proposal 6: For BFR-1,the test configuration shall include FR1 PCell only, 15PRB PDCCH and 12PRB SSB in non-DRX mode.

For intra-frequency measurement and inter-frequency measurement, some companies proposed that the inter- frequency measurement shall also consider the SSB index. However, the inter-frequency measurement requirements are also reused the less than 5MHz in TN network:

- SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

The measurement samples are also extended by different Es/Iot conditions. But the legacy test cases in TN can be seen as follows:

- Intra-frequency L3 measurement with SSB index reading without DRX - Inter-frequency L3 measurement without SSB index reading with DRX

By comparison, we think the motivation for inter-frequency L3 measurement with SSB index is not clear enough. From our side, we strongly suggest to follow the same manner as test configuration in TN.We propose the following:

Proposal 7:

Intra-frequency L3 measurement with SSB index reading without DRX, no gap Inter-frequency L3 measurement without SSB index reading with DRX, with gap.

For cell re-selection, although there is no core part impact on cell re-selection requirements, regarding there is no exact Tsi-nr given in current spec, only the testing parameter will be given. Thus, we deem that the testing parameter shall be defined. In legacy TN less than 5M WI, the reason to define new test case for cell re-selection is that UE needs to read SIB/MIB from the target cell, which is same in HO. RAN4 did not define new core requirements because the exact delay for UE to read SIB/MIB is included in Tsi but the exact value is not specified. In the test, Tsi is defined as 1280ms as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

Since the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH, thus, the legacy test requirements on less than 5M TN can be seen as below:

Legacy TN with less than 5M for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

We observe that the exact value in test requirements for legacy NTN on Tsi is as below:

Legacy TN test requirements for Tsi: TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

The same logic shall be considered when defining cell re-selection test requirements with less than 5M in NTN scenario.

Observation 10: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 11: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 8: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

Conclusion

In this paper we provided our views on RRM performance requirements on R19 NR less than 5M NTN enhancement.

Observation 1: The existing test cases on “Cell re-selection tp FR1 intra-frequency NR case” and “Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements” have different sub clauses in NTN clause.

Observation 2: If option 1 applies, the different feature has clear clarification on test cases. The same method as RedCap NTN in WI [NTN Phase 3], which has also introduced a new clause.

Proposal 1: The detailed testing configuration and requirements of cell re-selection shall be captured in the new sub clause in NTN clause.

Observation 3: Tsearch and Tdelta are the same between RACH-based and RACH-less andover defined in core requirements.

Proposal 2: Only RACH-based handover test configuration shall be defined.

Observation 4:  The core requirements are totally different between CHO with L3 measurement and CHO without L3 measurement.

Proposal 3: The test configuration for CHO with L3 measurement and CHO without L3 measurement shall be both studied.

Observation 5:  UE supporting soft switch will also support hard satellite switch.

Proposal 4: For satellite switch in NTN, soft satellite switch shall be tested.

Observation 6: RAN4 has already agreed that the PDCCH transmission bandwidth is 15PRBs and 12PRBs for SSB/PBCH assumption.

Observation 7: In current spec, it says “the requirements in clause 8.1C apply for radio link monitoring on PCell and the UE is configured with only PCell”.

Observation 8: For less than 5MHz in TN, RLM OOS used the DRX mode and IS used the non-DRX mode.

Proposal 5: For RLM, we raise the following test configurations:

RLM-1: FR1 PCell only, in DRX mode, 15PRB PDCCH and 12 PRB SSB RLM-2: FR1 PCell only, in non-DRX mode, 15PRB PDCCH and 12 PRB SSB

Proposal 6: For BFR-1,the test configuration shall include FR1 PCell only, 15PRB PDCCH and 12PRB SSB in non-DRX mode.

Proposal 7:

Intra-frequency L3 measurement with SSB index reading without DRX, no gap Inter-frequency L3 measurement without SSB index reading with DRX, with gap.

Observation 10: The legacy test requirements without less than 5MHz in TN scenario, the legacy delay as 1280ms is not enough to accommodate longer delay for SIB/MIB reading due to punctured PBCH.

Observation 11: The legacy test requirements with less than 5MHz in TN scenario, TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case.

Proposal 8: The concrete test requirements could be seen as below:

TSI-NR Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1320 ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20 ms period and 80 ms period, respectively.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133

- R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi.