3GPP TSG-RAN WG4#116	R4-2511054

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	6.11.2

Source: 	Qualcomm

Title: 	CA_n5A-n8A

Document for:	Approval

# Introduction

Aspects for CA_n5A-n8A are discussed in this contribution.

- WF in RAN4-115

WF was agreed in [1].

<Way forward>:

For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19:

Support case 1 and case 2 via RRC reconfiguration.

- Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated.

- Case 2: 1UL/2DL configuration

Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19.

Whether new capability is needed or not would be further discussed.

- Discussion

Case 2 is already specified in Rel-18 so we don’t expect any issues with it. Case 1 is obviously a new thing which requires investigations. Basically, even it is “2UL/2DL”, there are lot of similarities to “2UL/1DL” which currently is not specified. Based on the several discussions during Rel-18 and now during Rel-19, the group realises this combination is a very specific one. The viewpoint here is whether this can be supported using the current specifications rather than are current specifications made assuming this kind of combination would be supported.

Observation 1: In case 1, there are a lot of similarities to “2UL/1DL” which is currently not specified.

An obvious detail to investigate is how to conduct power control for n5 in Case 1 as there is no DL resource allocation and UE is not listening SSB’s. Any issue here would basically trigger RAN1 specification change. It seems that n5 UL can use the n8 DL for pathloss reference in power control through the configuration of pathlossReferenceLinking. This obviously requires collocation between n5 and n8 basestations.

Observation 2: Assuming n5 and n8 are collocated, power control does not seem to be an issue as n5 UL can use the n8 DL for pathloss reference in power control through the configuration of pathlossReferenceLinking.

RRM part seems trickier. We identified at least three issues which could trigger RRM specification change.

Even with restriction of no DL resource allocation, UE would expect DL resources normally without specifications changes and would reserve resources for DL.

Observation 3: According to current specifications, UE would allocate DL resources for n5 DL as just agreeing restriction of no DL resources on n5 DL would not change that.

When there is no dedicated resource allocation, the SCell (n5) activation sequence may not function as required from an RRM perspective, since activation procedure involves CSI-RS measurements from the SCell. To make n5 reception possible, some dynamic reconfiguration of RFFE would be required which was excluded earlier from Rel-19 scope.

Observation 4: Making CSI-RS measurements does not seem viable as in Case 1 there is no n5 reception possibilities.

Additionally, SSB reception is necessary during serving/neighbor cell L3 measurements and SCell activation if the SCell operates in a different band than any of the currently activated serving cells. SSB is present as it is broadcasted by the network, but in Case 1 there is no  n5 reception possibility, so as with CSI-RS measurements some dynamic reconfiguration of RFFE would be required which was excluded earlier from Rel-19 scope.

Observation 5: SSB reception is necessary during serving/neighbor cell L3 measurements and SCell activation if the SCell operates in a different band than any of the currently activated serving cells.

In conclusion, Observation 3 , 4, and 5 seem challenging. The group needs to discuss whether RRM impacts due to these observations can be avoided or not. As agreed in WF, in case RRM impacts can be avoided then this can be specified in Rel-19 but if there are RRM (or RAN1) impacts, the this is not pursued in Rel-19.

Proposal 1: The group needs to discuss whether RRM impacts due to at least observations 3, 4, and 5 can be avoided or not.

In case this combination is specified, i.e. there are no RRM or RAN1 impacts, we don’t see any reason for and additional UE capability. It suffices to have a note in UL CA configuration table stating “When UL CA_n5A-n8A is configured, there is no reception at n5DL”.

Proposal 2: There is no need to an additional UE capability if this UL CA band combination is specified. Restriction of n5DL reception when UL CA is configured can be handled by a note in UL CA configuration table.

- Conclusion

Considerations on CA_n5A-n8A with UL CA_n5A-n8A were considered with the following observations and proposals.

Observation 1: In case 1, there are a lot of similarities to “2UL/1DL” which is currently not specified.

Observation 2: Assuming n5 and n8 are collocated, power control does not seem to be an issue as n5 UL can use the n8 DL for pathloss reference in power control through the configuration of pathlossReferenceLinking.

Observation 3: According to current specifications, UE would allocate DL resources for n5 DL as just agreeing restriction of no DL resources on n5 DL would not change that.

Observation 4: Making CSI-RS measurements does not seem viable as in Case 1 there is no n5 reception possibilities.

Observation 5: SSB reception is necessary during serving/neighbor cell L3 measurements and SCell activation if the SCell operates in a different band than any of the currently activated serving cells.

Proposal 1: The group needs to discuss whether RRM impacts due to at least observations 3, 4, and 5 can be avoided or not.

Proposal 2: There is no need to an additional UE capability if this UL CA band combination is specified. Restriction of n5DL reception when UL CA is configured can be handled by a note in UL CA configuration table.

- Reference

[1] R4-2507910, “WF on simultaneous Rx/Tx requirements”, Huawei