3GPP TSG-RAN WG4 Meeting #116					 R4-2511067 
Bengaluru, India, 25th Aug, 2025 – 29th May, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Agenda item:	 7.13.3

Document for:	Discussion

# 1.Introduction

### In [1] and in earlier RAN4 meetings, RAN4 agreed to scenarios and test settings for multiple performance requirements. The final alignment of PDSCH and CQI requirements is open and shall be completed in this meeting. In this paper, we share simulation results and our view on receiver definition.

# 2. PDSCH Demodulation Requirements inter-cell interference (ICI)

## 2.1 Simulation results

### Initial simulation results for inter-cell interference FDD and TDD scenarios are respectively shared in Table 2.1, and Table 2.2 with the test settings agreed in earlier RAN4 meetings.

Table 2.1 FDD simulation results

| Setting | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) |
| --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 8.7 | 11.1 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 8.3 | 12.3 |

Table 2.2 TDD simulation results

| Serving cell | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) |
| --- | --- | --- | --- | --- |
| Serving cell Rank 2 + 2 interfering cells | TDLA 30-10 | 17 | 9.2 | 11.2 |
| Rank 4 serving cell + 1 interfering cell | TDLA 30-10 | 13 | 8.7 | 12.9 |

# 3. PDSCH Demodulation Requirements intra-cell inter-user interference

## 3.1 Simulation results

Initial simulation results for Rank2 for both target UE and co-scheduled UE are shared in Table 3.1.

Table 3.1 Intra-cell and Inter-user interference simulation results

| Setting | Channel | Duplex | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Rank 2 Target UE + Rank2 co-scheduled UE | TDLA30-10 | FDD | 17 | 11.2 | 13.7 |
| Rank 2 Target UE + Rank2 co-scheduled UE | TDLA 30-10 | TDD | 17 | 11.3 | 13.7 |

# 4. CQI requirements

In [1] Rank2 from serving cell along with Rel-17 interference settings were agreed for initial simulations. Table 4.1 and Table 4.2 has simulation results.

Table 4.1 Gamma values for FDD CQI scenarios

|  | Rank2 serving cell |  |
| --- | --- | --- |
| SINR (dB) | Baseline Gamma (A) | Simplified Gamma (B) |
| -2.0 | 2.7 | 2.5 |
| -1.0 | 3.0 | 2.4 |

Table 4.2 Gamma values for TDD CQI scenarios

|  | Rank2 serving cell |  |
| --- | --- | --- |
| SINR (dB) | Baseline Gamma (A) | Simplified Gamma (B) |
| -2.0 | 2.9 | 2.6 |
| -1.0 | 3.2 | 2.7 |

Since the span among submitted gamma values in RAN4#115 was tighter for SINR = -2 dB, SNR point is tentatively agreed to be -2 dB. With a single Gamma requirement for both receivers, 80% of the minimum among gamma values of both simplified and baseline is a reasonable choice to specify the requirement. Based on results submitted so far,

Proposal 1: RAN4 to use Gamma = 2 for FDD CQI requirements.

Proposal 2: RAN4 to use Gamma = 2.1 for TDD CQI requirements.

# 5 Receiver definition

Definition of MMSE-IRC receiver capability for 8Rx UE

Options:

- Option 1: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 2: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 3: 
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 4:
Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

- Option 5:
Re-use existing definition in Rel-18.

Since the default receiver has always been MMSE-IRC, we don’t think that this needs to be explicitly mentioned in the definition. For advanced receivers that can do more than MMSE-IRC, explicit capability and name was captured in specification so far. Therefore, our preference is for Option 3.

Proposal 3: RAN4 to choose Option 3.

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

# 7. Conclusions

Following is the summary of the proposals:

Proposal 1: RAN4 to use Gamma = 2 for FDD CQI requirements.

Proposal 2: RAN4 to use Gamma = 2.1 for TDD CQI requirements.

Proposal 3: RAN4 to choose Option 3.

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

# References

[1] R4-2508620, “Way Forward for [115][323] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, RAN4#115, May 19 – May 231, 2025, Malta.