3GPP TSG-RAN WG4 Meeting # 116															R4-2511068

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.20.3

Source:	Samsung

Title:	Views on remaining issues of NR MIMO Phase 5

Document for:	Discussion

1	Introduction

RAN4 has discussed UE RF aspects of Rel-19 NR MIMO Phase 5 focusing on the objectives of 3-antenna-port transmissions in FR1 and beam correspondence requirements for UL mTRP in FR2 in the WID [1]. Thanks to the companies who have contributed and compromised until the last meeting, RAN4 has achieved considerable progress and come to a close for the technical discussion as endorsed in [2]. In this contribution, we would like to share additional views on remaining issues based on the latest WF approved in [3].

2	Discussion

2.1	FR1 (3Tx)

Regarding the MPR requirements for each power class for 3Tx, in the beginning of the WI, it was first discussed how to derive MPR requirement for 3Tx power classes given the small gap between 1Tx (Table 6.2.2-2) and 2Tx (Table 6.2D.2-1) MPRs for PC2 as discussed in the summary copied below [4]. RAN4 finally reached a consensus to reuse the single carrier PC3 MPR for 3Tx PC3 [5] and 2Tx PC2 MPR for 3Tx PC2 [6].

Observation 1:	RAN4 finally reached a consensus on legacy 2Tx PC2 MPR for 3Tx PC2 MPR between 1Tx (Table 6.2.2-2) and 2Tx (Table 6.2D.2-1) for PC2.

Proposal 1:		RAN4 to take values of Table 6.2D.2-1 for 3Tx PC MPR requirements as endorsed in R4-2508115.

2.2	FR2 (UL mTRP)

For the beam correspondence requirement considering UL mTRP under the objective of asymmetric DL sTRP/UL mTRP, RAN4 has discussed whether it has impact on the UE RF requirement in Rel-19, and got a confirmation that it can also be considered to transmit SSB from the UL only TRP in this WI [7].

However, there would be no motivation to introduce PL offset to identify pathloss between the UL TRP and the UE if the UL TRP can receive SSBs (Scenario 4). Also, as shown in Figure 1, Scenario 4 is definitely the same scenario with Scenario 1 for sTRP operation of Rel-15 and Scenario 2 for general mTRP operation of Rel-16 in the view of the beam correspondence requirement.

Figure 1: Beam correspondence under multi-TRP operations

In our view, it does not make sense to have the duplicate requirement for different feature for RAN4. It should not be considered until it shows any difference between scenarios in Figure 1 on the beam correspondence requirements. If some companies still want it without a technical reason, it should be applicable to all UL mTRP scenarios enabled from the previous release although it may worsen the quality of the specification.

Observation 2:	Scenario 4 is definitely the same scenario with Scenario 1 for sTRP operation of Rel-15 and Scenario 2 for general mTRP operation of Rel-16 in the view of the beam correspondence requirement.

Proposal 2:		It should not be considered until it shows any difference between scenarios in Figure 1 on the beam correspondence requirements.

3	Conclusion

This contribution provides a summary of WI discussions and some takeaways along the UE RF part based on the previous WFs. Following observations and proposals were derived for consideration in this meeting.

Observation 1:	RAN4 finally reached a consensus on legacy 2Tx PC2 MPR for 3Tx PC2 MPR between 1Tx (Table 6.2.2-2) and 2Tx (Table 6.2D.2-1) for PC2.

Proposal 1:		RAN4 to take values of Table 6.2D.2-1 for 3Tx PC MPR requirements as endorsed in R4-2508115.

Observation 2:	Scenario 4 is definitely the same scenario with Scenario 1 for sTRP operation of Rel-15 and Scenario 2 for general mTRP operation of Rel-16 in the view of the beam correspondence requirement.

Proposal 2:		It should not be considered until it shows any difference between scenarios in Figure 1 on the beam correspondence requirements.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2508115,	draft CR for TS 38.101-1 to introduce FR1 3Tx RF requirements, RAN4#115

[3] R4-2507960,	WF on NR MIMO Phase 5 UE RF, RAN4#115, RAN4#115

[4]	R4-2500685,	Topic summary for [114][133] NR_MIMO_Ph5_UE, RAN4#114

[5] R4-2502951,	WF on UE RF requirements for MIMO enhancement, RAN4#114

[6]	R4-2505111,	WF on UE RF requirements for MIMO enhancement phase 5, RAN4#114-bis

[7] R4-2500005,	Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN4#114