3GPP TSG-RAN WG4 Meeting #116															R4-2511072

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.3.3

Source:	Samsung

Title:	Views on remaining issues on low band carrier aggregation via switching

Document for:	Discussion

1	Introduction

RAN4 has discussed a Rel-19 Work Item (WI) on low band (LB) carrier aggregation (CA) via switching, which was initiated to address a market demand for LB utilizations [1]. At the latest meeting, RAN4 achieved concrete progress such as time mask with transient period, switching period and switching gap as captured in [2]. It has been also discussed the technical content of the CR to 38.101-1 given the agreements made in both RAN4 and RAN1 considering the time limitation of the Rel-19 WI. Building on the previous discussion, in this contribution, we would like to provide our views focusing on the remaining issues from UE RF aspects.

2	Discussion

In [2], following agreements are captured:

Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered. Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

Based on the agreements and issues above, a draft CR to introduce the feature of low NR band aggregation via switching has been discussed with following potential changes:

- Change 1: clarify the applicability of simultaneous Rx/Tx requirements to band combinations which are supported via switching in Clause 5.2A.2 - Change 2: introduce the indication of support via switching for the example band combination (CA_n5A-n29A) in Clause 5.5A.3.1 - Change 3: define the applicability of the ON/OFF time mask for carrier aggregation via switching in Clause 6.3A.3.3.1 - Change 4: define the ON/OFF time mask for the UL transmissions in the new Clause 6.3A.3.3.7. - Change 5: provide a reference for the RMCs applicable to REFSENS testing of devices which support low band aggregation via switching - Change 6: define the SCS15 FDD and SDL RMCs applicable to REFSENS testing of devices which support low band aggregation via switching

2.1	Change 1: Simultaneous Rx/Tx for CA band table

In clause 5.2A.2, inter-band CA operating bands involving FR1 are specified including the applicability of simultaneous Rx/Tx requirements to band combinations as follows:

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, Table 5.2A.2.2-1, Table 5.2A.2.3-1, Table 5.2A.2.4-1 and Table 5.2A.2.5-1, where all operating bands are within FR1. If the mandatory simultaneous Rx/Tx capability applies for a lower order band combination, when the applicable lower order band combination is a band pair in a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order band combination. Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD and TDD-SDL CA combinations. Simultaneous Rx/Tx capability is mandatory without signaling for FR1+FR1 FDD-FDD and FDD-SDL CA combinations.

Since the simultaneous Rx/Tx capability should not be considered for this switching-based feature between two low bands, some clarifications are necessary to avoid any confusion for the case of FDD-SDL CA combinations which are mandatory to support simultaneous Rx/Tx capability without signaling.

Observation 1:	Some clarifications are necessary to avoid any confusion for the case of FDD-SDL CA combinations which are mandatory to support simultaneous Rx/Tx capability without signaling.

We support the proposed clarification, which is not to apply simultaneous Rx/Tx capability for low NR band inter-band configurations supported via switching based on the capability indication.

Proposal 1:		It is supported by the proposed clarification, which is not to apply simultaneous Rx/Tx capability for low NR band inter-band configurations supported via switching based on the capability indication.

Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD and TDD-SDL CA combinations. Simultaneous Rx/Tx capability is mandatory without signaling for FR1+FR1 FDD-FDD and FDD-SDL CA combinations. For low NR band inter-band configurations supported via switching [supportedLowBandSwitching-r19], the simultaneous Rx/Tx capability does not apply. NOTE 21: Simultaneous Rx/Tx is not applicable to UEs indicating support of low NR band aggregation via switching [supportedLowBandSwitching-r19] for this band combination

2.2	Change 4: Time mask figure

Regarding the time mask for switching, at the last meeting, a figure for ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL was agreed to be captured in TR 38.768, which is considering both transient period and switching period [3]. Given the agreement, two alternatives have been proposed for further discussion, which does not run counter each other:

Figure 1: Time mask agreed in RAN4#115 (Alt-1)

Figure 2: Proposed time mask for RAN4#116 (Alt-2)

As an alternative in addition to Figure 1, Figure 2 is to consider a RAN1 agreement, which is that the end of transmission part of the mask is not actually end of slot.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier.

In our view, Figure 2 would be more suitable than the original figure in [3] to avoid any unnecessary confusion based on the RAN1 agreement on the switching pattern for switching from FDD carrier to SDL carrier. We also believe that any additional information beyond transient period and switching period is not required for CR to be aligned with existing time mask requirements in the clause. Such information including DL carrier relation, TA or switching gap are demonstrated in other specifications, but basically not about UL requirements/characteristics of UE RF specification, also have not been considered there. In that sense, Figure 2 should be the baseline for further CR discussion while others such as the relationship with DL FDD carrier or switching gap can be captured in the TR as they are already there.

Observation 2:	Figure 2 would be more suitable than the original figure in [x] to avoid any unnecessary confusion based on the RAN1 agreement on the switching pattern for switching from FDD carrier to SDL carrier.

Proposal 2:		Figure 2 should be the baseline for further CR discussion while others such as the relationship with DL FDD carrier or switching gap can be captured in the TR as they are already there.

2.3	Change 6: RMC for REFSENS

Another discussion point raised after the last meeting is about RMC, which is the reference measurement channels to evaluate ≥ 95 % of the maximum throughput to define reference sensitivity requirements. Some companies argue that new RMC for low NR band carrier aggregation via switching is required based on the semi-static RRC configuration switching pattern is totally different with it for legacy RMCs for testing. Also, since all FDD UL RMC are 100% UL, it is proposed to have new UL RMC that respects the RAN1 pattern design for switching time mask.

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

However, in our view, it could be a complicated discussion to have a common reference channel, which is also related to other parts of discussion, e.g., RAN1 or RAN4 performance spec. Given the situation of this WI discussion so far, RMC for REFSENS performance could be discussed at the last meeting of the WI, but it also could be continued until the performance part considering the impact on other specifications.

Observation 3:	Given the situation of this WI discussion so far, RMC for REFSENS performance could be discussed at the last meeting of the WI, but it also could be continued until the performance part considering the impact on other specifications.

3	Conclusion

In this contribution, we provide our views focusing on the UE RF aspects taking into account the WID assumption and discussion status across WGs. Following observations and proposals were derived.

Observation 1:	Some clarifications are necessary to avoid any confusion for the case of FDD-SDL CA combinations which are mandatory to support simultaneous Rx/Tx capability without signaling.

Proposal 1:		It is supported by the proposed clarification, which is not to apply simultaneous Rx/Tx capability for low NR band inter-band configurations supported via switching based on the capability indication.

Observation 2:	Figure 2 would be more suitable than the original figure in [x] to avoid any unnecessary confusion based on the RAN1 agreement on the switching pattern for switching from FDD carrier to SDL carrier.

Proposal 2:		Figure 2 should be the baseline for further CR discussion while others such as the relationship with DL FDD carrier or switching gap can be captured in the TR as they are already there.

Observation 3:	Given the situation of this WI discussion so far, RMC for REFSENS performance could be discussed at the last meeting of the WI, but it also could be continued until the performance part considering the impact on other specifications.

Reference

[1] 	RP-243317,	New WID on low band carrier aggregation via switching, RAN#106

[2]	R4-2508118,	WF on NR_LBCA_Sw, RAN4#115

[3]	3GPP TR 38.768 V1.0.0