3GPP TSG-RAN WG4 Meeting #116                                    R4-2511081

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:              Discussion on RRM requirements for NES OD-SSB

Agenda item:			7.23.3.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES OD-SSB. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1]. The corresponding RAN1& RAN2 agreements update, please find in Appendix

2 Discussion

OD-SSB based deactivated SCell L3 measurement (Case 1)

2.1 Sub-issue 2: OD-SSB in FMW collided with MG

Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

From our understanding, when measurement opportunities are punctured by MG under deactivated SCell status, the principle analysis of UE behaviour for OD-SSB based deactivated SCell L3 measurement (case 1) is not dissimilar to what we discussed in the legacy, just the discrepancy between the two types of measurements need to be considered, namely, the type of RS, the RS transmission characteristics and the measurement window. Particularly, for OD-SSB based deactivated SCell measurement here, UE shall perform measurement according to the activated OD-SSB based on the configured SCell MO after reception of the activation command. Besides, answer from the legacy discussion: the existence of MG in deactivated SCell status and how to treat the case that SMTC and MG are partially overlapping are clear. In this sense, there is no need to define priority-based rules for collision handling between OD-SSB in FMW and MG, but to introduce a scaling factor. On the other hand, based on the discussion in RAN1 and RAN2, OD-SSB specific SMTC is, to some extent, benign. From this, we think the scope of the existing definition of scaling factor Kp can cover OD-SSB based deactivated SCell measurement.

Proposal 1: For OD-SSB based deactivated SCell L3 measurement (Case 1), when OD-SSB in FMW collided with MG,

- UE is allowed to skip the OD-SSB measurement in the overlapping occasion if FMW collides with MG occasion, a scaling factor is applied to scale the measurement period

  - Clarification on the existing definition of Kp applies to OD-SSB based deactivated SCell measurement can be needed

2.2 Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs

Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

From our understanding, the discussion on the requirement corresponding to the case that: when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs is a little trivial. That since: 1) The start point of T2 is the time point of OD-SSB deactivation, while based on the RAN1 agreement: Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. 2) The intention of defining the minimum interval between FMWs as the agreement can mean that the same function as the condition of known cell is also expected here, and more NW power saving gain can be obtained, accordingly, otherwise, the meaning of introducing T2 is not that much huge. From this, in some sense, OD-SSB further activation/reconfiguration occurs within T2 between two FMWs can be treated as a corner case, and our view is that if such case truly happens, it is feasible for UE that to ignore the OD-SSB activation command, and to follow legacy deactivated SCell measurement after T2.

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2

OD-SSB based deactivated SCell L3 measurement (Case 2)

2.3 Issue 1-2-2: Requirement for RAN1 Alt Time-C2

Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

As per RAN1 agreement guidance, for the case when the center frequency of AO-SSB and OD-SSB is the same, the relation in terms of time location between AO-SSB and OD-SSB can be categorized into

- Time-C1：a union of periodic time domain pattern

- Time-C2: the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern,

depending on NW deployment, with at least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB. And for Time-C2, half-frame index may be different for AO-SSB and OD-SSB.

From our understanding, although it is not clear what is the use case of AO-SSB and OD-SSB are distributed with non-equal intervals in the practical deployment, from UE side, even the two sets of SSBs are configured to transmit in the same time occasion, UE does not need to distinguish the SSBs since they are the same. And based on the RAN1 agreement: SS/PBCH blocks with the same SSB indexes according to OD-SSB configuration and indication are QCLed, the possibility of combining the measurements of both SSBs configuring the same PBCH payload does hold under some cases, while, to avoid UE implementation complexity, if to define the requirement for Time-C2, it is suggested to define the requirement based on OD-SSB periodicity.

Proposal 3: RAN4 to define the requirement for RAN1 Alt Time-C2 based on the OD-SSB periodicity

2.4 Sub issue 2: Requirement for scenario 3

Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

2.4.1 Information obtained from RAN1 and RAN2

Fig. 3 RAN1 and RAN2 discussion on scenario 3

- Different from RedCap configuration, in which, the IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

  - OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

### [Closed] Proposal #6-4-1 (QCL for different frequency locations): - For the case when the center frequency locations of AO-SSB and OD-SSB are different, - It is ONLY supported that SSB indices within OD-SSB burst is subset of SSB indices within AO-SSB burst or equal to SSB indices within AO-SSB burst.

2.4.2 Potential issues

Before defining the requirements, what are the intra-f and inter-f under scenario 3 need to be clarified first.

Intra-frequency and inter-frequency definition

38.300 Intra-frequency and inter-frequency definition Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different. NOTE 2: For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells. NOTE 2a: If a RedCap UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

Based on the NOTE 2 above,

Observation 1: The legacy UE assumption that one MO corresponds to one SSB and SSBs in different MOs are regarded as different cells does not hold for SSB based measurement in scenario 3 any more

Besides, based on the existing definition, whether the SSB based measurement is deemed as intra-frequency or inter-frequency measurement depends on whether the center frequency of the SSB of the neighbour cell is of the same or different as the one of the serving cell (i.e., the reference SSB) and whether the SCS of the two SSBs are the same or not.

However, in case 3, from our understanding, no matter how to define inter-frequency or intra-frequency, there is one SS block of neighbor cell will be not aligned with serving cell SSB block (i,e, transmitted not on the same frequency) if two SSB blocks (AO-SSB and OD-SSB) are both within UE current operating bandwidth. In this sense, if AO-SSB and OD-SSB with different frequencies are both configured for the serving SCell measurement and :

Observation 2: For SSB-based measurement in scenario 3, it is necessary to discuss the center frequency of which SSB (i.e., AO-SSB or OD-SSB) can be taken as the reference to define intra-frequency/inter-frequency measurements

One solution:

A. Inspiration from intra-frequency measurement for Redcap, below

38.133 9.2B NR intra-frequency measurements for RedCap If the UE supports ncd-SSB-BWP-Wor-r18, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2].

from our understanding, one of possible solutions to define intra-frequency measurements for scenario 3 can be:

- When OD-SSB is activated, the reference SSB of the serving cell is the SSB defined in servingCellMO-OD under ServingCellConfig;

  - Neighbour cell measurement on serving cell OD-SSB frequency layer Intra-frequency measurement

- when OD-SSB is deactivated for the SCell, the reference SSB of the serving cell is the SSB defined in servingCellMO-AO under ServingCellConfig

  - Neighbour cell measurement on serving cell AO-SSB frequency layer Intra-frequency measurement

B. At the same time, as the current TS 38.331 says:

38.331 5.5.3.1 The UE shall: 1> whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows: 2> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO: 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb: 4> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 3> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

in theory, the UE is mandated to perform serving cell measurement when servingCellMO is configured, based on RAN2 discussions, in scenario 3, it is feasible to allow UE not to perform measurement for the new servingCellMO-OD when OD-SSB transmission is not activated. On top of such limitation, to be further discussed, the intra-frequency measurement can be interpreted as

- when OD-SSB is deactivated for the SCell (e.g., when UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated) or OD-SSB transmission is not activated,

  - Neighbour cell measurement on serving cell AO-SSB frequency layer Intra-frequency measurement

Combining with A and B, we find that distinct from Redcap analysis, in our case, the reference SSB of the serving cell is flexibly switched in line with the dynamic OD-SSB activation/deactivation command indication. How to robustly define the corresponding mobility management based on the switched intra-/inter-f measurement requirement subjecting to the changeable reference SSB of the serving cell need to be copied with carefully

Proposal 4: For SSB-based measurement in scenario 3, one of possible solutions to define reference SSB and the corresponding intra-frequency measurement can be

- When OD-SSB is activated, the reference SSB of the serving cell is the SSB defined in servingCellMO-OD under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell OD-SSB frequency layer

- When OD-SSB is deactivated for the SCell, the reference SSB of the serving cell is the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell AO-SSB frequency layer

- When OD-SSB transmission is not activated for the SCell, the reference SSB of the serving cell is the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell AO-SSB frequency layer

Proposal 5: For SSB-based measurement in scenario 3, if to define reference SSB and the corresponding intra-frequency measurment in line with the OD-SSB activation/deactivation command indication

- RAN4 to discuss how to robustly define the corresponding mobility management based on the switched intra-/inter-f measurement requirement subjecting to the changeable reference SSB of the serving cell

Another solution:

Regardless of whether OD-SSB is activated or not, the reference SSB of the serving cell is always the SSB defined in servingCellMO-AO under ServingCellConfig, one of reasons is that AO-SSB is CD-SSB and always exists, OD-SSB is NCD-SSB, and the denser periodicity of SSB can be used to expedite measurement/SCell activation process for traffic scheduling, it is demand based. In this sense,

  - Neighbour cell measurement on serving cell AO-SSB frequency layer Intra-frequency measurement

  - Neighbour cell measurement on serving cell OD-SSB frequency layer Inter-frequency measurement

Proposal 6: For SSB-based measurement in scenario 3, another possible solution to define reference SSB and the corresponding intra-frequency measurement can be

- Regardless of whether OD-SSB is activated or not, the reference SSB of the serving cell is always the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency neighbour cells measurement on serving cell AO-SSB frequency layer

Meanwhile, for intra-frequency measurement, if seving cell and neighbor cell SS block (AO-SSB or OD-SSB) overlap, are located in the same active BWP, and are within the UE operating bandwidth, no gap needed (i.e., the SSBs are completely in the active BWP of the UE). Besides, since there is a note agreed in RAN1 that UE is not required to measure both AO-SSB and OD-SSB, we can have the following baseline assumption: UE can only perform measurements on one frequency location at a time.

Proposal 7: For SSB-based measurement in scenario 3, when the two SS blocks (AO-SSB and OD-SSB) are completely located in the active BWP of the UE, if to perform OD-SSB inter-frequency/intra-frequency measurement, no gaps needed.

Proposal 8: For SSB-based measurement in scenario 3, based on the agreed Note: UE is not required to measure both AO-SSB and OD-SSB, the baseline assumption of UE can only perform measurements on one frequency location at a time can be considered

First active DL BWP

Although the SCell is configured with AO-SSB and different frequency OD-SSB, the OD-SSB and AO-SSB are located in the same BWP, and the two SSBs needs to be placed in the first active DL BWP, which is used upon activation of an SCell.

Proposal 9: For SSB-based measurement in scenario 3, the AO-SSB and OD-SSB within different frequencies need to be placed in the first active DL BWP

How UE to prioritize OD-SSB based SCell measurements

Furthermore, based on previous discussions, OD-SSB based L3 measurement is expected to be prioritized over other legacy L3 measurements when OD-SSB transmission is triggered/activated. And to ensure the fast OD-SSB based deactivated SCell L3 measurement can be carried out (factor in CSSF =1 for OD-SSB layer), the SCell’s new servingCellMO for OD-SSB needs to be prioritized even if the UE is configured with multiple serving cell MOs, accordingly. However, except for in Rel-19 RRM enhancment ph5 WI, a new enhancement mechanism of measuring one serving CC per band was introduced, in other previous releases, it is up to UE implementation to determine which MO can to measure first. In this sense, to meet OD-SSB based measurement expectation, from our understanding, there are two solutions:

Proposal 10: To enable the UE to prioritize OD-SSB based SCell measurements (e.g., FMW-based fast deactivated SCell measurement) when OD-SSB transmission is activated.

- UE to measure the servingCellMO-OD when the OD-SSB activation command is indicated

Proposal 11: Consider the mentioned issues are not easy to solve in one meeting, fine to deprioritize scenarios 3 requirement discussions in Rel-19 NES

General issues

2.5 Issue 1-4-5: Additional processing time for OD-SSB parameter update /RAN1 LS - OD-SSB deactivation timeline

Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

1. Based on the agreement we achieved in RAN4 #114-bis, the additional processing timing is only applied in deactivated SCell measurement. 2. The time instance A for MAC-CE based OD-SSB prameter update/adaptation was discussed in RAN1, and seems that a consensus regarding: the same principle as MAC-CE based activation of the SSB/PBCH blocks transmission can be applied to MAC-CE based adaptation of the SSB/PBCH blocks transmission was achieved. In this sense, for simplicity, we prefer that RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. And the applicability of the capability of additional processing time 5/2 ms can be applied to the case: When the activation or adaptation of the OD-SSB transmission is indicated by the MAC CE

Proposal 12: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The applicability of the capability of additional processing time 5/2 ms can be applied to the case: When the activation or parameter updates of the OD-SSB transmission is indicated by the MAC CE

Actually, from our understanding, OD-SSB transmission contains: OD-SSB activation, deactivation and parameters update. Theoretically, additional processing time discussed for OD-SSB activation can also be used in OD-SSB deactivation. However, in our opinion, OD-SSB deactivation timeline is relatively simpler, and the deactivation behaviors do not require any subsequent OD-SSB transmission communication with NW, while how UE to treat the “awkward” OD-SSB resource (e.g., measurement filtering/result reporting/ resources release, etc. for case 1/case 2) is an implementation specific issue and UE can have different understandings for such issue. In this sense,

Proposal 13: It is fine to clarify: No UE additional processing time for OD-SSB deactivation

2.6 RAN2 related issue: whether to introduce OD-SSB MAC CE triggered MR or not to RAN4

In last meeting, RAN2 have the following agreement

Agreements on OD-SSB - RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Recap the discussions on how to report L3 measurement results, in the current spec., reporting mechanisms containing event triggered reporting and periodical reporting are widely used. Besides, in Rel-18 FR2 CA enhancement, the type of one-time report configuration (i.e., reportOnScellActivation-r18) was defined, to be specific, SCell activation MAC-CE can be used to trigger UE to report valid L3 measurement results during FR2 unknown SCell activation. No doubt that some benefits, for example, expedite SCell activation process based on timely measurement and report when OD-SSB is activated, can be obtained by introducing OD-SSB MAC CE triggered MR. However, new enhancements with RAN2 on the existing signalling (i.e., reportOnScellActivation-r18) or additional MAC-CE triggered solution analysis including: signalling design and definition, applicable scenarios (not only scenario #2A), ReportConfigNR definition linked to a measID, etc, maybe is necessary, and will be discussed thoroughly. We have concerns on the huge workload will be brought to RAN2 if RAN4 truly to introduce OD-SSB MAC CE triggered MR in the future. At the same time, from our understanding, the existing event triggered reporting, e.g., A1/A2, can also work in L3 measurement based on OD-SSB

Proposal 14: Not to introduce OD-SSB MAC CE triggered MR in Rel-19 NES OD-SSB based L3 measurement

- RAN4 can to discuss the feasibility/applicability of the existing reporting mechanism, e.g., event triggered reporting A1/A2 in Rel-19 NES OD-SSB based measurement and report

2.7 RAN2 related issue: Requirement for RAN1 Alt Time-C1

(OD-SSB based L3 meas.(Case 2), the center frequency locations of always-on SSB and on-demand SSB are same)

In last meeting, RAN2 have the following agreement

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

Issue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

From our understanding, such case is very similar to what we discussed in intra-frequency connected mode measurement, in which UE can be configured with multiple SMTCs (e.g., dual SMTC) with different periodicities. Based on the RAN2 agreement and RAN1 discussions for Time-C1, we have: 1) the periodicity of the OD-SSB specific SMTC 2 is smaller than the SMTC 1 corresponding to AO-SSB; and 2) particularly, when the periodicity of OD-SSB is set as 5 ms, there would always be an overlapping case between SMTC 2 and SMTC 1, given that the OD-SSB would be transmitted in every half frame; 3) When AO-SSB and OD-SSB are configured with the same time location (i.e., same HF index and SFN offset), it would be highly possible that SMTC 1 and SMTC 2 are overlapped in time domain; 4) When time location of OD-SSB is different to that of AO-SSB, non-overlapping case may happen in time domain. Besides, considering the aim of denser periodicity of SSB is to perform fast measurement, assist the NW in requiring quick L1/L3 measurement report based on OD-SSB, and/or to accelerate SCell activation process for the bursty traffic, we think to define L3 requirement for RAN1 Alt Time-C1, the window periodicity is the shortest one among required measurement window (i.e., SMTC 1 & SMTC 2) periodicities for the serving SCell frequency carrier.

Proposal 15: To define L3 measurement requirement for RAN1 Alt Time-C1, if OD-SSB is activated and the serving SCell is activated, the assumed periodicity of SMTC occasions corresponds to the value of OD-SSB specific SMTC, otherwise, AO-SSB SMTC

3 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES OD-SSB. The following observations and proposals are obtained:

Observation 1: The legacy UE assumption that one MO corresponds to one SSB and SSBs in different MOs are regarded as different cells does not hold for SSB based measurement in scenario 3 any more

Observation 2: For SSB-based measurement in scenario 3, it is necessary to discuss the center frequency of which SSB (i.e., AO-SSB or OD-SSB) can be taken as the reference to define intra-frequency/inter-frequency measurements

Proposal 1: For OD-SSB based deactivated SCell L3 measurement (Case 1), when OD-SSB in FMW collided with MG,

- UE is allowed to skip the OD-SSB measurement in the overlapping occasion if FMW collides with MG occasion, a scaling factor is applied to scale the measurement period

  - Clarification on the existing definition of Kp applies to OD-SSB based deactivated SCell measurement can be needed

Proposal 2: When OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2

Proposal 3: RAN4 to define the requirement for RAN1 Alt Time-C2 based on the OD-SSB periodicity

Proposal 4: For SSB-based measurement in scenario 3, one of possible solutions to define reference SSB and the corresponding intra-frequency measurement can be

- When OD-SSB is activated, the reference SSB of the serving cell is the SSB defined in servingCellMO-OD under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell OD-SSB frequency layer

- When OD-SSB is deactivated for the SCell, the reference SSB of the serving cell is the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell AO-SSB frequency layer

- When OD-SSB transmission is not activated for the SCell, the reference SSB of the serving cell is the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency measurement on serving cell AO-SSB frequency layer

Proposal 5: For SSB-based measurement in scenario 3, if to define reference SSB and the corresponding intra-frequency measurment in line with the OD-SSB activation/deactivation command indication

- RAN4 to discuss how to robustly define the corresponding mobility management based on the switched intra-/inter-f measurement requirement subjecting to the changeable reference SSB of the serving cell

Proposal 6: For SSB-based measurement in scenario 3, another possible solution to define reference SSB and the corresponding intra-frequency measurement can be

- Regardless of whether OD-SSB is activated or not, the reference SSB of the serving cell is always the SSB defined in servingCellMO-AO under ServingCellConfig

  - UE to perform intra-frequency neighbour cells measurement on serving cell AO-SSB frequency layer

Proposal 7: For SSB-based measurement in scenario 3, when the two SS blocks (AO-SSB and OD-SSB) are completely located in the active BWP of the UE, if to perform OD-SSB inter-frequency/intra-frequency measurement, no gaps needed.

Proposal 8: For SSB-based measurement in scenario 3, based on the agreed Note: UE is not required to measure both AO-SSB and OD-SSB, the baseline assumption of UE can only perform measurements on one frequency location at a time can be considered

Proposal 9: For SSB-based measurement in scenario 3, the AO-SSB and OD-SSB within different frequencies need to be placed in the first active DL BWP

Proposal 10: To enable the UE to prioritize OD-SSB based SCell measurements (e.g., FMW-based fast deactivated SCell measurement) when OD-SSB transmission is activated.

- UE to measure the servingCellMO-OD when the OD-SSB activation command is indicated

Proposal 11: Consider the mentioned issues are not easy to solve in one meeting, fine to deprioritize scenarios 3 requirement discussions in Rel-19 NES

Proposal 12: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The applicability of the capability of additional processing time 5/2 ms can be applied to the case: When the activation or parameter updates of the OD-SSB transmission is indicated by the MAC CE

Proposal 13: It is fine to clarify: No UE additional processing time for OD-SSB deactivation

Proposal 14: Not to introduce OD-SSB MAC CE triggered MR in Rel-19 NES OD-SSB based L3 measurement

- RAN4 can to discuss the feasibility/applicability of the existing reporting mechanism, e.g., event triggered reporting A1/A2 in Rel-19 NES OD-SSB based measurement and report

Proposal 15: To define L3 measurement requirement for RAN1 Alt Time-C1, if OD-SSB is activated and the serving SCell is activated, the assumed periodicity of SMTC occasions corresponds to the value of OD-SSB specific SMTC, otherwise, AO-SSB SMTC

4 Appendix

4.1 RAN1 agreement update (SSB related)

Agreement For a cell supporting on-demand SSB SCell operation, - The following cases are NOT supported in Rel-19. - AO-SSB is CD-SSB and not on a synchronization raster. - OD-SSB is CD-SSB and on sync-raster. - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB. - When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB. Agreement For a cell supporting on-demand SSB SCell operation, the following is supported for QCL between different OD-SSB configurations - SS/PBCH blocks with the same SSB indexes according to OD-SSB configuration and indication are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index according to OD-SSB configuration and indication with the same QCL parameters according to existing specifications. - FFS: Case where there is different SSB indexes before and after OD-SSB transmission adaptation Conclusion For a cell supporting on-demand SSB SCell operation, - LTM based on OD-SSB is NOT supported in Rel-19. - L1 measurement based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19. - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321). - L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19. Agreement For a cell supporting on-demand SSB SCell operation, - If a PDCCH candidate and a currently being transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate. Agreement For a UE supporting on-demand SSB SCell operation, - When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS. - At least the UE shall assume that the PRBs containing currently being transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is currently being transmitted on the cell. Agreement For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell), - For RO validation before SSB-to-RO mapping, - Alt 3: RO validation rule is determined based on all configured OD-SSB configuration(s) in RRC. - For SSB-to-RO mapping, - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC. Conclusion For a cell supporting on-demand SSB SCell operation, - Whether or not to assume quasi co-location between different SSB indices is up to UE implementation Agreement For a cell supporting on-demand SSB SCell operation, - For OD-SSB, the SS/PBCH block in TS 38.213 Clause 11.1 can be replaced by the SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell. Note: Whether/How to capture the above is up to the editor. Agreement For a cell supporting on-demand SSB SCell operation, it is supported to use OD-SSB as - PL-RS, - Reference signal for QCL/TCI configuration, or - Candidate beam detection for beam failure recovery. Note: Whether/How to capture the above is up to the editor. Agreement For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication. Agreement For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values}, - N2= 8 - Note: This is updated from the previous RAN1 agreement. - The following values for od-ssb-nrofBurst are taken as the starting point and to be confirmed in RAN1#122 - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}. Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - For RO validation before SSB-to-RO mapping, - Alt 3: RO validation rule is determined based on all configured AO-SSB and OD-SSB configuration(s) in RRC. - For SSB-to-RO mapping, - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst and SSB indices configured for AO-SSB in RRC. - NW to ensure that the SSB to RO mapping is the same between different UEs under the same serving cell. Agreement - For a cell supporting on-demand SSB SCell operation, - For Case #1 (i.e., No always-on SSB on the cell), - A CSI report configuration is associated with OD-SSB only, - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - For the case where AO-SSB and OD-SSB have the same center frequency, - A CSI report configuration is associated with both of AO-SSB and OD-SSB, - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met. - For the case where AO-SSB and OD-SSB have the different center frequencies, - Option 2: A CSI report configuration is associated with both of AO-SSB and OD-SSB. - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met. - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. - Note: The CSI report in the above agreement applies only to what is supported up to Rel-18. Agreement For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB, - For a CSI report configuration configured with reportConfigType set to periodic, - Do not support periodic CSI reporting based on OD-SSB, for Case #1 and Case #2

4.2 RAN2 agreement update

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

5 References

[1] R4-2508277 “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson