3GPP TSG-RAN WG4 Meeting #116                                                                            R4-2511086

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:                           Discussion on RRM performance requirement for LB CA

Agenda item:			7.3.5

Document for:	Discussion

1 Introduction

In this meeting, we would like give some high-level preliminary discussions on RRM performance requirement for LB CA

2 Discussion

2.1 Test scope

Due to the introduction of LB CA switching pattern, the measurement requirements of L1/L3 become disparate from what we used in the legacy. Besides, to extend the delay based on missed measurement occasion of necessary DL signals/channels on FDD PCell or SDL SCell, there is a wish to define a new scaling factor to replace the existing one. In the scope, impact on PCell and SCell including complete SCell life cycle are both analyzed, and the measurements involved capture L1 measurement and L3 measurement. Besides, it was agreed that the measurements on SDL carrier expect to be performed on the overlapping occasions between SSB/CSI-RS and SDL duration in switch pattern. In this sense, new TCs for SSB and/or CSI-RS-based L1 and L3 measurements for reporting are needed, the impacted factor is the measurement period, e.g., measurement period TL1-RSRP_Measurement_Period_SSB.

Proposal 1: RAN4 to introduce new test cases for SSB and/or CSI-RS based measurement requirements for LB CA switching, the measurement includes L3 and L1

- New TCs are to verify the measurement procedure when switching pattern does work

Based on the on-going interruption requirement discussion, X% will be introduced with very high probability, and the interruption rate X% is different for different SDL SCC measurement cycle. While, such definition is very different from the existing one, i.e. single rate 0.5% for measurement cycle >=640 ms, that is to says, there is a risk that existing “how to test” including PCell/SCell test condition and test criteria is no longer applicable. In this sense,

Proposal 2: RAN4 to introduce new test cases on the interruption during the deactivated SCell measurement for LB CA switching.

- How to define the test condition and test criteria depends on the specified interruption requirement

Proposal 3: Whether to/How to define other test cases, e.g., TCI state switch, PL-RS switch and RLM depends on the corresponding core requirement discussion

3 Conclusion

In this contribution, we provided our viewpoints on test case design for LB CA switching. The following observations and proposals are obtained:

Proposal 1: RAN4 to introduce new test cases for SSB and/or CSI-RS based measurement requirements for LB CA switching, the measurement includes L3 and L1

- New TCs are to verify the measurement procedure when switching pattern does work

Proposal 2: RAN4 to introduce new test cases on the interruption during the deactivated SCell measurement for LB CA switching.

- How to define the test condition and test criteria depends on the specified interruption requirement

Proposal 3: Whether to/How to define other test cases, e.g., TCI state switch, PL-RS switch and RLM depends on the corresponding core requirement discussion