3GPP TSG-RAN WG4 Meeting #116                                                                            R4-2511088

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:                           Discussion on L3 measurement enhancement by optimizing Rx BSF

Agenda item:			7.14.3.1

Document for:	Discussion

1 Introduction

n last meeting, RAN4 had extensively discussions on L3 measurement enhancement by optimizing Rx BSF. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer expires after UE quits from Multi-Rx mode.

From our understanding, 1) T1 can be flexibly configured/indicated to comply with different requirements under various scenarios. In this sense, the set of T1 is expected to contain multiple candidate values. The following interpretations on the candidate numbers for T1 can be fully discussed:

- The number N of transmitted bursts/samples

- A recap from previous discussions on samples for inter-f and intra-f, below (taking FR2-1 Inter-frequency measurement with gap as example)

Table 1. Samples needed for inter-f and intra-f measurement FR2-1

| Inter-frequency measurement with gap |  |  |  |
| --- | --- | --- | --- |
|  | PSS/SSS detection | Time index detection | Measurement |
| Samples | Mpss/sss_sync_inter=40 | MSSB_index_inter=24 | Mmeas_period_inter=40 |

- We agreed that: Introduce the UE capability of reduced N, the values of which are {2,4,6}

From this, considering all the samples we defined in inter-/intra-f case, the set containing the candidate number N of transmitted burst/samples for FR2-1 L3 FBS case can be defined as：T1={6, 10, 15, 20, 25, 30, 35, 40, 45} (number of transmitted samples) the timer expires when the sample measurement is completed

- A real timer, the values corresponding to the existing timer (e.g., sCellDeactivationTimer, bwp-InactivityTimer, drx-InactivityTimer, etc. ) can be applied to T1. The T1 can be terminated when the timer expires.

As an example, T1 for FR2-1 L3 FBS case can be essentially equivalent to drx-InactivityTimer

drx-InactivityTimer  { ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

. ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1},

drx-HARQ-RTT-TimerDL                INTEGER (0..56),}

- T1 for FR2-1 L3 FBS case can be ssentially equivalent to TTT

TimeToTrigger ::=                   ENUMERATED {

ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256,

ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560,

ms5120}

- Total measurement/detection period. The timer expires when time’s up, i.e., the measurement/detection is completed and the valid result is reported. The benefit is that it can be expected the measurement result for all frequency carriers is available after the period. In this sense, as one of the starting points: T1={0.2, 0.4, 0.6, 4, 9, 15, 18, 24, 26, 30, 44, 47, 70, 115, 154, 180} (s) could to cover all possibilities of the configuration parameters

Proposal 1: The T1 is expected to contain multiple candidate values to comply with different requirements under various scenarios. The candidate numbers for T1 can be:

- Alt. 1 T1={6, 10, 15, 20, 25, 30, 35, 40, 45} (number of transmitted samples), the timer expires when the sample measurement is completed

- Alt. 2 T1= {the values corresponding to the existing timer}

- Alt. 3 T1= {the values corresponding to the existing TTT}

- Alt. 4 T1={0.2, 0.4, 0.6, 4, 9, 15, 18, 24, 26, 30, 44, 47, 70, 115, 154, 180}  (s) can be taken as a starting point, which is based on total measurement/detection period, the timer expires when time’s up

It is plausible to consider the gap between the time point A: the timer expires (i.e., UE quits from Multi-Rx mode) and time point B: UE to re-evaluate the activation condition. While, we think it may be a little too detailed if to define another conditions, e.g., threshold/time duration related, we prefer that after the timer expires/UE quits from Multi-Rx mode, it is up to UE implementation when to re-evaluate the activation condition

Proposal 2: Agree the mechanism, and after UE quits from Multi-Rx mode, it is up to UE implementation when to re-evaluate the activation condition

3 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for L3 measurement enhancement by optimizing Rx BSF. The following observations and proposals are obtained:

Proposal 1: The T1 is expected to contain multiple candidate values to comply with different requirements under various scenarios. The candidate numbers for T1 can be:

- Alt. 1 T1={6, 10, 15, 20, 25, 30, 35, 40, 45} (number of transmitted samples), the timer expires when the sample measurement is completed

- Alt. 2 T1= {the values corresponding to the existing timer}

- Alt. 3 T1= {the values corresponding to the existing TTT}

- Alt. 4 T1={0.2, 0.4, 0.6, 4, 9, 15, 18, 24, 26, 30, 44, 47, 70, 115, 154, 180}  (s) can be taken as a starting point, which is based on total measurement/detection period, the timer expires when time’s up

Proposal 2: Agree the mechanism, and after UE quits from Multi-Rx mode, it is up to UE implementation when to re-evaluate the activation condition

4 References

[1] R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2