3GPP TSG-RAN WG4 Meeting #116                                        R4-2511092

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:              General discussion on AI mobility regarding testability and interoperability

Agenda item:			7.19.4

Document for:	Discussion

1. Introduction

Some AI mobility regarding testability and interoperability were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Prediction consistency in time domain

<Way forward >: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

From our understanding, a RSRP value can be modeled as a : P|x, given a set of physical features x. The prediction P is the maximum likelihood estimation value minimizing the prediction accuracy metrics (e.g., MES). The set of features can contain: Location, cell ID, time features, velocity, device model (e.g., RF, BB), DL carrier frequency and lBS-lj2, depending on the degree of influence on the prediction result/AI model output, as a remarkable instance, RSRP manifests quite different characteristics on different locations when the UE moves toward the destination along a path with velocity v, where the trajectory for the UE can be defined as a sequence of time-stamped points, i.e., location. In practice, time-varying fading that may occur at individual positions, if to model different time-varying characteristics per cell/site in FR1, select/fix one UE trajectory model as reference can predict the time-varying value for the RSRP measurements at the same location at least. And at each location, we think the RSRPs exhibit strong spatial correlation to some extent

Proposal 1: On how to model different time-varying characteristics per cell/site in FR1:

- Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

Regarding option 2, from our understanding, for RRM testing, the fading and multipath can be modelled electronically by using channel emulator before signal transmission on N antennas, that means the Tx power transimitted from TE would be changed with the channel condition variation

Proposal 2: If Tx power is used to model different time-varying characteristics per cell/site in FR1, Tx power is to change with channel condition variation

2.2Testing Setup in FR2

<Way forward >: - For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting: - Number of SSBs per cell - Channel model - FFS the same or different for temporal/frequency domain prediction - Impact of UE trajectory - FFS the same or different for temporal/frequency domain prediction - Dynamic range of received power (consistency of channel) - Measurement prediction specific parameters - Emulation of noise and EPRE - L3 filter coefficient

From our understanding, the RSRP of one cell was affected by complicated environment of the cell, the maximum and minimum RSRP of different cells fluctuated significantly. At the same time, considering the relationship between signal detectability and the SNR, if SNR is too low, no signal can be detected, while the signal can be detected at a location where the SNR is sufficient to slightly distinguish a signal from noise but not strong enough to enable the signal be measured and acquired. Put the analysis into the testing setup, that means regarding the emulation of noise and EPRE: How the emulated noise is generated/transmitted, and for the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point to ensure the signal can be detected.

Proposal 3:

- Dynamic range of received power under certain SNR

  - From signal can be detected to signal can be acquired

- Emulation of noise and EPRE

  - How the emulated noise is generated/transmitted, and for the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point

3 Conclusion

In this contribution, we provided our viewpoints on AI RRM requirements for testability and interoperability. The following observations and proposals are obtained:

Proposal 1: On how to model different time-varying characteristics per cell/site in FR1:

- Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

Proposal 2: If Tx power is used to model different time-varying characteristics per cell/site in FR1, Tx power is to change with channel condition variation

Proposal 3:

- Dynamic range of received power under certain SNR

  - From signal can be detected to signal can be acquired

- Emulation of noise and EPRE

  - How the emulated noise is generated/transmitted, and for the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point

4 References

[1] R4-2508269 WF on FS_NR_AIML_Mob_Part2, OPPO