3GPP TSG-RAN WG4 Meeting #116                                          R4-2511105 Bangaluru, India, August 25 - 29, 2025

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	7.21.2.1

Document for:	Approval

Introduction

During RAN4#115, general aspects for SBFD have been discussed. Following discussion points are reproduced from [1].

##### Declaration and flexibility for SBFD operation Agreement: - For all FR2 BS classes, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting. - DL/UL subband configurations support for SBFD Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. ##### Restriction on the number of DL subband sizes supported Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not. ##### Restriction on guardband size for SBFD - No agreement in this meeting and the way forward from RAN4#114bis can still be followed. ##### Restriction on DL subband size Agreement: - If the SSB symbol is overlapped with SBFD symbol, - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration - FFS the test configuration during performance part discussion. ##### The necessity to update for HPUE duty cycle evaluation - No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

In this contribution, we would like to share our views regarding this topic.

Discussion

The power limitation for FR1 MR SBFD-capable BS

In last meeting, whether and how to restrict the declaration and flexibility for SBFD operation for FR1 MR BS has been discussed. The proposed justification on the lower power level threshold i.e. 33dBm is that it is closely related to whether specific implementation e.g. the usage of analog filter can be expected. Regarding the proposed value of such threshold, we take it with a grain of salt. Reviewing the feasibility study captured in TR 38.858 clause 9.3, we identify that the feasibility of FR1 MR for SBFD was mainly concluded with 38dBm Tx power assumption. Therefore we have the following proposal.

Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS.

Spec update for HPUE duty cycle

Proponent raised one question that whether the current specification “uplink symbols transmitted in a certain evaluation period” is clear enough for SBFD-aware UE. Note that for legacy UE the “uplink symbols” actually means the symbols with uplink transmission, which include e.g. flexible symbols configured with periodic uplink transmission. Thus it may be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” in order to better accommodate SBFD-aware UE.

Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better implementing max duty cycle evaluation for an SBFD-aware UE max duty cycle evaluation.

Terminology alignment

In WF [2], one guidance for CR drafting is captured as below. RAN4 is tasked to align the terminology for describing the granularity of SBFD in time domain.

- Issue 2-7: One guidance for CR drafting - Way forward - Check and align the terminology for “SBFD slot”, “SBFD symbol”, “SBFD symbol/slot” or “SBFD period” to be used in specification drafting. - The decided terminology should be clarified in clause 3. - If there are existing requirements can be referred to, then the corresponding requirement can be cited, rather than copying/pasting.

We are sceptical about the necessity of using “SBFD period”. Note that both slot and symbol are used in the specification since the beginning, and both RAN1 scheduling and RAN4 requirements definition/verification are based on those granularities. When it comes to SBFD, RAN1 would use SBFD symbol specifically, it is better for RAN4 to comply with the same terminology but definitely not “SBFD period” also considering the fact that it won’t introduce fundamental change to RF requirements. Hence we think there is no need for RAN4 to use “SBFD period” for specification drafting.

Proposal 2: Not to use terminology “SBFD period” for specification drafting.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observations and proposals:

Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better implementing max duty cycle evaluation for an SBFD-aware UE max duty cycle evaluation.

Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS.

Proposal 2: Not to use terminology “SBFD period” for specification drafting.

- References

- R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.

- R4-2508713, “Way Forward for [115][307] NR_duplex_evo_General”, Huawei, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.