3GPP TSG-RAN WG4 Meeting #116                                          R4-2511106 Bangaluru, India, August 25 - 29, 2025

Source: 	Huawei, HiSilicon

Title: 	On intra-operator adjacent channel inter-cell UE-UE CLI

Agenda Item:	7.21.2.2

Document for:	Approval

Introduction

During RAN4#115, the issue on intra-operator adjacent channel inter-cell UE-UE CLI for SBFD have been discussed. Following contents are reproduced from WF [1].

Way Forward - UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception - WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model - WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any. - WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required:

In this contribution, we would like to share our views regarding this topic.

Discussion

Concerned scenario and potential solution(s)

As captured in the TR 38.858, some DL throughputs degradation can be identified for the urban hotspot deployment where legacy TDD DL reception plays as victim and SBFD co-existed in adjacent channel. Such degradation is observed only at cell edge mainly due to UE-UE CLI.

- 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1. Table 11.3.1-1: Case 1 co-existence conclusions Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations.. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (46 dBm to 53 dBm).. Urban Macro -> Urban Micro Scenario 4; FR1; No DL throughput degradation for both average throughput and cell edge throughput is observed.. Indoor -> Indoor Scenario 3 and Scenario 9; FR1 and FR2-1. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8; FR1 and FR2-1.

For Solution-1, it is proposed to introduce hard-limit for aggressor output power by allowing network configuration of a dedicated p-Max so that the determined PCMAX can be capped. However, we think the following points should be carefully considered.

- One fundamental issue that would shake the feasibility of this solution is that neither Rel-16 nor Rel-19 RRM CLI measurement mechanism can support adjacent frequency measurement. Then the basis of aggressor UE screening is actually absent. Subsequent RAN2/RAN3 signalling design even for intra-operator usage aiming to exchange information on that aspect is not possible at least in this release.

- If the network can only do a rough selection on aggressor, then the cost of network coverage performance must be expensive over a very limited gain on UE-UE CLI reduction.

- Considering the complex situation in the field contributed by the number of serving UEs with different movement, any static or even semi-static mechanism kept with the proposal might not have sufficient performance, while a dynamic solution must come with more complexity.

Observation 1: For Solution-1, the basis of aggressor UE identification is absent, because neither Rel-16 nor Rel-19 RRM CLI measurement mechanism can support adjacent frequency measurement.

Observation 2: If the network can only do a rough selection on aggressor, then the cost of network coverage performance must be expensive over a very limited gain on UE-UE CLI reduction.

When we trace back to the co-ex study, we understand that the conclusion must have tight relationship with the simulation modelling e.g. ACIR model which plays as UE-UE CLI resistance. Based on flat model, 30dBc ACLR for PC3 and 33dBc ACS would generate 28.2dBc ACIR only. However under stepwise model, ACLR can increase to a higher value e.g. 43dBc considering the DL subband between the aggressor SBFD UL subband and the victim legacy TDD DL CBW. Consequently, the equivalent ACIR can approach to 33dBc then there is around 5dB improvement. From real implementation perspective, the ACIR performance must be better than those models. Thus it somehow shows the possibility on performance improvement by frequency selective scheduling.

Observation 3: For co-ex study, the ACIR generated by flat model reflects the worst case, while stepwise model can be relatively closer to real implementation and somehow shows the possibility on UE-UE CLI handling by frequency selective scheduling.

Coincidentally, the MSD issue caused by DC/CA operation can be compared with this one, since it is also about in some specific frequency combinations, DL degradation would become more as the UL transmission power ramping up. Instead, the specification didn’t introduce any Tx power limitation but only a REFSENS exemption as a reference for deployment/scheduling.

Observation 4: MSD issue is somehow similar with UE-UE CLI issue, but spec didn’t introduce any Tx power limitation but only a REFSENS exemption as a reference for deployment/scheduling.

In conclusion, we think the whole UE-UE CLI issue can be resolved by network scheduling without the proposed network configuration or air interface enhancement in Rel-19.

Proposal 1: In Rel-19, do not specify Solution-1 in Rel-19. How to accommodate UE-UE CLI can be up to gNB implementation.

Conclusion

In this contribution we discussed on the UE-UE CLI issue for SBFD. We have the following observations and proposals:

Observation 1: For Solution-1, the basis of aggressor UE identification is absent, because neither Rel-16 nor Rel-19 RRM CLI measurement mechanism can support adjacent frequency measurement.

Observation 2: If the network can only do a rough selection on aggressor, then the cost of network coverage performance must be expensive over a very limited gain on UE-UE CLI reduction.

Observation 3: For co-ex study, the ACIR generated by flat model reflects the worst case, while stepwise model can be relatively closer to real implementation and somehow shows the possibility on UE-UE CLI handling by frequency selective scheduling.

Observation 4: MSD issue is somehow similar with UE-UE CLI issue, but spec didn’t introduce any Tx power limitation but only a REFSENS exemption as a reference for deployment/scheduling.

Proposal 1: In Rel-19, do not specify Solution-1 in Rel-19. How to accommodate UE-UE CLI can be up to gNB implementation.

- References

- R4-2508708, “Way Forward on UE-UE CLI problem statement and potential solutions”, Samsung, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.