3GPP TSG-RAN WG4 Meeting #116                                          R4-2511108 Bangaluru, India, August 25 - 29, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirements for NR NTN HPUE

Agenda Item:	7.27.3

Document for:	Approval

Introduction

During RAN4#115, a quite good discussion on NTN HPUE had been conducted. As captured in WF [1], the PC1 MPR for 4Tx is FFS.

Issue 1-8 ~ Issue 1-9 PC1 MPR related Agreement: - Interested companies are encouraged to work on two tables with tentative values for PC1 MPR, one for 1Tx and one for 4Tx - Further check these tentative values, especially for the edge RB results by considering the working assumption of L_CRB as [8]RB. - With the understanding the [4Tx] architecture for PC1 is not agreed yet. - If 4Tx would be agreed, it can be further discussed if we can merge two tables then. Table 1. PC1 MPR [with 1Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; [Note 1]; ≤ [0.5]; ≤ [0]. QPSK; [Note 1]; ≤ [1]; ≤ [0]. 16 QAM; [Note 1]; ≤ [2]; ≤ [1]. 64 QAM; [Note 1]; ≤ [2.5]; ≤ [2.5]. CP-OFDM; QPSK; [Note 1]; ≤ [3]; ≤ [1.5]. 16 QAM; [Note 1]; ≤ [3]; ≤ [2]. 64 QAM; [Note 1]; ≤ [3.5]; ≤ [3.5]. NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB.. [Table 2. PC1 MPR with 4Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ [9.5]; ≤ [2.0]; ≤ [0.5]. QPSK; ≤ [10.0]; ≤ [2.5]; ≤ [0.5]. 16 QAM; ≤ [10.0]; ≤ [3.5]; ≤ [1.5]. 64 QAM; ≤ [10.0]; ≤ [4.0]; ≤ [3.5]. CP-OFDM; QPSK; ≤ [10.0]; ≤ [4.5]; ≤ [2.0]. 16 QAM; ≤ [10.0]; ≤ [4.5]; ≤ [2.5]. 64 QAM; ≤ [10.0]; ≤ [5.0]; ≤ [5.0].

In this contribution, we would like to share our views on this topic.

Discussion

- 2.1 PC1 MPR requirement with 4T

As agreed in last meeting, 34dBc was adopted as the ACLR requirement for PC1. Although one can argue that it is relaxed compared to the existing 37dBc PC1 ACLR in TN specification, we still need to emphasize that additional 3dB emission suppression will have to be achieved comparing to a PC1.5 4Tx UE due to its 31dBc ACLR requirement. Such efforts and subsequent power consumption issue shall not be overlooked during the MPR requirement discussion. Moreover, spare enough margin would definitely thrive the evolution of NTN UE ecosystem. Based on our measurements, we have the following proposal on MPR requirement for PC1 with 4Tx.

Proposal: Adopt the following values as MPR requirements for PC1 with 4Tx.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 10.0 | ≤ 3 | ≤ 1 |
|  | 16 QAM | ≤ 10.0 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 10.0 | ≤ 4.5 | ≤ 4 |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 5 | ≤ 2.5 |
|  | 16 QAM | ≤ 10.0 | ≤ 5 | ≤ 3 |
|  | 64 QAM | ≤ 10.0 | ≤ 5.5 | ≤ 5.5 |

Conclusion

In this contribution we discussed on the Rel-19 NTN HPUE. We have the following proposal:

Proposal: Adopt the following values as MPR requirements for PC1 with 4Tx.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 10.0 | ≤ 3 | ≤ 1 |
|  | 16 QAM | ≤ 10.0 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 10.0 | ≤ 4.5 | ≤ 4 |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 5 | ≤ 2.5 |
|  | 16 QAM | ≤ 10.0 | ≤ 5 | ≤ 3 |
|  | 64 QAM | ≤ 10.0 | ≤ 5.5 | ≤ 5.5 |

- References

- R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.