3GPP TSG-RAN WG4 Meeting#116                                R4-2511126

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussions on RF requirements for A-IoT BS

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

Discussion

- 1/32.1 RF requirement impacts for A-IoT BS

- Table 2.1-1. The initial analysis for RF requirement of A-IoT BS in D1T1.

| Tx part |  |
| --- | --- |
| Base station output power | Agreement in TR38.769: Base station output power requirement is potentially needed, RAN4 agrees to use current requirements of Micro NR BS type 1-C as the starting point. RAN4#114 meeting: Agreement: - Agreement: - A-IoT BS output power, Prated,c,AC≤ [38]dBm Status: Complete |
| Output power dynamics | Agreement in TR38.769: Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement. Power boosting requirement is not needed in R19 with the assumption that ambient IoT BS only transmits and receives AIOT signal. Status: RE power control and total dynamic range power is still open. RAN4#114 meeting: - Agreement: - Check whether RAN1 supports RE granularity operation for R2D transmission or not. RAN4#114: - Agreement: - Check whether RAN1 supports PRB granularity operation for R2D transmission or not RAN4#114bis: - Issue 1-2-1: RE power control dynamic range - Agreement: - No need to define. - Issue 1-2-2: Total power dynamic range - Agreement: - No need to define. Status: complete |
| Transmit ON/OFF power | Agreement in TR38.769: Regarding the transmit on/off requirement, it usually includes of transmitter off power requirement and transmitter transient period requirement, RAN4 agrees to further discuss in the WI phase. Status: transmit ON/OFF power and transition period requirement is still open. RAN4#114: - Agreement: - Define Transmitter OFF power. - values FFS - Agreement: - Define Transmitter transient period. - FFS values. RAN4#114bis: - Agreement: - Transmitter OFF power spectral density shall be less than [-85] dBm/MHz per antenna connector. RAN4#115: - Agreement: - Transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector. Status: complete |
| Transmitted signal quality | Agreement in TR38.769: Frequency error and modulation quality (EVM) are potentially needed. TAE is only applicable for BS transmitting from multiple antennas via MIMO, CA, or combination of them. A-IoT BS doesn’t support these features, so TAE requirement is not needed. Status: freq error and EVM requirement are still open. RAN4#114 meeting: - Agreement: - Reuse the NR micro BS requirements（±0.1 ppm）for A-IoT BS. Status: freq error is complete. RAN4#114 meeting: - Agreement: - FFS whether to define requirements such as modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS. In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT BS EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. Some updated simulation results are provided as following OOK-4 with M=2 and Rapp model with target power as 33dBm used. Figure 2.4-1. time domain waveform for OOK with M=2 chipset Based on the above simulation results, we propose to define the EVM requirement for A-IoT BS as below. Proposal 1: propose to define the EVM requirement for A-IoT BS as above. Chip duration Tc; Parameter; Symbol; Mimimum; Nominal; Maximum; Units. - 1/(M*15KHz)*10^6 - Where M could be 2, 6,12,24; Modulation Depth; (A–B)/A; 80; 90; 100; %. RF Envelope Ripple; Mh = Ml; 0; 0.15(A–B); V/m orA/m. RF Envelope Rise time; Tr,10-90; 0.66Tc; µs. RF Envelope fall time; Tf,10-90; 0.66Tc; µs. RF Pulsewidth; PW; 1 Tc; 1.3 Tc; µs. |
| OBW | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. Since OBW requirement is the regulatory requirement, so the legacy OBW requirement could be reused for A-IoT BS; Some initial simulation results are provided for R2D with 1-4 PRB transmission bandwidth configuration together with some guard band reserved for the digital spectrum shaping.W-OFDM waveform with 7% CP is also considered in this evaluation. For single PRB R2D transmission with OBW as 200KHz, it seems not been able to meet the 99% regulatory requirement, however some further spectrum sharping might help to improve this requirement e.g. increasing the ratio of CP for W-OFDM spectrum shaping. For R2D transmission; OBW measurement results. 1 PRB; OBW for 200KHz CBW: 97.42%.. 2 PRB; OBW for 400KHz CBW: 99.71%.. 3 PRB; OBW for 600KHz CBW: 99.85%. 4 PRB; OBW for 800KHz CBW: 99.99%. RAN4#115 - Agreement: - The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth. Status: complete. |
| ACLR | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. RAN4#114 meeting: - Agreement: - Further discuss ACLR requirements considering following aspects: - No legacy NR UE indoor assumption - Whether 100KHz offset is needed or not RAN4#114bis meeting: - Agreement: - Consider following options for ACLR requirements - Option 1: ACLR1 is 40dB and ACLR2 is 50dB for A-IOT BS 1PRB transmission BW with 100kHz offset - Option 2: ACLR 1 is 30dB and ACLR2 is 40dB for A-IoT BS 1PRB transmission BW with Foffset=100kHz. - FFS on whether frequency offset is needed or not for other PRB numbers During the Rel-19 SI phase, A-IoT BS ACLR requirement is assumed as 40dBc for coexistence study, there are not significant performance degradation observed. For Scenario 2-1 case c, some further evaluation results are provided with A-IoT ACLR requirement from 20dB to 30dB assumed. Based on the following simulation results, it seems that ACLR as 20dBc could also ensure the good coexistence performance due to UE location limited to outdoor deployment. Observation 1: if ACLR requirement of A-IoT BS is relaxed as 20dBc, there are no significant performance degradation for NR outdoor UE at 900MHz. In addition, some initial simulation results are provided for R2D with 1 PRB transmission bandwidth configuration together with some guard band (e.g. 10kHz for single PRB, 20kHz for two PRB, 30kHz for 3 PRBs and 40kHz for 4 PRBs) reserved for the digital spectrum shaping.W-OFDM waveform with 7% CP is also considered in this evaluation. The PSD for R2D with single PRB transmission is shown as following. If ACLR requirement is defined as 30dBc which is the same as FR1 PC3 UE ACLR requirement, then for R2D transmission with single PRB bandwidth and two PRB bandwidth configuration without considering 100KHz guard band, then it could be difficult to meet the requirement, however if considering the 100KHz guard band, 30dBc ACLR requirement could be fulfilled for R2D transmission. R2D transmission bandwidth with single PRB. Without 100KHz freq offset; With 100KHz freq offset. ACLR1; 22.7dBc; 29.86dBc. ACLR2; 31.95dBc; 35.08dBc. R2D transmission bandwidth with two PRBs. Without 100KHz freq offset; With 100KHz freq offset. ACLR1; 28.7452; 32.3783. ACLR2; 37.4387; 38.7110. R2D transmission bandwidth with three PRBs. Without 100KHz freq offset; With 100KHz freq offset. ACLR1; 32.55; 35.06. ACLR2; 38.31; 39.62. R2D transmission bandwidth with four PRBs. Without 100KHz freq offset; With 100KHz freq offset. ACLR1; 32.2745; 34.1212. ACLR2; 41.9774; 42.9096. Observation 2: If ACLR requirement is defined as 30dBc, If without considering 100KHz freq offset, - for R2D transmission with single/two PRB bandwidth without considering 100KHz guard band, then it could be difficult to meet the requirement; for R2D transmission with three/four PRB bandwidth without considering 100KHz guard band, then ACLR requirement could be fulfilled; If considering the 100KHz freq offset, - 30dBc ACLR requirement for R2D transmission with 12/3/4 PRB could be fulfilled. - Observation 3: If ACLR requirement is defined as 20dBc, 20dBc ACLR requirement for R2D transmission with 12/3/4 PRB could be fulfilled. Proposal 2: propose RAN4 to define the ACLR requirement as 30dB for 1st adjacent channel and 40dBc for 2nd adjacent channel. R2D carrier bandwidth [KHz]; Freq offset next to the carrier [KHz]; ACLR1 [dB]; ACLR2. 200; 100; 30; 40. 400; 100; 30; 40. 600; N/A; 30; 40. 800; N/A; 30; 40. |
| Operating band unwanted emissions | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. This depends on the conclusion of ACLR and carrier bandwidth requirement etc. In the previous RAN4 meeting, there were some proposals to reuse standalone NB-IoT UEM requirement for A-IoT BS. At least from our understanding, this might be not necessary. Here is the reasons from our side. First of all, the standalone NB-IoT BS UEM requirement is inherited from MSR BS with GSM carrier placed at the band edge with 100KHz frequency offset. For A-IoT BS, we don’t see the necessity for MSR BS and it should be standalone NB-IoT. In addition, proposed ACLR requirement for A-IoT BS is around 10dB relaxed compared with standalone NB-IoT BS. If companies want to reuse the standalone NB-IoT BS emission mask, then we propose to consider 10dB relaxation on top of MSR standalone NB-IoT emission mask. Proposal 3: propose to consider the 10dB relaxation on top of standalone NB-IoT BS emission mask. |
| Transmitter spurious emissions | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. The Tx transmitter spurious emission requirement is the regulatory requirement, therefore we propose still to reuse the current FR1 requirement for it. The legacy transmitter spurious emission requirement could be reused since this is coming from the ITU regulatory definition. RAN4#114 meeting: - Agreement: - Use the FR1 NR BS transmitter spurious emission requirement for A-IoT BS as starting point. - FFS on the co-location requirements if the A-IOT BS is co-located with BS operating in other bands. - During the last RAN4 meeting, there were some discussions on the necessity of co-location requirement for A-IoT BS, from the deployment perspective, such kind of co-location is still possible e.g.. 900MHz A-IoT BS sharing with the same installation site as 2.3GHz or 2.6GHz or 4.9GHz. Status: complete. |
| Tx intermodulation | Agreement in TR38.769: Unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed. Status: OBW, ACLR, OBUE and transmitter spurious emission requirement are still open. Based on the some discussion in other WIDs, it seems that Tx intermodulation requirement is kind of regulatory requirement from the co-existence perspective, therefore we propose still to define this requirement. Agreement: - Agreement: - FFS. - RAN4#115 - Agreement: - No need Status: complete. |
| Rx part |  |
| REFSENS | Agreement in TR38.769: Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus: - For BS type 1-C, PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target] - BW is the transmission bandwidth for D2R. - NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS. - IM is the implementation margin, equal to 2dB. - SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase. - Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology. Status: open [simulation results will be provided later on.] For D2R transmission bandwidth, this is still under the discussion in RAN1. For modulation for D2R transmission, whether this is OOK signal or BPSK modulation, this is not decided yet. Meanwhile whether CRC other coding scheme is to be specified for D2R transmission is also not decided yet. In short, it is premature to discuss the test metric for REFSENS requirement from BW, SNR; Parameters; Assumptions. Carrier frequency; 900 MHz. SCS; 15 kHz as baseline. Block structure; Depend on RAN1. Channel model; AWGN. Number of Tx/Rx chains for Ambient IoT device; 1. Number of BS antenna elements; 1. Message size; 96 bits. BLER target; 10%. Sampling frequency（SFO）; Randomly select a value from the range of [104 ppm~105 ppm] for device 1. Transmission bandwidth（DSB）; 15kHz. [OOK/BPSK chip rate]; 7.5kbps. Receiver bandwidth; 15KHz. Waveform (CW); unmodulated single tone. Modulation; OOK. Line code; Manchester encoding. FEC; 1/3 Convolutional Code. ADC bit width; Ideal. D2R receiver; coherent receiver. Repetition; None. Preamble; 31bit. Midamble; 31bit. Proposal 4: propose to use the above FRC as REFSENS definition and SNR as -4.2dB for BPSK and 0.5dB for OOK reception. In addition, regarding the desens due to the CW transmission, this also need more discussion since CW signal is outside of topology in D1T1-B, after the A-IoT receive the CW signal, then CFO estimation and following other CW cancellation technique (e..g high pass filtering or digital IC etc ) to remove the received CW signal. Some initial considerations for CW maximum input power for refesens requirement, as shown in the following figure, CW is assumed as outside of topology with the isolation distance as 20m for the intended coverage, consider the InF pathloss as captured in TR38.901 and transmitter/receiver side lobe attenuation, the expected maximum input power as -36dBm/CW signal. Based on the above analysis for the potential CW maximum input power and CW cancellation capability for CFO estimation and following CW cancellation technique, the desens requirement could be derived as [-36] dB. Proposal 5: propose to define the input power of CW signal as -36dBm. In the previous RAN4 meeting, CW signal would have the following impacts on the REFSENS requirement: - CW cancellation on CW signal and its corresponding phase noise; [high impacts] - Noise floor rise according to the -36dBm CW signal input; [limited impacts] - Potential reciprocal mixing between A-IoT BS LO and CW phase noise. [High impacts] - The CW and phase noise cancellation in the baseband is also somehow dependent on the 2Rx IRC receiver. However according to the legacy REFSENS requirement definition, it is defined based on single antenna connector which also make the difficulty to conduct the CW/phase noise cancellation. Based on above consideration, considering the phase noise as 50dBc within DSB 15KHz with -36dBm CW input power at the reader connector, then REFSENS degradation is around 36dBc on top of baseline REFSENS requirement. - Proposal 6: propose to define the REFSENS relaxation as 36dBc on top of baseline REFSENS requirement; |
| Dynamic range requirement [? any agreement] | Agreement in TR38.769: For NR and LTE, receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. The interference level is derived from simulation of co-channel interference from the same operator. It measures the effects of base station receiver impairments. For Ambient IoT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference. The necessity and the value can be studied at WI phase. Status: open [simulation results will be provided later on.] In the previous RAN4 meetings, there were no discussions on the necessity of dynamic range requirements. From our understanding, co-channel interference might be limited since the inventory for device is based on FDM by the repetition or TDM for different device under the same reader site. For the simultaneous activation of the reader within the indoor factory scenario, the isolation distance is assumed as 60m in the SI phase. In other words, the uplink coverage for different regions are also isolated. In addition, considering the passive device with the backscattering loss, then it would be fine not to define this BS dynamic range requirements. Proposal 7: propose not to define the dynamic range requirement for A-IoT BS. |
| ACS, | Agreement in TR38.769: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase. Status: open [simulation results will be provided later on.] Based on the simulation results captured in TR38.769, it could be found that the impacts of NR UE in-band emission requirements towards the A-IoT BS receiver is quite limited, indeed for relative IBE is around 20-30dB considering different UE transmission power and also modulation order and worst freq location with aggressor and victim spectrum to be adjacent placed, in other words, if the receiver selectivity requirement could be also set as 20-30dBc, then there would be quite negligible performance impacts. . Proposal 8: propose to define the ACS requirement as 20-30dBc for A-IoT BS with 3MHz DFT-s-OFDM signal configuration. |
| Blocking requirement | Agreement in TR38.769: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase. Status: open For IBB requirement, propose to reuse the existing MR in-band blocking requirement since lots of front-end hardware design are reusing from the legacy MR BS. RAN4#115: - Agreement: - Reuse the in-band blocking requirement from legacy NR Medium Range BS for A-IoT BS - Status: complete. |
| OOBB | Agreement in TR38.769: Regarding Out-of-band blocking requirement, the existing out-of-band blocking requirement may be still applicable for A-IoT BS except for the wanted signal level. Don’t see the reason not to reuse the -15dBm CW signal as interference signal of OOBB requirement. For f_OOBB requirement, this could be further discussed once we have more clear assumption on A-IoT BS. Status: open Status: complete. |
| RX IMD | Agreement in TR38.769: Regarding the general intermodulation requirement, it is agreed to postpone to WI phase due to the potential relationship between D2R operation assumption e.g. single tone or multiple tone D2R assumption. Regarding the narrowband intermodulation requirement, it is potentially necessary due to the potential Narrow band blocking at the operating frequency range, however its detailed requirement could be postponed to WI phase. This is somehow similar as Tx intermodulation requirement. Proposal 9: propose not to define the Rx IMD and NBB IMD requirement at the current phase. |
| Rx spurious emission | Agreement in TR38.769: Regarding the receiver spurious emission requirement, this requirement might differ as following: - For D1T1-A1 and D1T1-B, the receiver spurious emission requirement is still needed due to is half duplex operation mode. - For D1TA-A2, due to the simultaneous transmission of CW signal and D2R reception, if the spurious emission requirement of CW signal is exceeding the receiver spurious emission requirement, then the receiver spurious requirement is not needed similar as FDD BS otherwise this requirement is still needed to regulate the receiver RF performance. - RAN4#114 meeting: - Agreement: - The receiver spurious emissions for A-IoT BS can reuse the NR BS requirements specified in Table 7.6.2-1 of TS 38.104. Status: complete. |
| ICS | Agreement in TR38.769: Regarding the receiver in-channel selectivity requirement, this is agreed to be discussed in WI phase. RAN4#114: - Agreement: - FFS We don’t see the necessity to have this requirement for standalone A-IoT BS or in-band/guard band operation with the individual RF hardware similar as standalone NB-IoT ICS requirement. RAN4#115: - Agreement: - No ICS requirement is needed to be introduced. Status: complete. |

Conclusions

In this contribution, we shared some further views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

Proposal 1: propose to define the EVM requirement for A-IoT BS as above.

| Chip duration Tc | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| - 1/(M*15KHz)*10^6 - Where M could be 2, 6,12,24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | 0.15(A–B) | V/m orA/m |
|  | RF Envelope Rise time | Tr,10-90 |  |  | 0.66Tc | µs |
|  | RF Envelope fall time | Tf,10-90 |  |  | 0.66Tc | µs |
|  | RF Pulsewidth | PW |  | 1 Tc | 1.3 Tc | µs |

Proposal 2: propose RAN4 to define the ACLR requirement as 30dB for 1st adjacent channel and 40dBc for 2nd adjacent channel.

| R2D carrier bandwidth [KHz] | Freq offset next to the carrier [KHz] | ACLR1 [dB] | ACLR2 |
| --- | --- | --- | --- |
| 200 | 100 | 30 | 40 |
| 400 | 100 | 30 | 40 |
| 600 | N/A | 30 | 40 |
| 800 | N/A | 30 | 40 |

Proposal 3: propose to consider the 10dB relaxation on top of standalone NB-IoT BS emission mask.

Proposal 4: propose to use the above FRC as REFSENS definition and SNR as -4.2dB for BPSK and 0.5dB for OOK reception.

Proposal 5: propose to define the input power of CW signal as -36dBm.

Proposal 6: propose to define the REFSENS relaxation as 36dBc on top of baseline REFSENS requirement;

Proposal 7: propose not to define the dynamic range requirement for A-IoT BS.

Proposal 8: propose to define the ACS requirement as 20-30dBc for A-IoT BS with 3MHz DFT-s-OFDM signal configuration.

Proposal 9: propose not to define the Rx IMD and NBB IMD requirement at the current phase.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR

- R4-2502235	On the RF requirements for Ambient IoT Device, Huawei , Noted

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2508101,WF on requirements for A-IoT BS and CW, CMCC, approved.

TP

7	Conducted receiver characteristics

7.1	General

Conducted receiver characteristics are specified at the antenna connector for BS type 1-C , with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements apply during the BS receive period.

-	Reference requirements defined for the conducted receiver characteristics do not assume HARQ retransmissions.

NOTE 1:	In normal operating condition, A-IoT BS is configured as HD-FDD operation.

7.2	Reference sensitivity level

7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C at which a throughput requirement shall be met for a specified reference measurement channel.

7.2.2	Minimum requirements for BS type 1-C

The BLER shall be less than or equal to 10% of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 for A-IoT Medium range BS.

Table 7.2.2-1: A-IoT Medium range BS reference sensitivity levels

| BS channel bandwidth (KHz) | DSB (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | A-FR1-A1-1 [BPSK] | TBD |
|  |  | A-FR1-A1-2 [OOK] | TBD |
| 200 | TBD | A-FR1-A1-1 [BPSK] | TBD |
|  |  | A-FR1-A1-2 [OOK] | TBD |
| NOTE 1: Reference sensitivity power level is defined based on the CW power at the antenna connector as TBD power. |  |  |  |

[7.3	Dynamic range

7.3.1	General

[The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C inside the received BS channel bandwidth. In this condition, a BLER requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.]

7.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The BLER shall be less than or equal to 10% of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.3.2-1 for A-IoT Medium Range BS.

Table 7.3.2-1: A-IoT Medium range BS dynamic range

| BS channel bandwidth (MHz) | DSB (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| TBD | 15 | G-FR1-A2-1 | TBD | TBD | AWGN |
|  | 15 | G-FR1-A2-2 | TBD |  |  |
| NOTE 1: Reference sensitivity power level is defined based on the CW power at the antenna connector as TBD power. |  |  |  |  |  |

- ]

Annex A (normative):
Reference measurement channels

A.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking, (BPSK, OOK)

| Reference channel | A-FR1-A1-1 | A-FR1-A1-2 |  |  |
| --- | --- | --- | --- | --- |
| DSB (kHz) | 15 | 15 |  |  |
| Payload size (bits) | 96 | 96 |  |  |
| CRC (bits) | 16 | 16 |  |  |
| Preamble length | 31 | 31 |  |  |
| Midamble length | 31 | 31 |  |  |
| Midamble configuration I | 48 | 48 |  |  |
| FEC | 1/3 | 1/3 |  |  |
| Line code | Manchester | Manchester |  |  |
| Modulation | BPSK | OOK |  |  |
| Waveform (CW) | unmodulated single tone | unmodulated single tone |  |  |
| Sampling frequency（SFO） | between 0.01 and 0.1 | between0.01 and 0.1 |  |  |
| Total symbols | 398 | 398 |  |  |