3GPP TSG-RAN WG4 Meeting #116                                                        R4-2511133

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the impacts of BS-to-BS isolation analysis on the legacy co-location requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To capture the impacts of BS-to-BS isolation analysis on the legacy co-location requirements: - Transmitter OFF power; - 2) Co-location spurious emission requirement/Protection of BS receiver of own or different BS - 3)Transmitter intermodulation requirement - 4)Co-located OOBB requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some background information of co-location requirement is missing in this TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

6.4.X	The Impact of BS-to-BS isolation analysis on the legacy co-location requirements

6.4.X.1	General

RAN4 has identified the impacts of BS-to-BS isolation analysis on the following legacy co-location requirements:

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of BS receiver of own or different BS

- Transmitter intermodulation requirement

- Co-located OOBB requirement

In the following sub-clause, the impacts on each individual requirement would be further elaborated.

6.4.X.2	Transmitter OFF power requirement

FR1 NR BS transmitter OFF power is defined in terms of PSD in 1MHz and derived according to the following approach:

Transmit OFF power < kT0  + NF – 6dB + MCL + 10log(1MHz)

< -174 + 5 – 6 + MCL+60

< -115dBm/MHz + MCL

Considering the worst case is that eNBs are assumed to be co-located, the minimum coupling loss between co-located BSs would have the impact on the definition of core requirement. For the existing FR1 transmitter OFF power -85dBm/MHz, it is based on the assumption of MCL between two co-located BS as 30dB.

6.4.X.3	Co-location spurious emission requirement/Protection of BS receiver of own or different BS

- The Spurious Emission Level of  FR1 NR BS should be:

- Spurious Emission LevelWA ＜ kT0 + NF  + 10*log10(100kHz) －7dB + MCL

- = -174dBm/Hz + 5dB + 50dB－7dB + MCL

- Where :

- MCL refer to the minimum coupling loss between co-located BSs, therefore the BS-to-BS isolation analysis would have the impact on the definition of the co-location spurious emission requirement/Protection of BS receiver of own or different BS. MCL is assumed as 30dB in the existing specification.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, it is assumed that the    WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

6.4.X.4	Transmitter intermodulation requirement

- For FR1 transmitter intermodulation requirement, interfering signal level is equal to Rated total output power (Prated,t,AC) in the operating band – MCL, therefore the BS-to-BS isolation analysis would have the impact on the definition of transmitter intermodulation requirement.. MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP.

[Table 6.7.2.1-1: Interfering and wanted signals for the co-location transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | NR single carrier, or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers, with NB-IoT operation in NR in-band if supported. |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5. |
| Interfering signal level | Rated total output power (Prated,t,AC) in the operating band – 30 dB |
| Interfering signal centre frequency offset from the lower/upper edge of the wanted signal or edge of sub-block inside a sub-block gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 38.141-1 [5] provides further guidance regarding appropriate test requirements. NOTE 2: In Japan, NOTE 1 is not applied in Band n77, n78, n79. |  |

]

6.4.X.5	Co-located OOBB requirement

- For FR1 co-located OOBB requirement, interfering signal level for WA, MR and LA is calculated based on the following assumption:

- 46dBm-MCL;

- 38dBm-MCL;

- 24dBm-MCL;

- Where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP, therefore the BS-to-BS isolation analysis would have the impact on the definition of receiver’s co-located OOBB requirement.

[Table 7.5.3-1: Blocking performance requirement for NR BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power for WA BS (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2, and 7.2.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

]

<End of CR>