3GPP TSG-RAN WG4 Meeting#116                                R4-2511134

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.9.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on Coexistence spurious emission requirements

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the RAN#106 meeting, the following objective is added for the reply LS to ETSI TFES. In this contribution, we would like to share our initial understanding on this work and its corresponding work plan for it.

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

Discussion

## 2.1. Antenna pattern evaluation of spurious emission at 6.4GHz

Based on the agreed coexistence scenario, aggressive system is at 2GHz or 3.5GHz and victim system is at 6.4GHz, and then the spurious emission of 2GHz and 3.5GHz will fall into the receiver bandwidth of both BS or UE. One key question how to model this antenna pattern for this spurious emission at 6.4GHz is quite critical, therefore in the following section, some initial simulation results are provided for further discussion in RAN4.

Way forward: - Aggressor carrier frequencies: prioritize 2 GHz, 3.5 GHz, - Victim frequency bands: prioritize lower bound frequency of band n104, i.e., 6425 MHz.

In the following section, radiation pattern at 2GHz and U6GHz of 2GHz antenna element and 3.5GHz antenna elements are provided to check the performance of wanted signal and spurious emission signal.

The simulation results for 2GHz antenna element are summarized in the following table 2.1-1:

Table 2.1-1. summarized simulation results for 2G antenna element

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.1dB | 6.22dB |
| VSWR | <2 | 2.2-4.4 |

Figure 2.1-1. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

The simulation results for 2GHz antenna sub-array with 1x4 configuration are summarized in the following table 2.1-2:

Table 2.1-2. summarized simulation results for 2GHz antenna sub-array with 1x4 configuration

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 13.4dBi | 10.4dBi |
| VSWR | <1.7 | <4.56 |

Figure 2.1-2. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

For the 3rd harmonic of 2GHz falling into U6GHz bands, the antenna aperture size of 2GHz antenna element/sub-array is indeed about 3 times of U6GHz which make the suitable for 3rd harmonic transmission. Based on the initial simulation results, comparable antenna gain could be still achieved at U6GHz to 2GHz band.

Observation 1: antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz.

The simulation results for 3.5GHz antenna element are summarized in the following table 2.1-3:

Table 2.1-3. summarized simulation results for 3.5G antenna element

|  | 3.5G | U6G |
| --- | --- | --- |
| Operating frequency | 3300-3700MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.7dB | -4.1dB |
| VSWR | <1.7 | 2.1-8.3 |

Figure 2.1-3. antenna gain diagram at 3.5GHz antenna element [3.5GHz left, U6GHz right]

For the 2rd harmonic of 3.5GHz falling into U6GHz bands, however due to the internal antenna cancellation in the antenna, this kind of even harmonic signal could be well alleviated as shown in the figure 2.1-3.

Observation 2: the antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal cancellation within the antenna.

All above evaluations are based on the antenna radiation pattern performance, for other factors e.g. VWSR of harmonic signal, the correlation of harmonic signal after the unlinear component, the impacts of phase shift on the harmonic signal, all these factors should be well considered.

Proposal 1: propose to further consider the beamforming gain for harmonic signal based on the OTA measurement results.

Proposal 2: propose not to consider the beamforming gain for the non-harmonic signal;

## 2.2. Simulation assumption for coexistence spurious emission requirements

The antenna configurations for U6GHz BS are summarized as following and whole antenna array is considered from the victim receiver perspective:

| Parameter | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | N/A |
| M | 8 | 8 | 8 |
| N | 16 | 16 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | −60≤φescan≤60 deg. | −60≤φescan≤60 deg. | −60≤φescan≤60 deg. |
| r | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 37 dBm |
| mech | 6 deg. | 6 deg. | N/A |

Proposal 3: propose to consider the 7GHz BS antenna configuration as captured in TR38.922 clause 5.4 for the assumption of U6GHz evaluation.

The antenna configuration for 2GHz BS and 3.5GHz are summarized as following and only antenna element pattern is considered without any beamforming gain assumed for 2nd and 3rd harmonic signal within U6GHz band.

| Parameter | Macro Rural | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 4 | 4 | 4 | 8 |
| N | 8 | 8 | 8 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | −60≤φescan≤60 deg. | −60≤φescan≤60 deg. | −60≤φescan≤60 deg. | −60≤φescan≤60 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 46 dBm | 37 dBm |
| mech | 3 deg. | 6 deg. | 6 deg. | N/A |

Proposal 4: propose to consider the 1710 to 4990 MHz BS antenna configuration as captured in TR38.922 clause 4.4 for the assumption of 2GHz and 3.5GHz.

## 2.3. Simulation results for BS2BS coupling loss

In the last RAN4 meeting, either deterministic way or system level simulation could be allowed to do the statistical analysis for BS2BS or BS2UE coupling loss analysis. In the following section, some simulation results of BS2BS MCL under the different grid shit are provided in Figure 2.3-1/2/3/4 for further discussions.

Figure 2.3-1. the network deployment layout [100% grid shift] for BS2BS coupling loss

Note: antenna elements considered for transmitter spurious emission and the whole antenna array considered from the receiver perspective which is applicable for non-harmonic spurious emission signal at least.

Figure 2.3-2. simulation results for BS2BS coupling loss at 6.4GHz, 100% grid shift

Figure 2.3-3. simulation results for BS2BS coupling loss at 6.4GHz, 100% grid shift

Figure 2.3-4. simulation results for BS2BS coupling loss at 6.4GHz, 10% grid shift

Based on the simulation results in Figure Figure 2.2-1/2/3/4/5, the BS2BS MCL values are summarized in the following table 2.3-1.

Observation 3: the BS2BS MCL under the different grid shift are summarized as following:

Table 2.3-1. the summarized BS2BS MCL under the different grid shift

| Grid shift | 99% CDF [dB] | Baseline [dB] | Relaxation [dB] |
| --- | --- | --- | --- |
| 100% | -94.69 | -67 | 27.6 |
| 50% | -91 | -67 | 24 |
| 10% | -77.76 | -67 | 10.76 |

## 2.4. Simulation results for BS2UE coupling loss

In the following section, we provide some analysis from BS2UE coupling analysis under the Micro deployment instead of Macro deployment.

Note: antenna elements considered for transmitter spurious emission and 0dBi considered from the receiver perspective which is applicable for non-harmonic spurious emission signal at least.

Proposal 5: for BS2UE CLI analysis, propose to consider the Micro deployment for the assumption.

Since antenna element pattern for the aggressive BS is fixed and UE is also omini antenna, we also use the deterministic way to figure out the worst MCL value which is slightly different from BS2BS CLI analysis where adaptive beamforming from the victim BS should be still considered.

Table 6.1.2.1.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 65.03 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 450 m is assumed. |  |  |  |

Figure 6.1.2.1.2-1: Network layout for dense urban

If we consider the antenna elements from the aggressor BS without any mechanical downtilt, based on the vertical angle between Micro BS and UE as shown in the following figure and also antenna pattern in the vertical domain, antenna gain at 73.3degree towards UE is -15.1dB, meanwhile considering the pathloss and element gain, then overall coupling loss could be calculated as:

-32.4+21*log10(d_3d)+20*log10(6.4)+6.4-15.1=-78.6dBc

Figure 2.4-1. the relative angular relationship between aggressive Micro BS and victim UE

Figure 2.4-2. the antenna element pattern in the vertical domain

The statistical simulation result for Micro deployment scenario is provided as following by the assumption as shown in Figure 6.1.2.1.2-1 with cell radius < 65.03 m.

Figure 2.4-3. the simulation results for

Based on the simulation results in Figure 2.4-3 and the above deterministic calculation, the BS2UE MCL values are summarized in the following table 1.

Observation 4: the BS2UE MCL values are summarized as following:

Table 2.4.-1. the summarized BS2UE MCL

| Deterministic coupling loss [dB] | Baseline [dB] | Relaxation [dB] |
| --- | --- | --- |
| -78.6 | -53 | 25.6 |
| Statistical coupling loss [dB] | Baseline [dB] | Relaxation [dB] |
| -85.3 | -53 | 32.3 |

## 2.5. Co-existence spurious emission requirement

Based on the coupling loss above and we also follow the legacy degradation approach.e g. UL with 0.7db ref degradation and DL with 3dB ref degradation.

Observation 5: the required BS2BS spurious emission and BS2UE spurious emission requirements are calculated in the following table.

Table 2.5.-1. the required BS2BS spurious emission requirement

| Grid shift | 99% CDF [dB] | Baseline [dB] | Relaxation [dB] | legacy NF | NF for band n104 | legacy one dBm/MHz | new one dBm/MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100% | -94.69 | -67 | 27.6 | 5 | 6 | -49 | -20.31 |

Table 2.5.-2. the required BS2UE spurious emission requirement

| coupling loss [dB] | Baseline [dB] | Relaxation [dB] | legacy NF | NF for band n104 | legacy one dBm/MHz | new one dBm/MHz |
| --- | --- | --- | --- | --- | --- | --- |
| -78.6 | -53 | 25.6 | 9 | 12.5 | -52 | -22.9 |
| -85.3 | -53 | 32.3 | 9 | 12.5 | -52 | -16.2 |

Lastly considering the band n104 is TDD band, the BS need to meet the most stringent requirement between DL and UL receiver protection. In short, based on our understanding, at least for the non-harmonic frequency position,

The coexistence spurious emission requirement could be relaxed down to -23dBm/MHz under the 100% grid shift assumption.

Proposal 6: for non-harmonic spurious emission requirement at least, the coexistence spurious emission requirement could be relaxed down to -23dBm/MHz under the 100% grid shift with isolation distance as 288m.

Conclusions

In this contribution, we shared some initial views on the applicability improvement of TRP in Annex in RAN4 and proposals are made as following:

Observation 1: antenna elements/antenna sub-array of 2GHz can still achieve comparable gain at the 3rd harmonic freq within U6GHz.

Observation 2: the antenna gain of 2nd harmonic of 3.5GHz signal is much reduced compared with 3.5GHz due to the internal cancellation within the antenna.

Proposal 1: propose to further consider the beamforming gain for harmonic signal based on the OTA measurement results.

Proposal 2: propose not to consider the beamforming gain for the non-harmonic signal;

Proposal 3: propose to consider the 7GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation.

Proposal 4: propose to consider the 2~4GHz BS antenna configuration as captured in TR38.922 for the assumption of U6GHz evaluation.

Observation 3: the BS2BS MCL under the different grid shift are summarized as following:

Table 2.3-1. the summarized BS2BS MCL under the different grid shift

| Grid shift | 99% CDF [dB] | Baseline [dB] | Relaxation [dB] |
| --- | --- | --- | --- |
| 100% | -94.69 | -67 | 27.6 |
| 50% | -91 | -67 | 24 |
| 10% | -77.76 | -67 | 10.76 |

Proposal 5: for BS2UE CLI analysis, propose to consider the Micro deployment for the assumption.

Observation 4: the BS2UE MCL values are summarized as following:

Table 2.4.-1. the summarized BS2UE MCL

| Deterministic coupling loss [dB] | Baseline [dB] | Relaxation [dB] |
| --- | --- | --- |
| -78.6 | -53 | 25.6 |
| Statistical coupling loss [dB] | Baseline [dB] | Relaxation [dB] |
| -85.3 | -53 | 32.3 |

Observation 5: the required BS2BS spurious emission and BS2UE spurious emission requirements are calculated in the following table.

Table 2.5.-1. the required BS2BS spurious emission requirement

| Grid shift | 99% CDF [dB] | Baseline [dB] | Relaxation [dB] | legacy NF | NF for band n104 | legacy one dBm/MHz | new one dBm/MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100% | -94.69 | -67 | 27.6 | 5 | 6 | -49 | -20.31 |

Table 2.5.-2. the required BS2UE spurious emission requirement

| coupling loss [dB] | Baseline [dB] | Relaxation [dB] | legacy NF | NF for band n104 | legacy one dBm/MHz | new one dBm/MHz |
| --- | --- | --- | --- | --- | --- | --- |
| -78.6 | -53 | 25.6 | 9 | 12.5 | -52 | -22.9 |
| -85.3 | -53 | 32.3 | 9 | 12.5 | -52 | -16.2 |

Proposal 6: for non-harmonic spurious emission requirement at least, the coexistence spurious emission requirement could be relaxed down to -23dBm/MHz under the 100% grid shift with isolation distance as 288m.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2504769	Way Forward for Tx spurious emissions co-existence limits re-evaluation, Huawei, approved.

- R4-2504298   Huawei, HiSilicon

- R4-2504267,   Ericsson