3GPP TSG-RAN WG4 Meeting#116                                R4-2511141

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.3.2

Source: 	ZTE Corporation

Title: 	Discussion on conformance testing for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some initial views on conformance testing for SBFD BS.

Discussion

## 2.1. Tx requirements

2.1. Test signal configuration

First of all, the minimum supported bandwidth for SBFD BS has been agreed in the following way, in other word, the current signal for SBFD conformance testing should be updated according to the latest agreement.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Note: In FR1 bands applicable to SBFD operation, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. Note: The BS channel bandwidth for SBFD operation shall be no less than 100MHz..

Proposal 1: propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Secondly, there are lots of test configuration for NR BS, however lots of test configuration which is not relevant with SBFD BS operation e.g. multi-band operation and also non-contiguous spectrum operation;

Proposal 2: propose to define the test configuration for SBFD BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |
| NRTC3: Non-contiguous spectrum operation | NRTC3: Non-contiguous spectrum operation |
| NRTC4: Multi-band test configuration for full carrier allocation | NRTC4: Multi-band test configuration for full carrier allocation |
| NRTC5: Multi-band test configuration with high PSD per carrier | NRTC5: Multi-band test configuration with high PSD per carrier |

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for SBFD BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 2: propose to define the test configuration for SBFD BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |
| NRTC3: Non-contiguous spectrum operation | NRTC3: Non-contiguous spectrum operation |
| NRTC4: Multi-band test configuration for full carrier allocation | NRTC4: Multi-band test configuration for full carrier allocation |
| NRTC5: Multi-band test configuration with high PSD per carrier | NRTC5: Multi-band test configuration with high PSD per carrier |

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.