3GPP TSG-RAN WG4 Meeting #116                                           R4-2511164

# Bengaluru, India, 25th – 29th of August 2025

Title: 	Further Updates on UE RF requirements for NTN NB-IoT TDD

Agenda Item:	7.29.5 – UE RF requirements

Release:	Rel-19

Source: 	THALES, Iridium Satellite LLC

Document for:	Discussion

# WID information

A new WID Introduction of IoT-NTN TDD mode (RP-242415, New WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024) was approved in RAN#105. That WID has been revised in RAN#106, as the study phase had been completed. Then, a work plan for Revised WID on Introduction of IoT-NTN TDD mode (RP-243293, Revised WID on introduction of IoT-NTN TDD mode, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024) is proposed for RAN 1, 2, and 4.

## Core part WI

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands.

The work objectives assume the following:

- LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763)

- Target the 1616-1626.5 MHz MSS allocated band

- Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT)

- Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO

- The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

This work item includes the following objectives:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including:

  - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2]

      - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band.

  - Other necessary impacts on higher layers [RAN2]

  - RRM and RF core requirements [RAN4]

- Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

  - Specify band numbering

  - Specify SAN and UE RF characteristics

  - Specify DL and UL channelization.

  - Specify channel bandwidth as 200 kHz

  - Note1: No NTN-NTN coexistence study needed.

  - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

## Performance part WI

Specify the following:

- RRM performance requirements, as needed [RAN4]

- Necessary SAN and UE demodulation requirements [RAN4]

- SAN conformance testing requirements [RAN4]

## Current Work Plan

Current WP from R4-2502291 (from RAN4#114):

| RAN#107 (Mar. 2025) | RAN1#121 | 0.5 | - Conclude on the solutions for the TDD support in NB IoT NTN - Further discuss (if needed) related RRC parameters and send LS to RAN2 - Finalize RAN1 CRs |
| --- | --- | --- | --- |
|  | RAN2#130 | 0.5 | - Complete the definition and RRC configuration of signaling of the periodic pattern, and associated UE procedures (if any) - Conclude support of TDD mode in NB IoT NTN - Discuss stage-2 design of TDD support in NB IoT NTN - Discuss RAN2 CRs |
|  | RAN4#115 | 0.5+0.25 | - Continue discussion on the UE AMPR, UE frequency error requirement, - Continue discussion on repetition number to be considered (associated with D/U=8 for 8 ms Rx/Tx time and N=9 for 90 ms periodicity) for both UE and SAN, - Continue discussion (at least) FRC for REFSENS, in-channel selectivity, dynamic range, and related parameters such as number of tones, Tx/Rx time, related SNR target for specific FRC configuration, - Derive related values for the at least for previous mentioned requirements: REFSENS, in-channel selectivity, dynamic range, etc., - Start discussion on UE and SAN CRs for RF part and SAN conformance testing, - Start CR split to prepare Draft CRs for the Core requirements |

Previous SAN & UE agreements from RAN4#114 were described in WF R4-2502292 (“Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF”, THALES) and from RAN4#114-bis in WF R4-2504719 (“Way Forward for [114bis][316] IoT_NTN_TDD_UE_SAN_RF”).

## Agreements from the previous WF at RAN#114-bis

Please recall the agreements from the previous WF at RAN#114-bis:

Observation 1: For a better representation, the reader shall use the following figure for information purposes:

Current NTN NB-IoT L-band representation (NTN band 249)

Topic #1: General part and band definition for [IoT_NTN_TDD]

Sub-topic 1-2: Other general topics

Issue 1-2-1: Remaining issues from other bands

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Issue 1-2-2: Operating bands for category NB1 and NB2

Agreement: Update the following phrase:

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode. Category NB1 and NB2 UE operate in TDD duplex mode for 249 band.

Topic #2: UE Requirements for [IoT_NTN_TDD]

Sub-topic 2-1: UE general core requirements

Issue 2-1-1: Discussion on UE Power Class (PC) for NB-IoT TDD NTN in TS 36.102

Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class.

Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Issue 2-1-2: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: RAN4 to further discuss what are the regulatory documents applicable for the emission requirements of 249.

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: RAN4 should discuss if regulatory requirements are defined per band or per channel, and the simulation/measurement assumption for NB-IoT NTN channel positioning with respect to the band.

Agreement: As a starting point, RAN4 can consider for discussions:

  - Option 1: Since similar frequency range between 249 and 254, A-MPR requirement for 249 shall follow simulation results in TS 36.764 and A-MPR requirements (to be defined) for 254 in TS 36.102.

1a/ As starting point, for 1-tone allocation (priority 1) – see TR 36.764:

Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

1b/ As starting point, for multiple-tone allocation (priority 2) – see TR 36.764:

Table 6.2.3.2-1: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

1c/ Other options not precluded.

  - Option 2: Re-evaluate A-MPR for 249 based on PC3/PC5 (priority for PC3) assumption using at least the following references: ETSI EN 301 441, ITU-R M.1343-1

Issue 2-1-3: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement:

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts”.

Sub-topic 2-2: Work Split for UE specification TS 36.102

Issue 2-2-1: Work Split for NB-IoT TDD NTN in TS 36.102

Agreement:

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR |  |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] |  |

## Agreements from the previous WF at RAN#115

Please recall the following agreements as part of agreed R4-2508781 “Way Forward for [115][316] IoT_NTN_TDD_UE_SAN_RF” (THALES):

Issue 2-1-1: Discussion on 254 & 249 frequency bands relationship

Continue to reuse previous agreements from RAN4#114-bis):

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Note: IoT_NTN_TDD WI currently considers the definition of 249 band in Rel-19. 254 band has been previously introduced during Rel-18 work.

Issue 2-1-2: Measurement BW for 249

RAN4 to further discuss other measurement bandwidths for the A-MPR evaluations in the context of satisfying requirements in ETSI EN 301 441 and ITU-R M.1343-1 that are currently under NS_04N in TS 36.102.

Issue 2-1-5: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement:

As starting point, the following tables could be used for band 249 NS_0xN A-MPR:

- For 1-tone allocation (priority 1)

1-Tone A-MPR for NS_0xN power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.2 dB | 0 dB | 0 dB |
| NOTE: The A-MPR is defined for a measurement bandwidth Bn (TBD) ≥ CBW (200kHz) as per [ETSI EN 301 441]. |  |  |  |

- For multiple-tone allocation (priority 2)

A-MPR for NS_0xN power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.2 dB | 0 dB |  | ≤ 0.2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |
| NOTE: The A-MPR is defined for a measurement bandwidth Bn (TBD) ≥ CBW (200kHz) as per [ETSI EN 301 441]. |  |  |  |  |

Agreement:

At RAN4#116, companies to share more details on the UE SEM measurement and/or simulation results including measurement/simulation assumptions.

Issue 2-1-6: NS flags for band 249

Agreement:

Further discuss if new NS_0xN for band 249, NS_0xN different from previous NS_04N and NS_05N.

Issue 2-1-7: UE maximum output power for category NB1 and NB2

At RAN4#114-bis it was also decided: Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class. Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Tentative Agreement for RAN4#115:

RAN4 shall update section 6.2B.1 as follows

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |
| 249 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum EIRP density limit |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) 15dBm/4kHz (peak EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
| 249 | NS_0xN | 0.2 | 1616 - 1626.5 | 27dBm/4kHz (mean EIRP density) or 15dBm/4kHz (peak EIRP density) (NOTE 1) |
| NOTE 1: As per [ETSI EN 301 441]. |  |  |  |  |

Issue 2-1-8: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Moderation Note: RAN4 should by default not exclude any kind of UE-specific implementation for the frequency pre-compensation mechanism. For the 249 band, a UE adjusts its frequency pre-compensation before/at the beginning of/during each set of consecutive uplink burst of 8 consecutive sub-frames. When segmentation is used (when segments are configured by the eNB), a UE adjusts its frequency pre-compensation at the beginning of each segment.

When segments are configured by eNB, and UE supports optional capability ntn-SegmentedPrecompensationGaps, then UE adjust also its timing pre-compensation before/at the beginning of/during each segment.

However, for the definition of the requirement:

Agreement:

The pre-compensation during the segments of an uplink burst of 8 consecutive sub-frames is considered as optional.

Agreement:

RAN4 is considering for the definition of the frequency error requirement:

- Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

- Add a note a new phrase/paragraph to the following text [Clause 6.4B.1of TS36.102] as follows:

  - Option 1:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. For band 249, UE may also adjust its frequency pre-compensation during or at the beginning the 8 consecutive uplink transmitted sub-frames.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

  - Option 2:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. For band 249, UE may adjust its frequency pre-compensation before, at the beginning of, and/or during each set of consecutive uplink sub-frames.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

  - Option 3:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Issue 2-2-1: (Updated) Work Split for NB-IoT TDD NTN in TS 36.102

Agreement:

- Update the Work Split for NB-IoT TDD NTN in TS 36.102 if needed

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| UE maximum output power for category NB1 and NB2 (Clause 6.2B.1) | [THALES]TBD |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR | [Nordic]TBD |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] | TBD |

Note: Other identified Clauses are not precluded

Issue 3-1-1: LS reply to RAN1 on pre-compensation

Agreement: The LS reply shall be based on both RF (frequency error) and RRM (timing error) requirement.

Agreement: The reply LS shall take into account both transparent and regenerative payload design and shall consider satellite and UE position error.

Note: Satellite and UE position error (17*Ts) were not included in the initial analysis for timing error, and other timing errors could be considered.

Agreement:

As starting point, RAN4 shall use as baseline for the discussion at next RAN4#116 meeting the following draft LS reply (to RAN1):

<<Start LS Reply>>

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

RAN4 received from RAN1 the following LS with the following RAN1 working assumptions:

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in IoT NTN TDD system regarding the pre-compensation. RAN4 does not see any issues with the agreement and has agreed to incorporate the agreement in TS 36.102 clause 6.4B.1 as follows:

At the RAN4#116 meeting, RAN4 reached the following agreement:

- For frequency error requirement (TS 36.102):

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Note: For TDD mode, the same TS 36.102 specification for NB-IoT NTN UE frequency error applies:

“Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

”

- For timing error requirement (TS 36.133):

[For NB-IoT NTN TDD, UE may adjust its time at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. Moreover, the time pre-compensation during the uplink burst of 8 consecutive transmitted sub-frames is considered up to UE implementation.]

Note: For TDD mode, the same TS 36.133 specification for NB-IoT NTN UE timing error applies:

“Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

”

As a general assumption for both frequency error and timing error, no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes. [However, when segmentation is applied (i.e. network configures the segmentation), UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. Therefore, RAN4 estimates that the use of pre-compensation gaps on the UE side may be allowed but is optional depending on the UE implementation. RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.]

<<End LS Reply>>

Moderator Note:

- one can find below a computation of frequency error per different configurations:

| Time considered for frequency variation computation | Maximum frequency shift | Observation |
| --- | --- | --- |
| 1 Tx burst (8ms of consecutive 8 UL sub-frames) | 3.45 Hz at 1.6 GHz | Negligible with respect to 160Hz (0.1 ppm at 1.6 GHz) |
| 9 radio frames (90ms transmission periodicity) | 38.88 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |
| 2 x 9 radio frames (180ms - multiple of 90ms transmission periodicity) | 77.76 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |

- one can find below a computation of timing error per different configurations:

| Time considered for timing variation computation | Maximum timing shift | Observations |
| --- | --- | --- |
| 1 Tx burst for regenerative payload (8ms of consecutive 8 UL sub-frames) with an assumption of +/-48 µs/sec. | 0.381 µsec @ 10deg elevation @600km LEO And other examples: [0.317 µsec @ 30deg elevation @600km LEO 0.348 µsec at 10deg elevation @800km LEO 0.306 µsec at 30deg elevation @800km LEO] | < 3.1575 µs (97*TS) Maximum timing shift = approx. 12% of Te timing error limit (without UE & Sat positioning error included and other errors) |
| 1 Tx burst for transparent payload (8ms of consecutive 8 UL sub-frames) with an assumption of +/-96 µs/sec. | 0.744 µsec @ 10deg @600km LEO As other examples: [0.634 µsec @ 30deg elevation @600km LEO 0.696 µsec @ 10deg @800km LEO 0.612 µsec @ 30deg elevation @800km LEO] | < 3.1575 µs (97*TS) Maximum timing shift = approx. 23.5% of Te timing error limit (without UE & Sat positioning error included and other errors). The total shift > 0.744 (caused by timing drift for 600km LEO @ 10deg elevation) + 0.544 µs (under assumption of Te_sat + Te_UE = 17*Ts µs which in practice may be higher) and representing 41% of Te timing error limit excluding other errors (from radio channel, propagator prediction errors etc.). Other example: The total shift > 0.612 (caused by timing drift for 800km LEO @ 10deg elevation) + 0.544 µs (under assumption of Te_sat + Te_UE = 17*Ts µs which in practice may be higher) and representing 37% of Te timing error limit excluding other errors (from radio channel, propagator prediction errors etc.). RAN4 may therefore further consider this timing drift issue for NB-IoT NTN TDD. |

Issue 3-2-1: Revisit the demodulation performance requirements for UE side

Agreement:

Demodulation and performance schedule is supposed to start in September 2025.

Companies are encouraged to discuss offline. In any case, the demodulation performance may be based on max 8ms transmission time, which may not require the modification of the current DL demodulation test configuration, just the SNR target value which needs to be re-computed.

| RAN#109 (Sep. 2025) | RAN1#123 |  |  |
| --- | --- | --- | --- |
|  | RAN2#132 |  |  |
|  | RAN4#117 | 0.25 | - Start discussion on demodulation performance and specific configuration related to the new TDD pattern |

# UE Requirements for IoT_NTN_TDD

## UE frequency error related requirements

Following RAN4#114-bis, and RAN4#115, we have the following observations and proposals for RAN4#116.

Observation 1: First of all, we do not think that the UE needs to update its frequency pre-compensation in-between 2 transmissions of 8ms, or during a transmission of 8ms, the UE just needs to update the frequency pre-compensation at the beginning of each transmission of 8ms.

The following assumptions have been considered by RAN1:

- 10 ppm initial cold start & 24 ppm for the maximum Doppler shift (before sync to NPSS)

- 0.27 ppm/sec drift (“Maximum Doppler Rate” or “Maximum Doppler shift variation” as per TR 38.821).

As a matter of fact, during 8ms transmission time the maximum expected Doppler shift (around 3.45 Hz) is under 160 Hz (corresponding to 0.1 ppm requirement for 249 L-band at 1.6 GHz), and according to RAN1 UE will need to pre-compensate before or at the beginning of each 8ms transmission burst (of consecutive 8ms uplink sub-frames).

Please find therefore below a computation of frequency shift per different durations/periods:

| Time considered for frequency variation computation | Maximum frequency shift | Observation |
| --- | --- | --- |
| 1 Tx burst (8ms of consecutive 8 UL sub-frames) | 3.45 Hz at 1.6 GHz | Negligible with respect to 160Hz (0.1 ppm at 1.6 GHz) |
| 9 radio frames (90ms transmission periodicity) | 38.88 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |
| 2 x 9 radio frames (180ms - multiple of 90ms transmission periodicity) | 77.76 Hz at 1.6 GHz | Under 160Hz (0.1 ppm at 1.6 GHz) |

Therefore, with respect to frequency error requirement, we propose the following modification:

Proposal 1 (updated from previous RAN4#114-bis and RAN4#115 options):

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note a new phrase/paragraph to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Proposal 2 (updated from previous RAN4#114-bis and RAN4#115 options) for the LS Reply to RAN1:

[For information purpose only] With respect to the LS Reply to RAN1 R1-2503142, the following text is proposed (for both RF and RRM parts):

<<Start LS Reply>>

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

RAN4 received from RAN1 the following LS with the following RAN1 working assumptions:

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in IoT NTN TDD system regarding the pre-compensation. RAN4 does not see any issues with the agreement and has agreed to incorporate the agreement in TS 36.102 clause 6.4B.1 as follows:

At the RAN4#116 meeting, RAN4 reached the following agreement:

- For frequency error requirement (TS 36.102):

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Note: For TDD mode, the same TS 36.102 specification for NB-IoT NTN UE frequency error applies:

“Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

”

- For timing error requirement (TS 36.133):

[For NB-IoT NTN TDD, UE may adjust its time at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. Moreover, the time pre-compensation during the uplink burst of 8 consecutive transmitted sub-frames is considered up to UE implementation.]

Note: For TDD mode, the same TS 36.133 specification for NB-IoT NTN UE timing error applies:

“Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

”

As a general assumption for both frequency error and timing error, no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes. [However, when segmentation is applied (i.e. network configures the segmentation), UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. Therefore, RAN4 estimates that the use of pre-compensation gaps on the UE side may be allowed but is optional depending on the UE implementation. RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.]

<<End LS Reply>>

## A-MPR and UE spectral emission mask requirements

During RAN4#114-bis it was decided to use at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

3GPP TS 36.102 indicates the following NB1 and NB2 spectral emission mask (where ΔfOOB is defined with respect to the edge of the NB-IoT Channel BandWidth):

Figure 1: NB1/NB2 spectrum Emission Mask (36.102)

Following RAN4#114-bis, we have the following observations and proposals for RAN4#115:

Observation 2: ETSI EN 301 441 unwanted emission requirements are using a “nominated bandwidth” concept, which is different from 3GPP channel bandwidth concept:

Figure 2: Maximum unwanted emissions within the band (ETSI EN 301 441)

Observation 3: As a matter of fact, ETSI EN 301 441 permits UE device manufacturers to declare a nominated bandwidth with some restrictions on the maximum nominated bandwidth that can be declared as below from Clause 3.1 ETSI EN 301 441. Basically, this ETSI EN 301 441 nominated bandwidth its one to five channel bandwidths as long as the nominated bandwidth is less than the bandwidth of the licensed spectrum, as per information below:

The rationale behind ETSI’s nominated bandwidth is related to a few reasons:

- Declaring a nominated bandwidth permits ETSI to know and control the maximum emissions that a UE will spill into adjacent licensed spectrum.

- A UE should not operate closer than its declared nominated bandwidth to its licensed band edges.

If one would represent all the previous information in one single figure, it would obtain the following limits:

Figure 3: SEM limitation with respect to different Nominated Bandwidth configurations

Observation 4: ETSI requirement seems to be very stringent for Nominated BW = CBW, and the ETSI requirement is not fully understood by all companies.

Observation 5: On the other hand, the definition is too flexible for ETSI nominated bandwidth concept with a and b values for the frequency interval (fc-a, fc-b), where fc is the carrier frequency, and (fc-a, fc-b) may vary with fc. The Nominated BW can vary for NB-IoT from 1 CBW up to 5 CBW.

Proposal 3: Further waiting for ETSI clarification on the Nominated BW concept, we further propose that in Rel-19 we use fixed “Guard Band” concept and we further discuss in Rel-20 or Rel-19 TEI the alignment with ETSI requirement.

Observation 6: As a matter of fact, the concept of the “Guard Band” already exists in current TS 36.102 specification, as it can be seen below:

With:

It can be therefore assumed that ∆fOOB domain starts after the Guard Band, as presented in the figure below:

Observation 7: Using Guard Bands (GBs) is already a concept used for NTN operation (see e.g. TS 36.102 and TS 38.101-5). SEM/∆fOOB already takes into account the Doppler/a Guard Band:

There are already some Guard Bands (GBs) in place for pre-compensation. Moreover, the NW can handle the resource allocation to assure that UEs are not interfering. One could therefore conclude:

- For in-band (within the band of the operator) the Operator seeks to find which is the acceptable interference level for its own use; => 3GPP limit is sufficient

- For out-of-band (outside the band of the operator) the Operator may use some Guard Bands to protect adjacent services. => GB required to meet ETSI limit

It is actually more convenient for the operator to have a fixed GB concept that he may use or not, depending on the deployment situation.

Moreover, for a LEO@600km constellation, the maximum Doppler is +/-24ppm for LEO@600km. At 1.6 GHz it corresponds to max 40 kHz, while at 2 GHz for instance it corresponds to max 50 kHz. A GB of 50 kHz could be therefore sufficient for the deployment.

Proposal 4: RAN4 to consider a Guard Band of [50] kHz for NTN TDD NB-IoT A-MPR definition and associated NS in Rel-19.

Moreover at least the following (potential) strategies can be considered for the resource allocation with different number of tones:

The worst case positions are with 1/3/6 tones at the edge of the band or 12 tones (full occupied 200 kHz CBW). We are further representing a few cases below for clarification, with different potential low-complexity strategies at the baseband level to reduce A-MPR:

### Simulations with 1 tone

Figure 4: Example with 1 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 4a: Zoom in on Example with 1 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Observation 8: Solutions do exist to reduce A-MPR, and 50 kHz GB seems sufficient to achieve a zero A-MPR, even with a LP filter within 1 CP.

Figure 4b: Zoom in on Example with 1 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Observation 9: Solutions do exist to reduce A-MPR, and 50 kHz GB seems sufficient to achieve a zero A-MPR, even with a low-complexity LP filter within 1 CP

### Simulations with 3 tones

Figure 5: Example with 3 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 5a: Zoom in on Example with 3 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 5b: Zoom in on Example with 3 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

### Simulations with 6 tones

Figure 6: Example with 6 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 6a: Zoom in on Example with 6 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 6b: Zoom in on Example with 6 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

### Simulations with 12 tones

Figure 7: Example with 12 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 7a: Zoom in on Example with 12 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

Figure 7b: Zoom in on Example with 12 tone allocation at the edge of the channel (worst case). ETSI limit represented in terms of Nominated BW = CBW of 200 kHz.

### A-MPR general observations

General message: Even if no LO leakage or IQ imbalance or frequency image and even non-linearity of PA is represented, these results presenting pure OFDM waveform are representative to understand the impact of different tone allocations.

Observation 10: As a matter of fact, simple low-cost filtering/pulse shaping techniques at the baseband level in digital domain are sufficient to meet both ETSI and 3GPP requirements with a zero A-MPR if a small Guard Band is considered.

Proposal 5: RAN4 to consider the following conclusions of the study:

- With 50 kHz GB, almost-zero A-MPR can be met using low-complexity Tx (digital) design with filtering/pulse-shaping

- With 50 kHz GB, 7 dB A-MPR can be met without any filtering/pulse shaping

- With 100 kHz GB, 2 dB A-MPR can be met without any filtering/pulse shaping in the vicinity of the band (assuming that some Tx RF (analog) filter is still used for all band)

Proposal 6: As starting point, the following tables could be used for band 249 NS_0xN A-MPR:

- For 1-tone allocation (priority 1)

1-Tone A-MPR for NS_0xN power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.2 dB | 0 dB | 0 dB |
| NOTE: The A-MPR is defined considering a guard band of [50] kHz adjacent to the CBW of 200 kHz. |  |  |  |

- For multiple-tone allocation (priority 2)

A-MPR for NS_0xN power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.2 dB | 0 dB |  | ≤ 0.2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |
| NOTE: The A-MPR is defined considering a guard band of [50] kHz adjacent to the CBW of 200 kHz. |  |  |  |  |

## Other issues

At RAN4#114-bis it was also decided:

Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class.

Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

At RAN4#116 we could therefore propose:

Proposal 7: RAN4 shall update section 6.2B.1 as follows

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |
| 249 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signalled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum EIRP density limit |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) 15dBm/4kHz (peak EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
| 249 | NS_0xN | 0.2 | 1616 - 1626.5 | 27dBm/4kHz (mean EIRP density) or 15dBm/4kHz (peak EIRP density) (NOTE 1) |
| NOTE 1: As per [ETSI EN 301 441]. |  |  |  |  |

# Conclusion

Proposal 1 (updated from previous RAN4#114-bis and RAN4#115 options):

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note a new phrase/paragraph to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Proposal 2 (updated from previous RAN4#114-bis and RAN4#115 options) for the LS Reply to RAN1:

[For information purpose only] With respect to the LS Reply to RAN1 R1-2503142, the following text is proposed (for both RF and RRM parts):

<<Start LS Reply>>

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

RAN4 received from RAN1 the following LS with the following RAN1 working assumptions:

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in IoT NTN TDD system regarding the pre-compensation. RAN4 does not see any issues with the agreement and has agreed to incorporate the agreement in TS 36.102 clause 6.4B.1 as follows:

At the RAN4#116 meeting, RAN4 reached the following agreement:

- For frequency error requirement (TS 36.102):

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission. [The frequency pre-compensation during the uplink burst is up to UE implementation.]

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts.”.

Note: For TDD mode, the same TS 36.102 specification for NB-IoT NTN UE frequency error applies:

“Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

”

- For timing error requirement (TS 36.133):

[For NB-IoT NTN TDD, UE may adjust its time at the beginning of the uplink burst of 8 consecutive transmitted sub-frames. Moreover, the time pre-compensation during the uplink burst of 8 consecutive transmitted sub-frames is considered up to UE implementation.]

Note: For TDD mode, the same TS 36.133 specification for NB-IoT NTN UE timing error applies:

“Table 7.20A.2-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

”

As a general assumption for both frequency error and timing error, no pre-compensation gap is needed before the beginning of each uplink burst of 8 consecutive transmitted subframes. [However, when segmentation is applied (i.e. network configures the segmentation), UE pre-compensation gap may or may not be used for timing adjustment between 2 consecutive segments. Therefore, RAN4 estimates that the use of pre-compensation gaps on the UE side may be allowed but is optional depending on the UE implementation. RAN4 also considers that segmentation can be applied during the transmission of an uplink burst of 8 consecutive transmitted subframes.]

<<End LS Reply>>

Observation 4: ETSI requirement seems to be very stringent for Nominated BW = CBW, and the ETSI requirement is not fully understood by all companies.

Observation 5: On the other hand, the definition is too flexible for ETSI nominated bandwidth concept with a and b values for the frequency interval (fc-a, fc-b), where fc is the carrier frequency, and (fc-a, fc-b) may vary with fc. The Nominated BW can vary for NB-IoT from 1 CBW up to 5 CBW.

Proposal 3: Further waiting for ETSI clarification on the Nominated BW concept, we further propose that in Rel-19 we use fixed “Guard Band” concept and we further discuss in Rel-20 or Rel-19 TEI the alignment with ETSI requirement.

Observation 6: As a matter of fact, the concept of the “Guard Band” already exists in current TS 36.102 specification, as it can be seen below:

With:

Observation 7: Using Guard Bands (GBs) is already a concept used for NTN operation (see e.g. TS 36.102 and TS 38.101-5). SEM/∆fOOB already takes into account the Doppler/a Guard Band:

There are already some Guard Bands (GBs) in place for pre-compensation. Moreover, the NW can handle the resource allocation to assure that UEs are not interfering. One could therefore conclude:

- For in-band (within the band of the operator) the Operator seeks to find which is the acceptable interference level for its own use; => 3GPP limit is sufficient

- For out-of-band (outside the band of the operator) the Operator may use some Guard Bands to protect adjacent services. => GB required to meet ETSI limit

Proposal 4: RAN4 to consider a Guard Band of [50] kHz for NTN TDD NB-IoT A-MPR definition and associated NS in Rel-19.

Observation 8: Solutions do exist to reduce A-MPR, and 50 kHz GB seems sufficient to achieve a zero A-MPR, even with a LP filter within 1 CP.

Observation 9: Solutions do exist to reduce A-MPR, and 50 kHz GB seems sufficient to achieve a zero A-MPR, even with a low-complexity LP filter within 1 CP

Observation 10: As a matter of fact, simple low-cost filtering/pulse shaping techniques at the baseband level in digital domain are sufficient to meet both ETSI and 3GPP requirements with a zero A-MPR if a small Guard Band is considered.

Proposal 5: RAN4 to consider the following conclusions of the study:

- With 50 kHz GB, almost-zero A-MPR can be met using low-complexity Tx (digital) design with filtering/pulse-shaping

- With 50 kHz GB, 7 dB A-MPR can be met without any filtering/pulse shaping

- With 100 kHz GB, 2 dB A-MPR can be met without any filtering/pulse shaping in the vicinity of the band (assuming that some Tx RF (analog) filter is still used for all band)

Proposal 6: As starting point, the following tables could be used for band 249 NS_0xN A-MPR:

- For 1-tone allocation (priority 1)

1-Tone A-MPR for NS_0xN power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.2 dB | 0 dB | 0 dB |
| NOTE: The A-MPR is defined considering a guard band of [50] kHz adjacent to the CBW of 200 kHz. |  |  |  |

- For multiple-tone allocation (priority 2)

A-MPR for NS_0xN power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.2 dB | 0 dB |  | ≤ 0.2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |
| NOTE: The A-MPR is defined considering a guard band of [50] kHz adjacent to the CBW of 200 kHz. |  |  |  |  |

Proposal 7: RAN4 shall update section 6.2B.1 as follows

### 6.2B.1	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |
| 254 | 23 | +/-2 | 20 | +/-2 |
| 253 | 23 | +/-2 | 20 | +/-2 |
| 249 | 23 | +/-2 | 20 | +/-2 |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

The UE shall meet the following additional requirements for maximum transmission power density specified in Table 6.2B.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2B.1-2: Additional requirements for transmit EIRP density

| EUTRABand | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum EIRP density limit |
| --- | --- | --- | --- | --- |
| 254 | NS_04N | 0.2 | 1610 - 1618.25 | 27dBm/4kHz (mean EIRP limit) 15dBm/4kHz (peak EIRP limit) |
|  | NS_05N | 0.2 | 1618.25 - 1626.5 |  |
| 249 | NS_0xN | 0.2 | 1616 - 1626.5 | 27dBm/4kHz (mean EIRP density) or 15dBm/4kHz (peak EIRP density) (NOTE 1) |
| NOTE 1: As per [ETSI EN 301 441]. |  |  |  |  |

# Annex – previous meeting agreements

4.1 Agreements from RAN4#113

The WF covers the contributions submitted under the following AI:

- 7.28.1 General aspects and work plan	[IoT_NTN_TDD]

- 7.28.2 Study of impact due to periodic pattern	[IoT_NTN_TDD-Core]

### Issue 0: Parameters of the baseline TDD pattern for analysis on the impact of requirements

Agreement:

- RAN4 to consider the following values for analyzing the impact of requirements, as necessary:

  - N = 9, D = {8, 20, 30}, U = {8, 20, 30}

  - N = 8, D = 20, U = 20

- Among the above sets of values, the set with N = 9 and D = 8 is considered the baseline TDD pattern for analysis, as it represents the most challenging case in terms of requirement analysis.

Note 1: The above values, based on RAN1’s discussion in RAN1#118bis meeting, are recommended examples for assessing the impact of requirements on RF, RRM, and Demodulation aspects. The exact values are subject to RAN1’s decision.

Note 2: In terms of SNR, the above analysis focuses on the requirements for normal coverage.

### Issue 1-2: Impact on SAN RF requirements

Agreement:

- The assessment of the impact on SAN RF requirements (TS 36.108) shown in the table below will serve as a baseline for further discussion on the exact requirement definition. For the requirements to be impacted for [249] band, if any, NTN IoT 254 L-band Tx/Rx requirements in TS 36.108 will serve as a baseline. If identified, new requirements can be defined for SAN RF, e.g. transient period.

| 6.2 Satellite Access Node output power | Not band specific --> No specification impact. |
| --- | --- |
| 6.3.2 RE power control dynamic range | Not band specific --> No specification impact. |
| 6.3.3 Total power dynamic range | Not band specific --> No specification impact. |
| 6.4 Transmit ON/OFF power | Add this specification to SAN starting from Rel-19 |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Not band specific --> No specification impact. |
| 6.6.4 Out-of-band emissions | Band specific --> FFS: Specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable to SAN. |
| 7.2 Reference sensitivity level | Not band specific --> No specification impact. |
| 7.3 Dynamic range | Not band specific --> No specification impact. |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Not band specific --> No specification impact. |
| 7.4.2 In-band blocking | Not applicable for SAN. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable for SAN. |
| 7.7 Receiver intermodulation | Not applicable for SAN --> No specification impact. |
| 7.8 In-channel selectivity | Not band specific --> No specification impact. |

### Issue 1-3: Impact on UE RF requirements

Agreement:

- The assessment of the impact on UE RF requirements (TS 36.102) shown in the table below will serve as a baseline for further discussion on the exact requirement definition.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [249] | Note |
| --- | --- | --- | --- |
| 6.2B.1: UE maximum output power | Yes | Yes | Support UE power class 3 (23 dBm) Support UE power class 5 (20 dBm) |
| 6.2B.1: UE maximum output power tolerance | No |  |  |
| 6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2B.3: UE additional maximum output power reduction | No | - | Note: Reuse which band A-MPR, B254 A-MPR is not defined yet and the band [249] UL frequency range is not fully aligned with B254. |
| 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes FFS | Note: For 36.102 6.3B.4, 36.102 refers to 36.101 6.3.5F, the testing period for aggerate power control may depend on different UL pattern. |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [249] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. |
| 6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of [B249]. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use IBB requirements from B255,254 and 253 |
| 7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. [B249] can re-use OBB requirements from B255, 254 and 253 |
| 7.6B.4: Narrow band blocking | No | Yes |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |
| A.1 DL reference measurement channels |  |  | NPDSCH Reference Channel for TDD should be introduced. Fixed Reference Channel for TDD should be introduced. |
| A.2 OFDMA Channel Noise Generator (OCNG) |  |  | OCNG Patterns for TDD should be introduced. |
| A.3 Testing related to Satellite Access |  |  | No impact. |
| A.4 UL reference measurement channels |  |  | Reference measurement channels for TDD should be introduced. |

### Issue 2-1: Impact on RRM requirements

Agreement:

- From the RRM requirement point of view, no impact on the following requirements is expected due to the new periodic pattern(s):

  - UE uplink timing

  - PUR

Agreement:

- For the following RRM requirements, modifications are expected due to the new periodic pattern(s):

  - RLM (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - MSG3-based channel quality report (example modifications include hypothetical NPDCCH configuration, extended evaluation period, side conditions, etc.)

  - Cell re-selection (example modifications include extended cell detection/evaluation/measurement periods, side conditions, etc.)

  - RRC Re-establishment (example modifications include extended cell detection/measurement periods, side conditions, etc.)

  - RRC Connection Redirection to Non-anchor Carrier (example modifications include updates to the definition of the symbols, side conditions, etc.)

  - Cell measurements (example modifications include extended cell detection/measurement periods, side conditions, etc.)

Note 1: The exact requirement modifications are subject to further discussion in future RAN4 meetings based on RAN1 conclusion/progress on this feature.

Note 2: Different modifications can be made for different TDD patterns, if multiple, DRX configurations, SNR values, etc.

Note 3: Not all existing requirements for NB-IoT NTN may be applicable to the IoT_NTN_TDD feature, which is subject to further discussions in future RAN4 meetings.

Note 4: It is assumed that UE will be made aware of the periodic pattern of frequency layers to be measured.

Note 5: UE power consumption aspect may be considered when modifying the above requirements as needed.

Note 6: The requirement applicability rule defined in Rel-17/18 applies to the IoT_NTN_TDD feature as well. For instance, the above requirements may not be applicable for inter-satellite scenarios if the UE is not provided with the target inter-satellite’s information.

Demodulation aspect:

No observation in this meeting.

4.2 Agreements from RAN4#114

General requirements:

Issue 1-1-1: Work Plan presented under R4-2500136.

Agreement: Updated Work Plan to be agreed under R4-2502291 (see Annex for the updated work plan).

Issue 1-2-1: Band number

Agreement: Adopt [249] E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Issue 1-2-2: E-UTRA operating band

Agreement: Confirm the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | - | 2010 MHz | 2170 MHz | - | 2200 MHz | FDD |
| 255 | 1626.5 MHz | - | 1660.5 MHz | 1525 MHz | - | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | 1616 MHz | - | 1626.5 MHz | 1616 MHz | - | 1626.5 MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. NOTE 3: Band 249 is defined only for NB-IoT operation. |  |  |  |  |  |  |  |

Issue 1-2-3: Gap between EARFCN FDD NTN and EARFCN TDD NTN

Agreement: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

Issue 1-2-4: EARFCN for TDD NTN

Agreement: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL).

Issue 1-2-5: Channel raster for TDD NTN

Agreement: Use a channel raster of 100 kHz and a Step size of <1>.

Issue 1-3-1: TS 36.108 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update table as part of E-UTRA channel numbers in Section 5.4A.2, TS 36.108:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-3-2: TS 36.108 5.7.3 Clause

Agreement: As an initial working assumption, with respect to the definition of IoT-NTN TDD L-band, add the following information in 5.7.3 for the carrier frequency and EARFCN for DL and UL respectively:

- “The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

- “The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

Moderator Note: Companies to further check.

Issue 1-4-1: TS 36.102 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update TS 36.102 with:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-4-2: TS 36.102 5.4B Clause

Agreement: Update TS 36.102 with:

== Start Change == ## 5.4B Channel arrangement for category NB1 and NB2 ### 5.4B.1 Channel spacing Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. ### 5.4B.2 Channel raster, carrier frequency and EARFCN The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1. The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification. NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0. == End Change ==

UE requirements:

Issue 2-1-1: Discussion on UE REFSENS for IoT TDD in TS 36.102

Agreement: Reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

Issue 2-1-2: Clause 7.3B Update - Reference sensitivity power level for UE category NB1 and NB2

Agreement: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 for FDD and TDD UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration.

Table 7.3B-1: Reference sensitivity for UE FDD and TDD category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Issue 2-1-3: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss AMPR for 249. Proponent to clarify at next meeting which band has to be protected through this AMPR mechanism.

Issue 2-1-4: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss the potential update of frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot/segment.

Moderator note1: further clarify if UL slot is the LTE Tx slot or is for the Tx transmission time U=8. In both cases, the issue seems to be implementation-related.

Moderator note2: [Clause 6.4B.1of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Moderator note3: According to the 2nd note, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame).

Issue 2-1-5: Discussion on EVM clause for NB-IoT TDD NTN in TS 36.102

Agreement: Add for EVM clause “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

4.3 Agreements from RAN4#114-bis

Topic #1: General part and band definition for [IoT_NTN_TDD]

Sub-topic 1-2: Other general topics

Issue 1-2-1: Remaining issues from other bands

Agreement: Remaining issues from 254 band introduction will remain out of scope of the IoT_NTN_TDD WI.

Issue 1-2-2: Operating bands for category NB1 and NB2

Agreement: Update the following phrase:

## 5.2B Operating bands for category NB1 and NB2 Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1. Category NB1 and NB2 UE operate in HD-FDD duplex mode. Category NB1 and NB2 UE operate in TDD duplex mode for 249 band.

Topic #2: UE Requirements for [IoT_NTN_TDD]

Sub-topic 2-1: UE general core requirements

Issue 2-1-1: Discussion on UE Power Class (PC) for NB-IoT TDD NTN in TS 36.102

Agreement: Use PC3 and PC5 as work assumptions. The NB-IoT TDD NTN WI shall not consider the definition of a new power class.

Note: the intention of NB-IoT TDD NTN WI was to re-use current legacy NB-IoT devices.

Issue 2-1-2: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: RAN4 to further discuss what are the regulatory documents applicable for the emission requirements of 249.

Agreement: RAN4 to consider at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

Agreement: RAN4 should discuss if regulatory requirements are defined per band or per channel, and the simulation/measurement assumption for NB-IoT NTN channel positioning with respect to the band.

Agreement: As a starting point, RAN4 can consider for discussions:

  - Option 1: Since similar frequency range between 249 and 254, A-MPR requirement for 249 shall follow simulation results in TS 36.764 and A-MPR requirements (to be defined) for 254 in TS 36.102.

1a/ As starting point, for 1-tone allocation (priority 1) – see TR 36.764:

Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

1b/ As starting point, for multiple-tone allocation (priority 2) – see TR 36.764:

Table 6.2.3.2-1: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

1c/ Other options not precluded.

  - Option 2: Re-evaluate A-MPR for 249 based on PC3/PC5 (priority for PC3) assumption using at least the following references: ETSI EN 301 441, ITU-R M.1343-1

Issue 2-1-3: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement:

  - Do not change the current frequency error requirement from TS 36.102 for NTN TDD NB-IoT:

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

  - Add a note to the following text [Clause 6.4B.1of TS36.102] as follows:

“When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

Note: For 249 band, UE is allowed to perform pre-compensation between two UpLink bursts”.

Sub-topic 2-2: Work Split for UE specification TS 36.102

Issue 2-2-1: Work Split for NB-IoT TDD NTN in TS 36.102

| Clause/Requirement | Company |
| --- | --- |
| Band definition (Clause 5.2) | Iridium |
| REFSENS update (Clause 7.3B) | THALES |
| A-MPR |  |
| Frequency error requirement | THALES |
| Blocking update (Clause 7.6B) | ZTE |
| Other topics (if any) e.g. [EVM Annex] |  |

Topic #3: SAN Requirements for [IoT_NTN_TDD]

Sub-topic 3-1: TDD specific requirements for NTN NB-IoT in TS 36.108

Issue 3-1-1: Add ON/OFF/transient definitions in TS 36.108

Agreement:

- Add the following definitions (currently not available in TS 36.108):

  - Transmitter ON period: time period during which the SAN transmitter is allowed to transmit data and/or reference symbols.

  - Transmitter OFF period: time period during which the SAN transmitter is not allowed to transmit.

  - Transmitter transient period: time period during which the transmitter is changing from the OFF period to the ON period or vice versa.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-2: OTA Transmit ON/OFF power in TS 36.108

Agreement:

- Remove Clause 9.5 (OTA transmit ON/OFF power) previously agreed at RAN#114 (while keeping Clause 6.4):

## 9.5 OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

- Update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

Issue 3-1-3: Requirement set applicability TS 36.108

Agreement: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Sub-topic 3-2: In-channel selectivity in TS 36.108

Issue 3-2-1: In-channel selectivity/OTA in-channel selectivity from specification TS 36.108

Agreement: Do not consider in-channel selectivity/OTA in-channel selectivity in the Rel-19 specification TS 36.108 (clauses 7.8 for conducted receiver characteristics and 10.9 for radiated receiver characteristics) as SAN requirement for TDD NB-IoT NTN band 249, since it was not considered in previous Rel-18 TS 36.108 specification for previous defined NB-IoT NTN bands.

Issue 3-2-2: Tables A.14-1 for TS 36.108

Agreement: Remove “in-channel selectivity” from e.g. tables A.14-1.

Sub-topic 3-3: REFSENS in TS 36.108

Issue 3-3-1: 15 kHz tone vs 3.75 kHz tone

Agreement:

- Option 1: Keep single tone 15 kHz (with different variants) and remove 3.75 kHz single-tone configuration for SAN reference sensitivity for NTN TDD NB-IoT in TS 36.108.

- Option 2: 3.75 kHz can be kept if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-2: New A14-1 variants (FRC REFSENS)

- Option 1: Consider at least one of the new A14-1 configurations below for NTN TDD NB-IoT:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | ⅓ | ⅓ | ⅓ |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Issue 3-3-3: SNR targets for new A14-1 variants (FRC REFSENS)

- Option 1: Consider the SNR target values corresponding to the new NTN TDD NB-IoT A14-1 configurations:

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Frequency offset | 0 | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 | 4 ms | 4 ms | 4 ms |
| Number of NPUSCH repetition | 1 | 1 | 1 | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 3 / 3 | 2 / 1 |
| Payload size (bits) | 32 | 32 | 16 bits | 40 bits | 24 bits |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.42 | 0.33 | 0.25 |
| Transport block CRC (bits) | 24 | 24 | 24 bits | 24 bits | 24 bits |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 192 | 192 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT if companies are bringing SNR values to support this configuration with a Tx time below 8 ms.

Issue 3-3-4: Equations and parameter values to compute REFSENS

Agreement: As starting point, the following equations and parameter values to compute REFSENS for TDD NTN NB-IoT SAN can be considered:

REFSENS [dBm] = -174 dBm/Hz noise density +10*LOG10(15*1000 Hz) bandwidth + SAN NF + IM + SNR_target,

where:

- SAN NF (Noise Figure):

  - NF LEO = 4.3 dB

  - NF GEO = 7.4 dB

- IM (Implementation Margin) = 2 dB

- (new TDD only) SNR_target values:

  - A14-1 new Option1 for TDD: -1.17dB;

  - A14-1 new Option2 for TDD: 0.54 dB;

  - A14-1 new Option3 for TDD: -0.61 dB,

Issue 3-3-5: Resulted REFSENS values (different options)

Agreement: In accordance with new A14-1 configurations, RAN4 to select between following (one or more) REFSENS values for TDD NB-IoT NTN band 249 SAN specification:

- Option 1:

  - GEO: -124.0 dBm;

  - LEO: -127.1 dBm;

- Option 2:

  - GEO: -122.3 dBm;

  - LEO: -125.4 dBm;

- Option 3:

  - GEO: -123.4 dBm;

  - LEO: -126.5 dBm;

- Other options not precluded.

| Reference channel | A14-1 for FDD | A14-2 for FDD | A14-1 new Option1 for TDD | A14-1 new Option2 for TDD | A14-1 new Option3 for TDD |
| --- | --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 kHz | 15 kHz | 15 kHz |
| Number of tone | 1 | 1 | 1 | 1 | 1 |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | QPSK | QPSK |
| Tx time (ms) | 16 | 64 | 8 ms | 8 ms | 8 ms |
| SNR target value (dB) | -2.1 dB as per TR 36.802 | .. as per TR 36.802 | -1,17 dB | 0,54 dB | -0,61 dB |
| REFSENS (dBm) | GEO: -124.9 LEO: -128.0 (current) |  | GEO: -124.0 LEO: -127.1 (new proposed) | GEO: -122.3 LEO: -125.4 (new proposed) | GEO: -123.4 LEO: -126.5 (new proposed) |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Issue 3-3-6: REFSENS (with updates from previous meeting RAN4#114)

Agreement:

- Option 1: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

  - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 new Option | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

- Option 2: 3.75 kHz can be kept for NTN TDD NB-IoT REFSENS if companies are bringing contributions to support this configuration with a Tx time below 8 ms.

Sub-topic 3-4: Dynamic range in TS 36.108

- Issue 3-4-1: A15-1 & A15-2 configurations for dynamic range

Agreement:

- Option 1:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - Reference channel A15-2 (with a 3.75kHz configuration) can be removed for NTN TDD IoT NTN dynamic range computation.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 for both FDD & TDD | A15-2 for FDD only |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

- Option 2:

  - Reference channel A15-1 (with a 15kHz configuration) can be reused for NTN TDD IoT NTN dynamic range computation.

  - If companies are providing a 3.75kHz configuration for Tx time below 8 ms, reference channel A15-2 can be kept for NTN TDD IoT NTN dynamic range computation.

- Issue 3-4-2: Equations and parameter values to compute dynamic range

Agreement: RAN4 to consider the following equations and parameter values to compute dynamic range for TDD NTN NB-IoT SAN:

- P_int = -174 dBm/Hz + 10 x log10 (BW) + NF + IoT;

- P_wan = -174 dBm/Hz + 10 x log10 (BW) + NF +IoT + SNR +IM;

with the following parameters:

  - SNR dynamic range NB-IoT = 4.8 dB (as per TR 36.802 and not -2,1 dB as for REFSENS);

  - IM=2.7 dB;

  - BW_Channel = 200 kHz for interferer;

  - BW_Channel =15 kHz for wanted signal;

  - IoT LEO=15 dB (IoT GEO =N/A);

  - NF LEO=4.3 dB (NF GEO=7.4 dB).

- Issue 3-4-3: Dynamic range double-check

Agreement:

- Re-evaluate dynamic range for NB-IoT LEO case. RAN4 to double-check the dynamic range values for TDD NTN NB-IoT SAN since computed values are P_int=-101,7 dBm; P_want=-105,4 dBm for LEO (it seems there is an error also for FDD NTN NB-IoT SAN values for LEO, and GEO should not have been considered):

Table 7.3.2-3: Dynamic range of SAN supporting standalone NB-IoT operation (LEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -89.4 | -85.7 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -95.3 | -85.7 | AWGN |

- Further discuss if dynamic range for NB-IoT GEO case should be de-scoped or not, in Rel-17 (and should have been the case also for NB-IoT in Rel-18) it was decided not to define any dynamic range requirement for SAN GEO class.

Table 7.3.2-2: Dynamic range of SAN supporting standalone NB-IoT operation  (GEO class payload)

| SAN channel bandwidth (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -97.3 | -93.6 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -103.2 | -93.6 | AWGN |

Sub-topic 3-5: General SAN requirements for NTN TDD NB-IoT

- Issue 3-5-1: Number of tones

Agreement: RAN4 may consider number of tones =1 for deriving the requirements, as long as transmission time less than 8ms.

- Issue 3-5-2: Repetition number

Agreement: RAN4 can consider Repetition number=1 to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

Sub-topic 3-6: Work Split for SAN specification TS 36.108

- Issue 3-6-1: Work Split for NB-IoT TDD NTN in TS 36.108

Agreement:

| Clause/Requirement | Company |
| --- | --- |
| Definitions/Abbreviations | Iridium |
| Band definition (Clause 5.2) | THALES |
| Conducted REFSENS update | THALES |
| Radiated REFSENS update |  |
| Dynamic range update | Iridium |
| Conducted ON/OFF requirements | Iridium |
| EVM Annex | ZTE |
| Other, miscellaneous |  |