3GPP TSG-RAN WG4 Meeting #116	R4-2511201

Bengaluru, India, 25th August – 29th August 2025

Source:	Ericsson, Rogers

Title:	TP for TR 37.719-11-11 adding DC_2A_n7A-n77A

Agenda item:			6.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_2A_n7A-n77A as defined in WID [1].

# ---Start of changes---

## 7.x	DC_2_n7-n77

### 7.x.1	Configurations for DC

Table 7.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration (NOTE X) |
| --- | --- |
| DC_2A_n7A-n77A | DC_2A_n7A DC_2A_n77A |
| NOTE X : |  |

### 7.x.2	Co-existence analysis for DC

Table 7.x.2-1: Band 2 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2500 | 2570 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 720 | 4350 | 4480 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1320 | 1130 | 3090 | 3290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6200 | 6390 | 6850 | 7050 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2980 | 3230 | 5590 | 5860 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1180 | 1440 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8050 | 8300 | 9350 | 9620 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8700 | 8960 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8090 | 8430 | 4830 | 5140 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3680 | 4010 | 730 | 410 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11850 | 12190 | 9900 | 10210 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11200 | 11530 | 10550 | 10870 |

Table 7.x.2-2: Band 2 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 2350 | 5150 | 6110 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 520 | 500 | 4690 | 6550 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 8020 | 8450 | 10310 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350 | 2430 | 7990 | 10750 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2780 | 4700 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9930 | 11750 | 14510 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10300 | 12220 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11290 | 14950 | 3200 | 4340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6080 | 8900 | 870 | 2850 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18710 | 10700 | 11840 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 16420 | 12150 | 14130 |

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 5th IMD products than may fall into band n7 and there are 5th IMD products than may fall into band n77.

### 7.x.3	∆TIB and ∆RIB values

Values are derived from DC_2-7_n77.

Table 7.x.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_2_n7-n77 | 0.6 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 7.x.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_2_n7-n77 | 0.2 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

### 7.x.4	Analysis of MSD requirements

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 5th IMD products than may fall into band n7. MSD value band n7 are derived from DC_2A-7A_n77A.

Based on Table 7.x.2-1 and Table 7.x.2-2 there are 5th IMD products than may fall into band n77. MSD value band n77 are derived from DC_2A_n7A-n78A.

Table 7.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n7A-n77A | 2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n7 | N/A | 5 | N/A | 2660 | 3.4 | IMD5 |
|  | n77 | 4120 | 10 | 50 | 4120 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n77 | N/A | 10 | N/A | 3775 | 4.2 | IMD5 |

---End of changes---

# Reference

[1] 	 RP-250966. Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia