3GPP TSG-RAN WG4 Meeting #116 	R4-2511214

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item: 7.21.4.2

Source: MediaTek inc.

Title: Discussion on SBFD Operation

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol. Therefore, several issues were brought up to the RRM discussion by multiple companies, which are captured in the WF.

# Discussion on the RRM impact due SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain:

- Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). Agreement from meeting 114-bis: - Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. Agreement from meeting 115: - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability. FFS: - Proposals for impact on measurement period due to processing time - Proposal 1 (vivo, OPPO, Samsung, CMCC, E///, HW, MTK): - Add a lower bound on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - 8ms (vivo, CMCC, E///, HW, MTK) - 10ms (OPPO) - Proposal 2 (CATT): - Existing measurement period for apply when the periodicity of CSI-RS is large enough to cover scaled processing time. Otherwise, a longer measurement delay can be expected. - Proposal 3 (Nokia): - For Case 3, if the accuracy requirements is adopted dependent on the X4 = 24 PRBs, the existing measurement delay shall apply i.e. lower bound is not needed. This applies when UE does not support simultaneous DL reception in 2 DL subbands. - For Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

Measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case: First of all, RAN4 RRM should refer to RAN1 agreement on the CSI processing delay:

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

On one hand, provided that the first UE capability indicates a UE being able to follow legacy CSI processing, hence, the measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case should be the same as legacy. On the other hand, the second UE capability increase the CSI processing delay by a factor of two, hence, RAN4 should check whether the new values of CSI processing delay would impact the CSI measurement requirements. Therefore, RRM group should check the existing spec in terms of CSI computation delay requirements as well as the CSI resource periodicity information, which are provided below:

From [38.214] Clause 5.4: Table 5.4-1: CSI computation delay requirement 1 Z1 [symbols]. Z1; Z'1. 0; 10; 8. 1; 13; 11. 2; 25; 21. 3; 43; 36. Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44,X3X3+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

From [38.331] Clause 6.3.2: #### CSI-ResourcePeriodicityAndOffset The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on. CSI-ResourcePeriodicityAndOffset information element -- ASN1START -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START CSI-ResourcePeriodicityAndOffset ::= CHOICE { slots4 INTEGER (0..3), slots5 INTEGER (0..4), slots8 INTEGER (0..7), slots10 INTEGER (0..9), slots16 INTEGER (0..15), slots20 INTEGER (0..19), slots32 INTEGER (0..31), slots40 INTEGER (0..39), slots64 INTEGER (0..63), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639) } -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP -- ASN1STOP

From the existing CSI computation delay requirements [38.214], the maximum delay is approximately 4ms regardless of the numerology being used and regardless of the CSI resource periodicity. This can be seen aligned with CSI resource periodicity of minimum of 4ms. However, provided that the new UE capability increases the CSI computation delay by a factor of 2, hence, RAN4 requirements should be at least of 8 slots instead of 4 slots. Thus, RRM to introduce new requirements where the new requirements include a minimum threshold of 8ms on top of existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Observation 1: The existing legacy CSI computation time can be up to 4ms.

Observation 2: The minimum periodicity for CSI resource configuration is 4 slots.

- For SBFD-aware UE with DUD case, and when UE indicates that CSI processing time is scaled by 2, RRM to introduce a lower bound of 8ms on top of existing delay requirements.

# Discussion on remaining issues

Issue 1-2-5: Requirements for RACH requirements:

During RAN2#127bis meeting, RAN2 made an agreement regarding RACH resource selection as below:

RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

To our understanding, the above agreement being captured in RAN2 spec, hence, there is no need to duplicate mentioning this agreement in RAN4 spec. Perhaps it worth waiting for RAN2 spec to capture the above agreement, then RAN RRM can just refer to that section in RAN2 spec.

- RAN4 RRM spec should refer to the agreement on PRACH transmission re-attempt in RAN2 spec without a need to duplicate the same line in RAN4 RRM spec.

# Summary

In this contribution, we have the following proposals:

Proposal 1: For SBFD-aware UE with DUD case, and when UE indicates that CSI processing time is scaled by 2, RRM to introduce a lower bound of 8ms on top of existing delay requirements.

Proposal 2: RAN4 RRM spec should refer to the agreement on PRACH transmission re-attempt in RAN2 spec without a need to duplicate the same line in RAN4 RRM spec.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.

- R4-2505568, “Topic summary for [115][219] NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.