3GPP TSG-RAN WG4 Meeting #116	R4- 2511221

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	7.17.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the performance monitoring and life-cycle management requirements including reporting delay for AI/ML-based CSI prediction use cases.

Discussion

Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. First, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

In RAN4 agreement, monitoring reporting is under RAN1’s scope. RAN1#120-bis and RAN1#121 have made the agreements as follows.

RAN1#120-bis Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

In RAN1 agreement, SGCS is agreed the metric of CSI prediction using UE-side model for performance metric type 3. In Type 3, the UE computes the performance metrics, reports them to the network, which then determines whether to initiate functionality fallback to legacy CSI reporting.

RAN1#121 Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

Based on the RAN1#121 agreement, the requirements for CSI prediction using UE-side models and reporting contents of UE-assisted performance monitoring are defined.

For CSI prediction using UE-side models, the agreement specifies that two types of SGCS values are calculated and reported. The first type, SGCS1, is based on the predicted CSI and the ground truth CSI. The second type, SGCS2, is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference reporting instance.

The RAN1 agreement also specifies that the SGCS values are reported with a wideband frequency granularity, only for one prediction instance configured by the Network (NW), and per layer. The total number of layers is the same as the reported RI (Rank Indication) in the associated inference report. Each SGCS value is quantized with 4-bit, using 15 codepoints to uniformly quantize in the range [0.3, 1] in linear scale. One codepoint is used to indicate the value range of (0, 0.3]. Only one monitoring resource set is configured in the CSI-ReportConfig, and the one configured time instance (i.e., the f-th Doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1.

It is also specified that the SGCS values are reported in a specific format, which includes a set of SGCSs (SGCS1 and SGCS2) with a variable number of layers (v). Each SGCS value is represented as SGCSi #k, where i is the index of the SGCS (1 or 2) and k is the layer index (1 to v). The value of v is equal to the reported RI in the associated inference report.

Based on RAN1 agreements providing a framework for reporting SGCS values for CSI prediction, when UE reports performance metric in SGCS to NW, RAN4 will study how to define the reporting delay requirements to support accurate performance monitoring.

Reporting delay in the context of AI/ML-based CSI prediction is defined as the time interval from the RRC (re)configuration, where the network activates the AIML functionality and configures the monitoring resources, to the transmission of the first performance monitoring report by the UE. This definition aligns with the principle agreed upon in beam management (BM) use cases and is applicable to CSI prediction scenarios as well. The delay requirement, representing the maximum allowable delay, is particularly relevant for the first monitoring occasion, as subsequent reports can be transmitted using pre-configured reporting resources. This delay encompasses the time needed for the UE to process the configuration, activate the prediction model, and generate the initial SGCS-based performance metric report. By adopting this definition, RAN4 can ensure consistency across use cases and support accurate performance monitoring, especially under the Type 3 monitoring framework agreed in RAN1, where SGCS serves as the key performance metric.

Therefore, RAN4 should define the delay requirement for the first monitoring report based on the agreed framework from beam management use cases. Additionally, RAN4 should study how different CSI reporting configurations (e.g., periodic, semi-periodic, aperiodic) impact the delay components.

Reporting delay of AIML CSI prediction should be defined as the time delay from the RRC (re)configuration complete message, including AIML functionality activation and monitoring resource configuration, to the transmission of the first monitoring report.

RAN4 should define the reporting delay requirements for the first monitoring report based on the agreed framework from beam management use cases.

To ensure reliable performance monitoring of AI/ML-based CSI prediction, RAN4 should consider specifying an accuracy requirement for the SGCS values reported by the UE. Since SGCS is derived from predicted and ground-truth channel information, its accuracy can be meaningfully assessed in controlled test environments where channel conditions are known. For example, under favorable channel conditions, a properly functioning prediction model should yield SGCS values close to 1. This behavior enables test equipment (TE) to evaluate whether the reported SGCS reflects accurate prediction performance. By leveraging this property, RAN4 can define threshold-based accuracy criteria, such as requiring SGCS to exceed a certain value (e.g., 0.8). and establish a corresponding testing framework.

The accuracy requirements for CSI prediction performance monitoring can be defined specifically for UE-reported SGCS1 values.

This approach justifies the feasibility and motivation for introducing accuracy requirements in UE-side performance monitoring. Hence, RAN4 should define the delay requirement for the first monitoring report based on the agreed framework from beam management use cases. Furthermore, RAN4 should discuss how to specify requirements on performance monitoring for CSI prediction.

RAN4 to define accuracy requirements for CSI prediction performance monitoring, specifically for UE-reported SGCS1 values.

A testing framework for performance monitoring of AI/ML-based CSI prediction is essential to ensure that the UE-reported metrics accurately reflect the real-time performance of the AI/ML model. This framework involves a structured procedure where the DUT (Device Under Test) and TE (Test Equipment) interact through a series of steps, including configuration, signal transmission, and performance evaluation. The DUT calculates and reports two SGCS values, SGCS1 and SGCS2, which quantify the similarity between predicted and ground truth CSI, and between ground truth and non-predicted CSI, respectively. These values are then analyzed by the TE to determine whether the AI/ML-based CSI prediction and its associated performance monitoring functionality are operating correctly. Without such a framework, the network risks making incorrect LCM decisions due to unreliable performance metrics.

The feasibility of this testing framework depends on the use of the two SGCS values, which serve as critical indicators of the AI/ML model's behavior under different channel conditions. SGCS1 and SGCS2 provide complementary insights: SGCS1 reflects the accuracy of the prediction, while SGCS2 offers a baseline comparison using non-predicted CSI. In static channel conditions, the framework checks for degradation in SGCS1 and compares it with SGCS2 to detect inconsistencies. In dynamic channels, the framework evaluates whether SGCS1 significantly outperforms SGCS2, as expected from a functioning predictor. These checks allow the TE to assess the correctness of the DUT’s performance monitoring, even without knowledge of the internal ML model, making the framework viable despite the abstraction introduced by functionality-based LCM in RAN2.

The complementary use of SGCS1 and SGCS2 enables the testing framework to assess AI/ML-based CSI prediction performance under both static and dynamic channel conditions, even without visibility into the DUT’s internal model.

However, the development and standardization of such a testing framework require further study within RAN4. The variability in SGCS values due to channel conditions and ML model quality introduces complexity in defining universal thresholds and test metrics. Moreover, since the TE lacks visibility into the DUT’s AI/ML model, it must rely solely on reported SGCS values and derived metrics to make pass/fail decisions. This necessitates a careful evaluation of the framework’s robustness, accuracy, and applicability across different implementations. Therefore, RAN4 should investigate whether it is feasible to define a standardized testing framework that ensures reliable performance monitoring for AI/ML-based CSI prediction.

RAN4 should study the feasibility of defining a standardized testing framework for performance monitoring of AI/ML-based CSI prediction using SGCS1 and SGCS2 values.

Life-Cycle Management related aspects

Following agreement was reached in RAN4#114bis meeting on the delay requirements for reporting/monitoring:

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

Below is the agreement from RAN4#115:

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

The deactivation delay for CSI prediction is also addressed below, as documented in the workflow from the RAN4#115 meeting

WF: Deactivation delay for CSI prediction: - The following questions were noted for further discussion - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting? - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation.

RAN4 is studying on how to define new delay requirements for reporting performance metrics related to the duration of the monitoring period. This is particularly relevant when the UE calculates performance monitoring metrics or performs event-based performance monitoring. The current requirements do not have the timely reporting of performance metrics. To address this issue, RAN4 needs to consider a delay requirement for CSI prediction performance monitoring using the approach established in the BM use case as a baseline.

Delay requirements for CSI prediction performance monitoring should use the approach established in the BM use case as a baseline.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- Reporting delay of AIML CSI prediction should be defined as the time delay from the RRC (re)configuration complete message, including AIML functionality activation and monitoring resource configuration, to the transmission of the first monitoring report.

- RAN4 should define the reporting delay requirements for the first monitoring report based on the agreed framework from beam management use cases.

- The accuracy requirements for CSI prediction performance monitoring can be defined specifically for UE-reported SGCS1 values.

RAN4 to define accuracy requirements for CSI prediction performance monitoring, specifically for UE-reported SGCS1 values.

The complementary use of SGCS1 and SGCS2 enables the testing framework to assess AI/ML-based CSI prediction performance under both static and dynamic channel conditions, even without visibility into the DUT’s internal model.

RAN4 should study the feasibility of defining a standardized testing framework for performance monitoring of AI/ML-based CSI prediction using SGCS1 and SGCS2 values.

Delay requirements for CSI prediction performance monitoring should use the approach established in the BM use case as a baseline.

- References

- R4-2508080, “WF on AI/ML air interface part 1,” RAN4#115, Malta.