3GPP TSG-RAN WG4 Meeting #116	R4-2511223

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Testability and Interoperability Issues for AIML Mobility

Agenda item:	7.19.4

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#115 in the scope of Rel-19 SI ‎[1]. In RAN4#116 meeting, following topics are to be discussed.

- RAN4 impacts for RRM measurement prediction

- RAN4 impacts for measurement event prediction

- RAN4 General aspects

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of testability and interoperability issues in RAN4 for AI/ML Mobility.

Discussion

This document addresses the Testability and interoperability issues related to AI/ML mobility SI in RAN4 based on the agreements and the recommended way forward of the issues from the RAN4#115 meeting. Specifically, we cover following topics:

- Testing setup,

- Testing goal,

- Multiple cells in testing setup,

- Prediction consistency in time domain,

- Test environment mismatch and

- Testing aspects of Measurement event prediction

Testing setup

Testing setup in FR1/ FR2

Issue 1-2-1: Testing Setup in FR2 Issue 1-2-2: Testing Setup in FR1 Way Forward: - For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting: - Number of SSBs per cell - Channel model - FFS the same or different for temporal/frequency domain prediction - Impact of UE trajectory - FFS the same or different for temporal/frequency domain prediction - Dynamic range of received power (consistency of channel) - Measurement prediction specific parameters - Emulation of noise and EPRE - L3 filter coefficient

One of the considerations for RAN4 in establishing RRM requirements and defining test cases for measurement prediction is the number of beams utilized within test environments. Currently, only a limited number of beams are typically simulated in the test setup.

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, For L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes. Furthermore, in some of the sub-use cases (for instance Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)) cell level measurements are predicted based on beam level measurements. Therefore, even if RAN4 agrees to consider only cell level measurements, a multi beam test setup would still be required to test such a use case.

Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may be equal to L3 beam-level RSRP predictions at Point E.

RAN4 to consider multi-beam scenarios while defining the test cases for AIML mobility.

Following is the RAN4#114 agreement for AIML Beam management use case:

Agreement: - Use CDL-based channel model as starting point

The legacy test is based on AWGN channel model which is a deterministic channel. However, for AIML Mobility and Beam management use cases, the fading channel is preferred to emulate variations of the radio channel for evaluating the prediction accuracy of the models/ functionalities.

RAN4#114 agreement for Beam management indicates the CDL channel model to be considered as the starting point for FR2. Hence the CDL channel model should be considered as a baseline for AIML Mobility for temporal and frequency domain scenarios.

The UE trajectory options considered for Beam management use case study are described in [3] clause 6.3.1. UE trajectory for spatial and time domain scenarios is currently being evaluated for RAN4 AIML Beam management use case.

For AIML Mobility use case, the different UE trajectories may be required for temporal and frequency domain scenarios since the evaluation parameters are different.  However, RAN4 should consider BM use case as the starting point for defining the reference UE trajectory.

RAN4 to consider CDL channel model (agreed for AIML Beam management use case) as baseline for AIML Mobility test.

RAN4 to wait for AIML Beam management use case outcome for reference UE trajectory and consider it as baseline for AIML Mobility test.

The measurement prediction specific parameters like Observation window, Prediction window, number of samples within the OW/PW, MRRT, skipping pattern for temporal domain case B have an impact on the performance requirements and should be part of testcase configuration. However, it does not have any impact on the overall test setup, hence should not be discussed in scope of test setup discussion.

Measurement prediction parameters in the RAN4 performance requirements have no impact on the test setup (i.e. test framework).

RAN4 should not consider the measurement prediction parameters (i.e., OW/ PW, MRRT, skipping pattern for temporal domain case B etc.) for the test setup discussions.

Impacts on Testability aspects for Time domain Case A

An example of Case A is depicted in Figure 2. Briefly, in this case, there is no measurement reduction, and the focus is instead to optimize mobility performance based on predictions.

Figure 2: Time domain case A RRM measurement prediction.

The delay parameter, δ, must be configured, which is based on the DUT (UE) capability to predict and measure concurrently or sequentially. However, when the UE does the measurement and prediction sequentially then there will be delay between the time measurement and prediction are reported. Hence, the gNB must account for those delays. In contrast, when the UE can do prediction and measurement concurrently then delay might be small to gather the values at the TE. However, in the latter case, there can be UE processing delays, which we leave to UE implementation details.

UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

TE must account for the delays in acquiring prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

Impacts on Testability aspects for Time domain Case B

In temporal domain case B, a measurement reduction is realized following the parameter MRRT (Measurement reduction rate in temporal domain). However, for a given selected MRRT value (e.g. 50%), different possible skipping patterns can be applied as shown in the example below.

Figure 3.  Example of intra-frequency temporal domain case B with MRRT of 50%

For the testing, the DUT needs to match a skipping pattern for a given configured MRRT value.

- At each iteration, the DUT should measure only at the identified slots following the skipping pattern within the current OW.

- The collected measurement samples are then input to AIML model to output predicted samples in the PW, reported to the TE.

- Additionally, DUT can be configured to measure and report the ground truth measurements within PW.

- At the TE, error (E) is calculated based on the selected evaluation metric (EM).

  - Generally, when E≤τ then it is assumed that test has passed. However, there can be a requirement that error for first EM, E1 (for absolute RSRP accuracy), and error for second EM, E2 (fore relative RSRP accuracy), both must satisfy the condition,  τ, or one of them at least. Further, there can be separate threshold values, τ1, τ2 for E1 and E2, resp.

  - Test procedure can also be looped over multiple iterations following the selected skipping pattern, where ‘Max’ value can be predefined at the TE.

    - When both E1 and E2 are configured by TE, TE calculates E, and then conditions E1 and E2 can be evaluated independently. The test results with respect to E1 and E2 are stored in the TE. The same test can be repeated until the iteration reaches to Max.

    - In addition to the aforementioned testing procedure, if the ratio of tests passed to tests failed has reached to a certain threshold, the testing can be deemed ended. For example, the ratio can be 0.9 or 90% of the tests passed out of the total number of tests.

The following two options may be considered:

- Skipping pattern may be optionally part of the ML inference configuration based on the RAN2 agreement, which means that this information is available along with OW and PW parameters

- Skipping pattern is not part of the ML inference configuration: UE may select autonomously the skipping pattern for a given configured MRRT value. However, in this case, RAN4 should agree on testcase configuration to explicitly indicate to DUT on the skipping pattern to use and identify the measurement slots.

For testing time domain prediction case B, RAN4 to follow RAN2 agreements regarding skipping pattern indication as part of the testcase configuration.

As the number of tests can increase considerably with the different possible configuration settings (MRRT with possible skipping patterns as well as OW and PW parameters), a possible optimization of the testing setup can be realized through parallel processing of the testing. In fact, using the same basis of measurements, DUT can apply different selections on the data to match different MRRT & skipping pattern configurations. Therefore, the different extracted data are used to test simultaneously different configurations (with their corresponding ML Models).

Figure 4.  Parallel processing of testing setup

For testing time domain prediction case B, RAN4 to consider parallel processing of testing procedure to evaluate different configurations (mainly MRRT and related skipping patterns).

Testing goal

Agreement from RAN4#113 meeting:

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

Way forward from RAN4#115 meeting:

Issue 1-1-7: Testing goal Way Forward: - Discuss whether to further clarify the testing goal for measurement prediction use case, e.g,, - The testing goal for measurement prediction is to verify: - The reported measured results meet certain requirements under certain conditions - The reported predicted results meet certain requirements under certain conditions - Note: the requirements include core and performance requirements - FFS the relative requirements for the difference between reported measured results and reported predicted results

Following scenarios are prioritized based on the agreement from RAN4#112bis meeting: 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell.

The goal (1st study goal defined in [1] clause 4.1) of the scenarios 2 and 3 ([1] Table 5.2.1.1-1) is to reduce measurement efforts in temporal or frequency domain by using predicted measurements and the goal (2nd study goal) of scenario 4 is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

For temporal domain case B, it is essential to understand the purpose of the measurement reduction to define the testing goals. Following are the two proposed options:

- Option 1: The predicted measurement is a replacement for the actual measurement

In this case, the predicted measurement should satisfy to the legacy accuracy requirement with additional side conditions like MRRT. Network may treat the reported measured and predicted results equally and use it for the handover decision, hence UE need not report separate measurement and predicted results. The testing goal is to verify if the predicted measurements satisfy the legacy measurement accuracy requirements.

- Option 2: The predicted measurement is not a full replacement of the actual measurement

In this case, the predicted measurement may not be used for handover decision. NW may use it for other purpose like configuring additional measurements and/or measurement events. Also in this option, network will require separate measured and predicted results to be reported by UE. The testing goal is to verify if the predicted measured results satisfy the new accuracy requirements (if defined) by RAN4.  The new requirements may be the relaxed version of the legacy accuracy requirements for the reported predictions.

For temporal domain case B, testing goal is to verify if the reported predicted results satisfy the legacy accuracy requirements with additional conditions like MRRT if the purpose of the predicted measurement is a replacement for the actual measurement (option 1).

For temporal domain case B, testing goal is to verify if the reported predicted results satisfy the new accuracy requirements if the purpose of the predicted measurement is not the replacement for the actual measurement (option 2).

For temporal domain case B, the testing goal is to verify if the reported predicted results satisfy the accuracy requirement based on the purpose of the measurement reduction (i.e. Option 1: The predicted measurement is a replacement for the actual measurement or Option 2: The predicted measurement is not a full replacement of the actual measurement).

For temporal domain case A, the aim is to improve the handover performance, hence the predicted measurement result should meet the new accuracy requirement which is yet to be defined as part of RAN4 performance requirements.

For temporal domain case A, testing goal is to verify if the reported predicted results meet the new accuracy requirement which is yet to be defined.

Multiple cells in the testing setup

Issue 1-2-3: Testing Setup for intra-frequency prediction Agreement: - Clarify the agreement in last meeting: - For intra-frequency intra-cell RRM measurement prediction/event prediction, RAN4 to consider two cells in the test: serving cell and another/target cell. - Note: The measurement and prediction are performed on the same cell.

In RAN2#128 meeting, following were the agreements related to cluster approach:

Agreements/Observations to be captured in TR - The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. - For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance. - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs

In RAN4#114bis meeting, following were the agreements for the multiple cells for the test setup:

Issue 4-1-3: Multiple cells in the testing setup Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

For inter-frequency inter-cell scenario (i.e., scenario 3), RAN2 agreements indicate that the measurements from more than one cell are required as input to the ML model/ functionality for the cluster approach. This indicates that at least two cells are required for input and at least one target cell is required for the output of the ML model/ functionality during the test.  Hence RAN4 should consider more than two cells for the inter frequency inter cell measurement prediction in the test setup.

RAN4 should consider more than two cells for inter-frequency inter-cell scenario (i.e., scenario 3) in the test setup based on the RAN2 agreements.

Based on RAN2 agreement, it is known that at least two cells should be considered as input to the ML model/ functionality for the cluster approach. The two input cells should be intra frequency cells that are not co-located. The target cell for which the prediction will be performed should be an inter-frequency cell and should be co-located with one of the input cells.

For the cluster approach, at least two input cells are required that should be intra-frequency, non co-located cells and the target cell should be inter-frequency cell and co-located with one of the input cells.

Prediction consistency in time domain

Issue 1-1-1: Prediction consistency in time domain Way Forward: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

A fading channel is required to emulate dynamic channel variation for the AIML mobility testing. However, uncontrolled channel variation will lead to increased complexity in the test verification. Hence there is a need for controlled channel variation that could involve time varying transmit power, SINR along the reference UE trajectory during the test. This will help in maintaining consistent channel condition during the prediction and the actual measurement (i.e. ground truth) for accuracy evaluation.

The following are the options for controlled channel variation:

- Option 1: Controlled variation of the transmit power at TE.

- The transmit power can be maintained the same (or minimum deviation) within the cell during the measurement and prediction periods. SINR should be maintained at a minimum level. This option is simpler since the transmitted power can be controlled by TE without the need for additional parameter variation like SINR, attenuation, LOS/NLOS etc.

- Option 2: Constant transmit power at TE with change in channel condition.

- Change in channel condition can be achieved by the following methods:

  - Change attenuation of the transmitted power

    - The transmitted power can be attenuated in a controlled manner to emulate the change in channel condition.

  - Change in the SINR of the channel

    - Transmit power may remain the same and the SINR may be induced during the test using a noise generator, for example OFDMA Channel Noise Generator (OCNG) as defined in [2] Annex A. OCNG is used to simulate channel noise and interference during testing and performance analysis.

- Option 3: Computer simulations alike test setup can be used to emulate option 1 and/or 2.

- Option 4: Reference UE trajectory should be defined based on the BM use case outcome.

As described above, option 1 is simpler as it can be handled by TE without increased complexity during the test.

UE trajectory refers to a predefined movement path, usually simulated or emulated within a test chamber—along which the device under test (DUT) is moved during testing. It defines the DUT path, how it changes position, velocity, and orientation over time during a test scenario.

It is essential to define the reference UE trajectory for AIML mobility to maintain the prediction consistency and verify if the UE meets the performance requirements along the pre-defined path. Currently, UE trajectory study is ongoing for the AIML BM use case. Hence, RAN4 should wait for AIML BM use case defined UE trajectory and consider it as baseline.

RAN4 should consider controlled variation of the transmit power at TE (i.e. option 1) to maintain prediction consistency since TE can maintain similar transmit power during observation and prediction windows without additional configurations like attenuation, SINR, LOS/NLOS etc.,

RAN4 should consider reference UE trajectory (i.e. option 4) to maintain the prediction consistency in time domain based on the ongoing discussions for AIML BM use case,

RAN4 should consider the combination of Option 1 and 4 to ensure Prediction consistency in time domain.

Test environment mismatch

Issue 1-2-6: Test environment mismatch Way Forward: - Discuss whether and how the following aspects should be considered for AI/ML mobility testing: - Data consistency between training and inference - Data consistency between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment

Following is the RAN4#114 agreement for CSI prediction:

Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Following is the RAN1#118bis agreement for Consistency of Positioning use case:

Consistency for Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning): Agreement: For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Reference Tables: [1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0 [2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

Following is the RAN1#119 agreement for Consistency of Positioning use case:

Consistency for Case 1 (UE-based positioning with UE-side model, direct AI/ML positioning): Agreement: For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE.

The consistency between training and inference are currently under discussion for the CSI and Positioning AIML use cases.

The dataset used for training the ML model/ functionality is an important aspect for the AIML mobility use case since it impacts the performance of the ML model/ functionality. The configuration used for the training dataset should be aligned with the inference of the model, else it will lead to inaccurate results. For example, if the input number of cell/ beams to the ML model/ functionality during training is not same as the input number of cells/ beams used during inference, the output of the ML model/ functionality will be inaccurate. Another example is the mismatch in the radio environment and observation/ prediction window configuration. If the radio condition is static/ semi-static during training and the radio condition is dynamic during the inference, the measurements during observation window and prediction window cannot be correlated leading to inaccurate predictions.

Hence, RAN4 should identify the minimum set of test configuration parameters to maintain consistency between the training and inference during the test for the UE sided AIML models/ functionalities.

RAN4 to study the minimum test configuration to be specified in order to ensure the consistency between training data set and test environment for the UE sided AIML models/ functionalities.

Testing aspects of Measurement event prediction

Issue 1-3-1: Testing aspects of measurement event prediction Proposals and Observations: - Proposal 1: - For the testing of event prediction, RAN4 to study how to know the real event occurs.

For the indirect measurement event prediction (e.g., temporal domain case A, temporal domain case B, or frequency domain) as illustrated in Figure 5, the RRM measurement prediction model first estimates the measurement result(s), optionally using AI/ML. The predicted result(s), optionally combined with historical measurement data from the same cell, are then used to determine whether a measurement event will occur (at a given future time instance). The subsequent post-processing is performed without further AI/ML processing.

Figure 5: : Indirect measurement event prediction

For legacy measurement-based reporting, several conformance test cases already exist to verify correct UE behaviour for various scenarios (e.g., FR1/FR2, intra-/inter-frequency). For example, test case A.6.6.1.1 “SA event triggered reporting tests without gap under non-DRX” verifies that the UE correctly reports an intra-frequency Event A3 when configured accordingly. In this test, the UE in RRC_CONNECTED on the PCell is configured with Event A3 and shall transmit exactly one Event A3-triggered measurement report within the specified reporting delay. No event-triggered measurement reports shall be sent if the reporting criteria are not met. The rate of correct event occurrences shall be at least 90% over repeated runs.

For event prediction testing, in Case A, one approach is to configure the UE to also trigger a conventional (actual) event alongside the prediction, enabling direct comparison between predicted events and those triggered by actual measurements. In Case B (temporal domain), ground truth can be derived from known channel conditions in conducted tests or from UE measurement reports (e.g., obtained under identical, repeated conditions). This enables verification of the UE’s predicted events against a reliable reference. In this regard, RAN4 could draw inspiration from and potentially reuse the agreed evaluation method for indirect measurement prediction.

For indirect event prediction, UE event reports based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, can be used to analyse the occurrence of the real event.

For the testing of event prediction, RAN4 may use UE measurement event reporting based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, for analysing the occurrence of the real event.

For the testing of event prediction, RAN4 may wait further progress on testing procedures for measurement prediction and consider reusing the agreed prediction evaluation approach for indirect event prediction.

Conclusion

In this paper, we have shared our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. The following Observations and Proposals were made.

Observation 1: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may be equal to L3 beam-level RSRP predictions at Point E.

Proposal 1: RAN4 to consider multi-beam scenarios while defining the test cases for AIML mobility.

Proposal 2: RAN4 to consider CDL channel model (agreed for AIML Beam management use case) as baseline for AIML Mobility test.

Proposal 3: RAN4 to wait for AIML Beam management use case outcome for reference UE trajectory and consider it as baseline for AIML Mobility test.

Observation 2: Measurement prediction parameters in the RAN4 performance requirements have no impact on the test setup (i.e. test framework).

Proposal 4: RAN4 should not consider the measurement prediction parameters (i.e. OW/ PW, MRRT, skipping pattern for temporal domain case B etc.,) for the test setup discussions.

Observation 3: UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

Proposal 5: TE must account for the delays in acquiring prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

Proposal 6: For testing time domain prediction case B, RAN4 to follow RAN2 agreements regarding skipping pattern indication as part of the testcase configuration.

Proposal 7: For testing time domain prediction case B, RAN4 to consider parallel processing of testing procedure to evaluate different configurations (mainly MRRT and related skipping patterns).

Observation 4: For temporal domain case B, testing goal is to verify if the reported predicted results satisfy the legacy accuracy requirements with additional conditions like MRRT if the purpose of the predicted measurement is a replacement for the actual measurement (option 1).

Observation 5: For temporal domain case B, testing goal is to verify if the reported predicted results satisfy the new accuracy requirements if the purpose of the predicted measurement is not the replacement for the actual measurement (option 2)

Proposal 8: For temporal domain case B, the testing goal is to verify if the reported predicted results satisfy the accuracy requirement based on the purpose of the measurement reduction (i.e. Option 1: The predicted measurement is a replacement for the actual measurement or Option 2: The predicted measurement is not a full replacement of the actual measurement).

Proposal 9: For temporal domain case A, testing goal is to verify if the reported predicted results meet the new accuracy requirement which is yet to be defined.

Proposal 10: RAN4 should consider more than two cells for inter-frequency inter-cell scenario (i.e., scenario 3) in the test setup based on the RAN2 agreements.

Proposal 11: For the cluster approach, at least two input cells are required that should be intra-frequency, non co-located cells and the target cell should be inter-frequency cell and co-located with one of the input cells.

Observation 6: RAN4 should consider controlled variation of the transmit power at TE (i.e. option 1) to maintain prediction consistency since TE can maintain similar transmit power during observation and prediction windows without additional configurations like attenuation, SINR, LOS/NLOS etc.

Observation 7: RAN4 should consider reference UE trajectory (i.e. option 4) to maintain the prediction consistency in time domain based on the ongoing discussions for AIML BM use case,

Proposal 12: RAN4 should consider the combination of Option 1 and 4 to ensure Prediction consistency in time domain.

Proposal 13: RAN4 to study the minimum test configuration to be specified in order to ensure the consistency between training data set and test environment for the UE sided AIML models/ functionalities.

Observation 8: For indirect event prediction, UE event reports based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, can be used to analyse the occurrence of the real event.

Proposal 14:   For the testing of event prediction, RAN4 may use UE measurement event reporting based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, for analysing the occurrence of the real event.

Proposal 15: For the testing of event prediction, RAN4 may wait further progress on testing procedures for measurement prediction and consider reusing the agreed prediction evaluation approach for indirect event prediction.

- References

- TR 38.744 V1.0.0 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Rel. 19).

- TS 38.133 V18.9.0 - 5G Requirements for support of radio resource management.

- TR 38.843 V18.0.0 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.