3GPP TSG-RAN WG4 Meeting #116	R4-2511229

Bangalore, India, 25th – 29th August 2025

Agenda Item:	7.13.3

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx reception

Document for:	Discussion

# 1	Introduction

After the discussion in RAN4 115 meeting in Malta, companies have reached agreements on almost all open issues. However, there are still two issues left for further discussions.

In this contribution, we provided our views on left two issues so that to boost the completion of this WI. Our corresponding simulation results can be found in the accompanied paper under this agenda item.

# 2	Discussion

In this section, we discussed the left open issues according to the agreed WF [1].

Regarding the definition of MMSE-IRC receiver capability for 8Rx UE, based on the agreed WF[1], there are several candidate options:

Definition of MMSE-IRC receiver capability for 8Rx UE Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

The current definition in the spec. is as follows:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

The differences among all options can be summarized as below:

|  | Core changes | Impacts on RAN2 |
| --- | --- | --- |
| Option 1 | Add IRC both in the name and in the content | Yes |
| Option 2 | Add IRC only in the content | No |
| Option 3 | No IRC at all; remove SU-MIMO from the name and the content | Yes |
| Option 4 | No IRC at all; remove SU-MIMO only from the name | Yes |
| Option 5 | Reuse existing definition in Rel-18 | No |

In our view, we support option 3. For option 3, we think it is the clearest definition among all options. Moreover, it is aligned with previous requirement definitions using MRC/IRC receiver to not mention the exact receiver type at all since they are all the default receiver types for most of NR requirements. The reason we mentioned ‘R-ML’ in its definition is because it is different from the default receiver assumption. In the meantime, removing ‘SU-MIMO’ both from the name and the content can avoid potential misunderstanding on the applicable scenarios of this receiver.

Proposal 1: Consider option 3 for the receiver type definition

As for the CQI reporting requirements, companies have agreed to use [-2dB] SINR for the requirement definition. For the gamma value, however, it has not been aligned based on the submitted results so far.

According to our simulation results[2], we first confirm that -2dB SINR is feasible for the requirement definition. As for the gamma value, we propose to use 2.0 after considering some margin.

Proposal 2: Consider SINR = -2dB, Gamma = 2.0 for the requirement definition

# 3	Summary

In this contribution, we shared our views and results on left open issues for introducing UE demodulation and CQI reporting requirement. Our proposals are summarized as follows:

Proposal 1: Consider option 3 and 5 for the receiver type definition

Proposal 2: Consider SINR = -2dB, Gamma = 2.1 for the requirement definition

# References

[1]	R4-2508620: Way forward for [115][323] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #115, Malta

[2]	R4-2511230: Simulation results for MMSE-IRC receiver for 8Rx reception, Ericsson, RAN4 #116, Bangalore, India