3GPP TSG-RAN WG4 Meeting #116	R4-2511233

Bangalore, India, 25th – 29th August 2025

Agenda Item:	7.1.4

Source:	Ericsson

Title:	Simulation results for 6Rx UE demodulation

Document for:	Information

# 1	Introduction

In RAN4 #114bis meeting, companies have agreed to introduce new 6Rx UE demodulation requirements which include PDSCH, SDR requirements. In this contribution, we provided our initial simulation results for the purpose of facilitate the test parameter assumptions.

# 2	4 MIMO layer cases

In this section, we mainly evaluate the performance with different candidates for MCS and propagation condition for 4 MIMO layer cases.

## 2.1	Simulation assumption

Table 2.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

## 2.2	Simulation results

Evaluations for Issue 2-2-1: MIMO correlation for 4 MIMO layers for Handheld UE testing

- Agreement

- Introduce correlation matrix for 6Rx in the specification:

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1

- Proposal

- Option 1: with α = 0 and β = 0.07179

- Option 2: with α = 0.3 and β = 0.07179

- Option 3: with α = 0 and β = 0

Table 2.2-1 Evaluation cases and results

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 10 / 15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, α = 0 and β = 0.07179 | 70 | 22 |
| 2 | 4 | 10/15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, α = 0.3 and β = 0.07179 | 70 | 29.5 |

Evaluations for Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

- Agreement: Down select the following two options in the next RAN4 meeting

- Option 1: MCS 20 (Table 2)

- Option 2: MCS 17 (Table 1)

Table 2.2-2 Evaluation cases and results

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 10 / 15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, ULA low | 70 | 11.5 |
| 2 | 4 | 10 / 15 | MCS 20 (Table 2) | TDLA30-10 | 4x6, ULA low | 70 | 13.5 |

# 3	SDR requirements

In this section, we mainly evaluate the performance of 1024QAM with 4 MIMO layers.

Figure 3.1-1 PDSCH performance of 1024QAM with 4 MIMO layers

# 4	Summary

In this contribution, we shared the simulation assumptions and initial results for PDSCH demodulation and SDR for 6Rx UE demodulation.

# References

[1]	R4-2508727: Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #115, St Julian’s, Malta