3GPP TSG-RAN4 Meeting #116	R4-2511238

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5980 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on Rel-18 LTM PDCCH ordered RACH core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements RAN4 agreement made in RAN4#115 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN4#115 agreement (R4-2508453) <Agreement>: The minimum gap from the PDCCH-order to the RO, in RAN4, is re-defined as ‘Max(X ms, the current RAN4 minimum timeline requirement).’ - X = 3.5 for intra-BWP early PRACH - X = 5.5 for inter-BWP early PRACH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 6.2.2C	PDCCH ordered Random Access for LTM

#### 6.2.2C.2	PDCCH ordered Random Access delay

PDCCH ordered RACH delay TLTM-PDCCH-order is the delay from PDCCH ordered RACH command until the time when the UE shall complete the RACH transmission to the  LTMcandidate cell.

The LTM PDCCH ordered RACH delay is defined as: TSSB∆RF/BB preparation

TLTM-PDCCH-order = Max (Tfirst-RO, NT,2 + ∆BWPSwitching + ∆Delay + Tswitch + TSSB + ∆RF/BB preparation)

Where:

-    Tfirst-RO is the minimum time gap between the PDCCH ordered RACH command to the first candidate cell RACH occasion UE shall be able to transmit (RO), where:

-    If PRACH resource is within the active UL BWP of serving cell(s), the Tfirst-RO is 3.5ms

-    If PRACH resource is outside of the active UL BWP of serving cell(s), the Tfirst-RO is 5.5ms

-     NT,2, ∆BWPSwitching, ∆Delay and Tswitch are defined in TS 38.213 [3] clause 8.1

-     TSSB 	TSSBis the time for T/F tracking and acquiring full timing information of the target cell

-    TSSB 	TSSB= 0, provided:

-	L1-RSRP measurement is configured and

-	The target LTM candidate cell is on an FR1 carrier whose TCI state associated with SSB indicated in the PDCCH-order is activated, and the time between the MAC-CE activating the target TCI state and the PDCCH is larger than TCI state activation delay defined in clause 8.25.3, and

-	L1-RSRP measurement period of candidate cell is not larger than 160 ms, or

-	The time between the MAC-CE activating the target TCI state and the PDCCH is not larger than TCI state activation delay defined in clause 8.25.3 + 160ms.

-	Otherwise, TSSBTSSBTfirst-SSB_RACHTSSB-proc = Tfirst-SSB-RACH + TSSB-proc, where

-	Tfirst-SSB-RACH Tfirst-SSB_RACHis the time to the first SSB occasion, after 1slot from the end of the slot of the PDCCH, when SSB is to be measured outside MG

-	Tfirst-SSB-RACH Tfirst-SSB_RACHis the time to the first SSB occasion, overlapped with MGL after 2ms and 1slot from the end of the slot of the PDCCH if the SSB to be measured within MG.

-	TSSB-proc TSSB-proc= 2 ms, which is the time for SSB processing;

-	∆RF/BB preparation ∆RF/BB preparationis the time for RF and baseband preparation. The following apply:

-	If PRACH bandwidth is outside active UL BWP of serving cell(s) but within one of configured UL BWPs of an active serving cell, the ∆RF/BB preparation ∆RF/BB preparationis the DCI based BWP switching delay specified in clause 8.6.

-	If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation ∆RF/BB preparation equals to capability value reported in pdcch-RACH-PrepTimeList-r18.

-	Otherwise, ∆RF/BB preparation ∆RF/BB preparation = 0.

<End of change>