3GPP TSG-RAN WG4 Meeting #116	R4-2511242

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on LP-WUR core requirements

Agenda item:	7.24.5.2

Document for:	Discussion

Introduction

This contribution discusses about LP-WUS measurements in idle-mode.

Discussion

Filtering requirements

In the last RAN4 meeting, it was discussed that there is no need for filtering requirements similar to PSS/SSS. Hence the following agreement was made:

Issue 1-2-4-1: On LR measurement filtering requirement <Agreement>: No need to define filtering requirement for LP-RSRP and LP-RSRQ measurements.

The justification for the above agreement was that the filtering is already considered in the number of samples required to receive the accuracy, and evaluation is based on multiple samples. However, we think that the filtering requirements is needed as the evaluation duration defined in the requirements only defines a time window rather than requiring the UE to filter measurements based on at least two samples. There should not be a measurement implementation that does not take at least two samples.

How to filter is up to UE for LP-SS, but evaluation needs to be at least two samples.

Capture in the specification that the UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurement samples.

On how to define LR evaluation requirements

Issue 1-2-4-2-3: On how to define LR evaluation requirements

Working assumption: Option 1: using x1=2*x and y1=2*y for the evaluation requirement. Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

The purpose of the simulations we performed was to resolve the number of samples required to reach a certain accuracy level within 95 percentiles with LP-WUR accuracy target of 3.5dB. Based on the simulations UE should perform two measurements samples to reach accuracy target of 3.5dB. For the evaluation, two sets of measurements are needed that meet the accuracy target. Hence, RAN4 should use 2 * (number of measurements to reach target accuracy). Based on two samples, the evaluation period would be 4 samples.

Evaluation period is 2 * (number of measurements to reach target accuracy). I.e., x = y = 2 and x1 = y1 =4.

LP-WUR status at legacy state

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Some companies are proposing that no LR measurement and evaluation requirements apply at the legacy state.  As discussed offline in the last meeting, we understand that there are some implementation differences between UEs (e.g. LR is part of MR or LR is an independent receiver). However, it is not currently clear for us if there are any scenarios where the LR would not need to fulfill the evaluation before entering relaxation / offloading. Therefore, the baseline in rel-19 should be that the UE always checks the LR condition before entering case 1 or case 3.

Currently it’s not clear in which scenarios the UE should not check LR before entering case 1 / 3

According to our understanding of the RAN2/4 agreements, if the LR thresholds are configured by the network, the UE shall perform LR entry threshold evaluation before entering case 1 or case 3. From this point of view, it is not clear what is meant with no LR measurement requirements apply at legacy state|. The purpose of LR entry threshold is to guarantee that LR is able to read LP-SS or PSS/SSS and that the LP-SS or PSS is above the configured threshold within the accuracy margin.

RAN2 agreements: 8.4.2 Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). Agreement on serving cell RRM relaxation criteria - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In legacy state, what is currently left up to UE implementation by RAN4 is when and how to turn on the LR. However, if MR and LR thresholds are configured by the network. For this threshold evaluation, RAN4 can either reuse the evaluation requirements already defined or define a new evaluation criteria when UE is coming from legacy state to MR relaxation / offloading. At least one set of (evaluation) measurements should be done before entering case 1/3 and the evaluation should be quite recent in order to make sure that the radio conditions that LR observes have not been changed, and LR is able to meet the thresholds with margin.

In case LR thresholds are not checked LR may end up of ping-pongs, especially when considering the measurement accuracy of MR and LR is different.

MR and LR accuracy are different, meaning that not performing any measurements with LR in legacy state may cause ping-pongs.

At legacy state, when both MR and LR entry thresholds are configured, the UE shall evaluate both MR and LR thresholds at least once before entering relaxation / offloading. When only MR threshold is configured the UE is required to only perform MR evaluation when entering relaxation / offloading.

LP-WUR and Redcap

In the previous RAN1/2 meetings the support for Redcap has been discussed. From our point of view, RedCap devices can use LP-WUS. Based on our analysis, the specification impact on RAN4 side should be reasonable if the scope is similar as it has been in RAN4 for non-redcap devices (e.g. no FR2, and no connected mode impact).

Add requirements support for RedCap without impact to connected mode requirements, and no FR2 impact, at least for LP-SS.

If 1 Rx redcap is supported, the number of samples and wake-up delays should be the same as for regular LP-WUS as otherwise this would need a separate implementation from the network side for the wake-up.

If 1 Rx is to be supported, no differentiation for 1 Rx redcap on number of samples

LP-WUR and SDT

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE or by the network. MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified for UE power saving purposes like LP-WUS and therefore it can be assumed that they should work together.

The following is capture in the TS 38.304 running CR:

5.2.4.x.3 Serving cell measurement offloading rules LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

If the UE evaluates conditions for SDT initiation when LP-WUS UE is performing serving cell measurement offloading, then MR RSRP measurement is not available since measurements are performed by LR. It needs to be decided whether LP-WUS UE can use RSRP measurement for SDT condition evaluation in this case. Also, MR RSRP measurement can be considered for this case, but using it causes delay (2 times DRX cycle), since RSRP needs to be measured according to RAN4 requirements.

SDT requirements apply also for UE supporting LP-WUS

The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading.

FR2 requirements

Issue 1-1-11: RRM requirements for FR2

When considering that FR2 would need simulations for both PSS/SSS and LP-SS based LP-WUR we think that it is better to limit the scope of the work as the WI is closing.

Prioritize defining RRM requirements for FR1 in this WI

RF impairment margin and Total accuracy margin

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Agreement: For FR1: - [±3.5] dB can be used for core requirements for LP-RSRQ accuracy and [±5.5 or ±6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB - [±3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [±5.5 or ±6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB - Note: [2 or 2.5] dB RF impairment margin is assumed

In the last meeting, RF impairment margin was discussed. Currently the simulations have been done based on 3.5dB accuracy marging, and when adding RF impairment margin to this, it means that the total accuracy marging in the specifications will be 3.5dB + RF impairment margin. 2dB margin has been used and we think that should be sufficient even for LP-WUS. The total margin is quite important as it impact how network needs to configure the exit threshold from the assumed cell centre. As the accuracy margin increases, the area where UE can relax / offload MR gets smaller, hence potential power saving is reduced.

Total accuracy margin is measurement accuracy (3.5dB) + RF impairment margin.

The larger the accuracy margin, the smaller the MR relaxation / offloading operation area is reduced.

Use 2 dB as the RF impairment margin for LP-RSRP accuracy requirements

Upper bound

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

SSB based measurements and evaluations are based on LO cycle which is same as DRX/eDRX cycle. DRX is configurable from the network so no need to define upper bound in RAN4 specification.

No need to define upper bound for SSB-based LP-WUR measurement periodicity

Idle-mode measurements and LR

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In the previous meetings, companies have brought up issues related to early measurement reporting. The rel-18 work on fast CA/DC setup expanded the idle-mode procedures to cover cell reselection measurements as well as CA/DC measurements. RAN4 should discuss whether the relaxation / offloading applies to all EMR measurements or whether the validity of the measurement results can be guaranteed when measurements have been relaxed.

Some companies have already proposed that the UE shall keep the MR measurements on while there is EMR timer running, this should also work when UE is in idle-mode less than the duration of the timer. However, we think that this should not exclude the measurements where T331 is not running.

The configuration for both EMR and LP-WUS is configurable by system information.  This means that the UE may not be configured by the same cell as the UE was released to IDLE-mode. Therefore, if the UE is configured simultaneously with EMR and LP-WUS one option is not use LP-WUS at all in that scenario.

UE may be configured with EMR by a different cell than when UE left

- Discuss about two alternatives to handle the situation

  - When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

  - UE shall not enter relaxation / offloading if configured simultaneously with EMR.

When the UE receives LP-WUS signal while configured with rel-18 EMR, the reported measurements need to meet accuracy requirements at the measurement instance and the measurements cannot be older than validityDuration, if configured. The UE fulfills at least some of the conditions there shouldn’t be a problem to report measurements with EMR framework with rel-18.

- UE may report measurements from MR relaxation with rel-18 EMR reporting

eDRX

RAN2 has made an agreement in RAN2#129bis that LP-WUS can be configured together with eDRX:

LP-WUS is supported with eDRX, FFS on exact impact if any

It would need to be clarified whether this has any impact to the LP-WUS configuration/operation, and if so, how. In legacy eDRX operation, UE wakes up every eDRX cycle to monitor paging during the PTW.

As agreed by RAN1/2, network can configure UE with the entry/exit thresholds, to determine the cell area/conditions where LP-WUS can be monitored. Currently there is no signalling to inform network whether UE is monitoring LP-WUS or whether UE has fallen back to legacy PO monitoring.  Hence, from system perspective, when network would page the UE it would need to assume that the PO’s within the PTW would be viable. Accounting this, it would appear that from LP-WUS monitoring perspective the most straight forward LP-WUS operation with eDRX would be that if UE is configured with LP-WUS, UE can monitor the LP-WUS before each paging occasion that falls within the PTW. This would be aligned with the Rel-17 PEI operation.

Proposal 6: If LP-WUS operation is configured together with eDRX, the UE is required to monitor LP-WUS signal only during PTW.

Another issue about that has been raised by the companies that whether the PTW window is sufficiently long for UE to perform measurements. Our understanding of this is that when UE is performing MR offloading / relaxation, the UE is required to monitor LO during PTW window and reply to paging during this duration. It is up to the network to transmit LP-WUS in relation to paging.

PTW window is used for PO monitoring and network guarantees that the LP-WUS is transmitted early enough before the next PO when UE is expected to wake up.

This however does not consider whether the MR evaluation needs to be contained within PTW when in MR relaxation or offloading, and whether the UE is required to measure outside of PTW in this case. We think that this issue needs further clarification as it may impact how fast MR can react to radio conditions getting worse.

Discuss if MR based RRM measurement/evaluation shall be contained in one PTW window or it can extend outside of the PTW.

MR measurements after wake-up

Issue 1-5-3: MR measurements after wake-up

One issue of using LP-WUS from the network point of view is the long time to establish measurements after UE has been in MR offloading. When UE exits MR offloading and wakes up MR due to exit threshold being met, the UE returns to legacy mode operation. If we follow idle-mode measurement requirements and measurement capability, the UE needs to detect and evaluate inter-frequency carriers which, depending on DRX or eDRX configuration takes a very long time. For this reason, when the UE exits MR relaxation / offloading, the UE should follow SSB periodicity, at least until it has detected and evaluated the cell reselection conditions for idle-mode mobility. Having good measurements for MR after exit should help to avoid ping pong between MR and LR.

After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation.

Conclusion

The following Observations and Proposals were made:

Observation 1: How to filter is up to UE for LP-SS, but evaluation needs to be at least two samples.

Proposal 1: Capture in the specification that the UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurement samples.

Proposal 2: Evaluation period is 2 * (number of measurements to reach target accuracy). I.e., x = y = 2 and x1 = y1 =4.

Observation 2: Currently it’s not clear in which scenarios the UE should not check LR before entering case 1 / 3

Observation 3: MR and LR accuracy are different, meaning that not performing any measurements with LR in legacy state may cause ping-pongs.

Proposal 3: At legacy state, when both MR and LR entry thresholds are configured, the UE shall evaluate both MR and LR thresholds at least once before entering relaxation / offloading. When only MR threshold is configured the UE is required to only perform MR evaluation when entering relaxation / offloading.

Proposal 4: Add requirements support for RedCap without impact to connected mode requirements, and no FR2 impact, at least for LP-SS.

Proposal 5: If 1 Rx is to be supported, no differentiation for 1 Rx redcap on number of samples

Proposal 6: SDT requirements apply also for UE supporting LP-WUS

Proposal 7: The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading.

Proposal 8: Prioritize defining RRM requirements for FR1 in this WI

Observation 4: Total accuracy margin is measurement accuracy (3.5dB) + RF impairment margin.

Observation 5: The larger the accuracy margin, the smaller the MR relaxation / offloading operation area is reduced.

Proposal 9: Use 2 dB as the RF impairment margin for LP-RSRP accuracy requirements

Proposal 10: No need to define upper bound for SSB-based LP-WUR measurement periodicity

Observation 6: UE may be configured with EMR by a different cell than when UE left

Proposal 1: Discuss about two alternatives to handle the situation

a.	When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

b.	UE shall not enter relaxation / offloading if configured simultaneously with EMR.

Proposal 2: UE may report measurements from MR relaxation with rel-18 EMR reporting

Observation 7: PTW window is used for PO monitoring and network guarantees that the LP-WUS is transmitted early enough before the next PO when UE is expected to wake up.

Proposal 3: Discuss if MR based RRM measurement/evaluation shall be contained in one PTW window or it can extend outside of the PTW.

Proposal 4: After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation.