3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511250

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Remaining issues for SBFD general aspects

Agenda item:			7.21.2.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints for remaining issues on the general aspects.

# 2. RAN4 aspects for SBFD system parameters

### 2.1 Declaration and flexibility for SBFD operation

Based on agreement in RAN4#114bis and RAN4#115, it is confirmed that the restriction (i.e., the SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested) shall be applied to FR1 WA BS and FR1 MR BS with TRP or total conducted power larger than a pre-defined threshold, while not applied to FR1 LA SBFD capable BSs.

<Agreement in RAN4#114> - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS <Agreement in RAN4#114bis> - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. <Agreement in RAN4#115> - For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting.

The remaining issues are (1) how to specify the pre-defined threshold for TRP or conductive power for FR1 MR BS, and (2) how to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”.

Issue-1: How to specify the pre-defined threshold for TRP or conductive power for FR1 MR BS

For FR1 MR SBFD capable BSs, it is expected that a power level e.g., a value from the range of [33, 38]dBm either as TRP or total conducted power threshold could be established. Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. Based on our analysis captured in TR 38.858, two analog filtering designs are provided to mitigate the self-interference and co-site inter-sector interference, for which 15dBc and 11.8dBc are considered. For FR1 MR BS, based on our analysis, we confirm the feasibility to have a 38dBm TX power while no analog filter is needed.

Proposal 1: For FR1 MR BS, use X = 38 dBm as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

- Note: As agreed, above this threshold, all configurations shall be tested; below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Issue-2: How to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”

For FR1 LA, FR1 MR with TRP or conductive power less than the pre-defined threshold, and all FR2 BS classes, it is “not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested”. How to interpret this agreement should be clarified.

Take the below band n77 as example, for the supported 100MHz CBW, if the FR1 LA BS support the following UL subband sizes (taken from Table 5.3.2-1 in TS38.104) and the following DL subband sizes accordingly:

|  | SCS | Channel BW | UL SB BW | DL SB BW | Guardband |
| --- | --- | --- | --- | --- | --- |
| FR1 LA BS declaration (SBFD DU) | 30kHz | 100MHz (273 RBs) | 20MHz (51 RBs) | 215 RBs | 7RBs |
|  |  |  |  | 214 RBs | 8RBs |
|  |  |  |  | 213 RBs | 9RBs |
|  |  |  | 40MHz (106 RBs) | 162 RBs | 5RBs |
|  |  |  |  | 161 RBs | 6RBs |
|  |  |  |  | 160 RBs | 7RBs |
|  |  |  |  | 159 RBs | 8RBs |
|  |  |  | 50MHz (133 RBs) | 134 RBs | 6RBs |
|  |  |  |  | 133 RBs | 7RBs |

Based on our interpretation of the principle “for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested”:

- For the 100MHz CBW (30kHz SCS):

  - UL subband size value: 20MHz (narrowest) and 50MHz (widest) shall be tested, while 40MHz is not required to be tested.

  - For 20MHz UL subband size:

    - 215 RBs as transmission bandwidth configuration for SBFD DL subband is chosen as the worst case

  - For 50MHz UL subband size:

    - 134 RBs as transmission bandwidth configuration for SBFD DL subband is chosen as the worst case

Proposal 2: For FR1 LA, FR1 MR with TRP or conductive power less than the pre-defined threshold, and all FR2 BS classes, the principle of “for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested” shall be interpreted as:

- For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations,

  - The configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

### 2.2 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based on the agreement given in RAN4#113, the option 2b is adopted to specify DL/UL subband configurations support for SBFD, with the following restrictions on SBFD configuration provided [11]:

<Agreement in RAN4#113> - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

And the option 2b is given in RAN4#112:

<Agreement in RAN4#112> - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Here we would like to further discuss the remaining issues on the restrictions on SBFD configuration.

Restriction-1: For DUD case, the UL subband should be placed in the middle of channel bandwidth

During last meeting, for DUD case, it is agreed that the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD, in which DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB. There is no remaining issue for this restriction except how to implement this agreement into the specification.

Restriction-2: Restriction on the number of UL subband sizes supported

Based on the agreement in RAN4#115, the following agreement is reached:

<Agreement in RAN4#115> - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

Based on the above proposal for “Declaration and flexibility for SBFD operation”, it is believed that the similar threshold should be adopted.

Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction of maximum 3 subband configurations is applied or not:

- i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold.

Restriction-3: Restriction on the number of DL subband sizes supported

It is agreed that for a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case, for FR1 WA BS. For FR1 MR BS, the following agreement is given in RAN4#114bis:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

Similar as the above proposal for “Declaration and flexibility for SBFD operation”, it is believed that the similar threshold should be adopted.

Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not:

- i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold.

### 2.3 Restriction on guardband size for SBFD

In RAN4#114bis meeting, the following agreement on restriction on guardband size for SBFD is provided:

Agreement: - Restriction on guardband size for SBFD: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband. - The declaration of guardband size should ensure the benefits from SBFD operation. - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

From testing perspective, as provided in Proposal 2, the configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested, therefore it is guaranteed that the guardband size should be minimized to ensure the benefits from SBFD operation.

From declaration perspective, the principle “it is expected that guardband size should be minimized to ensure the benefits from SBFD operation” is hard to be captured in RAN4 specification.

Observation 1: For the restriction on guardband size of SBFD,

- From testing perspective, as provided in Proposal 2, the configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested, therefore it is guaranteed that the guardband size should be minimized to ensure the benefits from SBFD operation for the relevant BS classes.

- From declaration perspective, the principle “it is expected that guardband size should be minimized to ensure the benefits from SBFD operation” is hard to be captured in RAN4 specification.

# 3. SBFD-aware UE related issues

### 3.1 Update for HPUE duty cycle evaluation

RAN4 has discussed the necessity of the specification update for HPUE duty cycle evaluation, and the following WF is provided in RAN4#114bis and no further agreement achieved in RAN4#115:

Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

Based on our understanding, the term “uplink symbols” used in TS38.101 is not a precise description, but it is intended to cover all symbols possible for UE transmission, i.e., including 'uplink' or 'flexible' symbols as defined in TS 38.211 even for Rel-15. After introducing SBFD feature, it is possible that SBFD-aware UE is able to perform UL transmission in 'uplink' or 'flexible' or “SBFD” symbols. It is also noted that the current SBFD WID does not contain TS38.101 series as the impacted specification. Considering it is the last RAN4 meeting for core requirement, we provide the following proposals to solve this issue.

Proposal 5: RAN4 recognize the followings as RAN4 common understanding (which can be captured as RAN4 agreement):

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

For existing UE RF requirement, RMC used for TDD in TS 38.101-1 are defined with the active UL slots specified in Table A.2.1-1 and TDD slot patterns as defined for reference sensitivity tests. In other words, from UE RF requirement perspective, only active uplink slots are considered for UE TX requirement.

Table A.2.1-1: TDD active uplink slots

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4, 9 |
| 30 kHz | 8, 9, 18, 19 |
| 60 kHz | 16, 17, 18, 19, 36, 37, 38, 39 |

Observation 2: The existing TDD UE TX RF requirement is specified only on active UL slots.

Because we expect the RMC will be not be changed for SBFD-aware UE, to further clarify “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Observation 3: Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Accordingly, we suggest the following proposal:

Proposal 6: For SBFD-aware UE, no RAN4 requirement impact is expected for UE specification series.

# 4. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters and SBFD-aware UE related issues, accordingly with the following observations and proposals obtained:

RAN4 aspects for SBFD system parameters

Proposal 1: For FR1 MR BS, use X = 38 dBm as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

- Note: As agreed, above this threshold, all configurations shall be tested; below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Proposal 2: For FR1 LA, FR1 MR with TRP or conductive power less than the pre-defined threshold, and all FR2 BS classes, the principle of “for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested” shall be interpreted as:

- For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations,

  - The configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction of maximum 3 subband configurations is applied or not:

- i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold.

Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not:

- i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold.

Observation 1: For the restriction on guardband size of SBFD,

- From testing perspective, as provided in Proposal 2, the configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested, therefore it is guaranteed that the guardband size should be minimized to ensure the benefits from SBFD operation for the relevant BS classes.

- From declaration perspective, the principle “it is expected that guardband size should be minimized to ensure the benefits from SBFD operation” is hard to be captured in RAN4 specification.

SBFD-aware UE related issues

Proposal 5: RAN4 recognize the followings as RAN4 common understanding (which can be captured as RAN4 agreement):

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

Observation 2: The existing TDD UE TX RF requirement is specified only on active UL slots.

Observation 3: Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

Proposal 6: For SBFD-aware UE, no RAN4 requirement impact is expected for UE specification series.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[8] R4-2416497, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#112bis.

[9] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[10] R4-2418097, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#113.

[11] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[12] R4-2500006 (R1-2410889), “LS on impact on transmit signal quality/MPR requirement”, Huawei, RAN1#119.

[13] R4-2501967, “Further discussion on general aspects for SBFD system”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.

[15] R4-2504218, “Further discussion on general aspects for SBFD system”, Samsung, RAN4#114bis.

[16] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis.

[17] R4-2507481, “Remaining issues for SBFD general aspects”, Samsung, RAN4#115.

[18] R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, RAN4#115.