3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511252

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Remaining issues for new requirements for SBFD operation for FR1 and FR2-1

Agenda item:			7.21.3.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111 to #114bis, further agreements are achieved on the potentially new requirements for SBFD operation [6] [8] [10] [12] [14] [16]. In this contribution, accordingly, we would like to further provide our analysis and viewpoints on the potentially new BS RF requirements for SBFD operation for FR1 and FR2-1.

# 2. Transient period btw. SBFD and non-SBFD operation

### 2.1 Text proposal related to transient period between SBFD and non-SBFD

In RAN4#115, the following agreement is achieved for the text proposal related to transient period between SBFD and non-SBFD:

Agreement: - Text proposal related to transient period requirement between SBFD and non-SBFD - Terminology “non-SBFD” need to be introduced. - Use below figure for the illustration of transient period between SBFD and non-SBFD in TS 38.104. Transmitter output power in UL subband/DL subband(s) Time Transmitter ON period (DL transmission) Transmitter OFF period Transmitter OFF period Transient period between SBFD and non-SBFD OFF power level ON power level UL transmission GP or UL transmission

Accordingly, the text proposal for non-SBFD operation is proposed:

Proposal 1: The following text proposal is adopted for clause 3.1 definitions:

- SBFD operation: SBFD-capable BS operation in non-SBFD symbols. - Non-SBFD operation: SBFD-capable BS operation in non-SBFD symbols.

And the text proposed is revised as follows to add the figure for the illustration of transient period between SBFD and non-SBFD in TS 38.104. It should be noted that y-axis title should be updated because the frequency regions for transition are “SBFD UL subband and guardband(s)” for Case-A (SBFD symbol/slot to DL symbol/slot) and Case-C (DL symbol/slot to SBFD symbol/slot). Therefore we suggest the following text proposal to make the figure more general and easier to comprehend.

Proposal 2: The y-axis title of the illustrative figure for transient period between SBFD and non-SBFD needs to be updated as “Transmitter output power in frequency regions for transition”

Accordingly, the text proposal is given as follows:

Proposal 3: The following text proposal is adopted for the clauses of transient period between SBFD and non-SBFD (take clause 12.2.8 for conducted TX requirement as example):

### 12.2.8 Transient period between SBFD and non-SBFD operation #### 12.2.8.1 General Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS. The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1. Transmitter output power frequency regions for transition Time Transmitter ON period (DL transmission) Transmitter OFF period Transmitter OFF period Transient period between SBFD and non-SBFD OFF power level ON power level UL transmission GP or UL transmission Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases: Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation Frequency regions for transition; BS transmitter behavior during transition. SBFD symbol/slot to DL symbol/slot; SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. SBFD symbol/slot to UL symbol/slot; SBFD DL subband; BS transmitter ON-to-OFF. DL symbol/slot to SBFD symbol/slot; SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. UL symbol/slot to SBFD symbol/slot; SBFD DL subband; BS transmitter OFF-to-ON. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. #### 12.2.8.2 Minimum requirement for BS type 1-H For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1. Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H Transition; Transient period length (µs). OFF to ON; 10. ON to OFF; 10.

# 3. In-channel adjacent subband requirements

### 3.1 In-channel adjacent subband leakage

In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, which is further confirmed to be an absolute PSD limit and the requirement is renamed as “in-channel adjacent subband leakage requirement” in RAN4#114 and 114bis:

Agreement (in RAN4#114): - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. Agreement (in RAN4#114bis): - The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”). Way forward (in RAN4#114bis): - No need to further discuss the necessity of in-channel adjacent subband leakage ratio, based on agreement from RAN4#114.

Furthermore, based on RAN4#115, the following agreement is achieved:

Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement

In the existing specification for a BS operating in non-contiguous spectrum, both ACLR and CACLR requirements are specified, while either ACLR or CACLR is applicable depending on the operating band and sub-block grap size (Wgap) where the limit applies. Particularly, the ACLR (CACLR) absolute basic limits or the ACLR (CACLR) limits, whichever is less stringent, shall apply.

Based on the study in Rel-18, it is demonstrated in our contribution that FR1 3.5 GHz testbed implementation which utilizes digital pre-distortion (DPD) techniques to improve upon the non-linearity characteristics of the PA can achieve 45 dBc isolation between the SBFD DL and UL subbands.

Issue 3-1-1: In-channel adjacent subband leakage absolute basic limit for SBFD DU or UD operation

For FR1 DU/UD case, by taking 80MHz (DL) – 20MHz (UL) as example, with 50dBm as BS TX power and 45dBc isolation assumed, the power density is around -14.03dBm/MHz. By considering the slow slop for adjacent channel leakage, the UL subband bandwidth to be measured could be higher than the average value -14.03dBm/MHz. From that perspective, we assume additional 3dB relaxation is necessary for the absolute basic limit, which is intended to be introduced for all SBFD configurations.

Proposal 4: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DU or UD operation shall be 3dB relaxation compared to corresponding ACLR limits, i.e.,

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DU or UD operation |
| --- | --- |
| Category A Wide Area BS | -10 dBm/MHz (3dB relaxation) |
| Category B Wide Area BS | -12 dBm/MHz (3dB relaxation) |
| Medium Range BS | -22 dBm/MHz (3dB relaxation) |
| Local Area BS | -32 dBm/MHz |

Issue 3-1-2: In-channel adjacent subband leakage absolute basic limit for SBFD DUD operation

For FR1 DUD case, we can take the typical configuration 40MHz (DL) – 20MHz (UL) – 40MHz (DL) as example, and assume 50dBm as the sum power for two DL subbands. By comparing the existing CACLR limit defined for non-contiguous spectrum, i.e., BWchannel = 40MHz and Wgap = 20MHz, the 45dBc CACLR limit shall be applied, which will be translated to -8dBm/MHz. Because the less stringent of CACLR absolute basic limits and the CACLR limits shall be applied, directly applying CACLR absolute basic limit will be too stringent for SBFD-capable BS. Therefor, we think 8dB relaxation is required for FR1 WA, while

Proposal 5: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DUD operation shall be 8dB relaxation compared to corresponding ACLR limits, i.e.,

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DUD operation |
| --- | --- |
| Category A Wide Area BS | -5 dBm/MHz (8dB relaxation) |
| Category B Wide Area BS | -7 dBm/MHz (8dB relaxation) |
| Medium Range BS | -17 dBm/MHz (8dB relaxation) |
| Local Area BS | -32 dBm/MHz |

### 3.2 In-channel adjacent subband blocking

In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity, and further agreement achieved in RAN4#115 as follows:

<Agreement in RAN4#113> - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

<Agreement in RAN4#115> - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

As follow-up, the following issues needs to be resolved, while the text proposal for in-channel adjacent subband blocking requirement shall be completed in this meeting.

Issue 3-2-1: Requirement applicable frequency range

The existing in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth, i.e., the interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. For in-channel adjacent subband blocking, since it is defined as in-channel requirement, it shall apply on the SBFD DL subband(s), more precisely within transmission bandwidth configuration for SBFD DL subband(s).

Proposal 6: In-channel adjacent subband blocking requirement shall apply within transmission bandwidth configuration for SBFD DL subband(s), i.e., NRB,SBFD,DL for DU/UD case, and both NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for DUD case.

Issue 3-2-2: Interfering signal minimum/maximum offset

In the existing in-band general blocking requirement, the interfering signal minimum offset is determined by the BS channel bandwidth of the lowest/highest carrier received and accordingly the interfering signal BW. For bands except n46, n96 and n102, the interfering signal centre frequency minimum offset is given as ±4.5MHz, ±7.5MHz, and ±30MHz as follows:

Table 7.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 3 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±4.5 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 5, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. For band n104, PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1a, 7.2.2-2c, and 7.2.2-3c. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

Similar as the existing in-band blocking requirement, we see the necessity of applying a similar interfering signal minimum offset. On the other side, the maximum offset is also needed to guarantee the whole bandwidth of interfering signal is within transmission bandwidth configuration for SBFD DL subband(s).

Proposal 7: For in-channel adjacent subband blocking requirement, the interfering signal centre frequency minimum and maximum offset shall be specified, to guarantee:

- The similar minimum offset as in-band general blocking requirement, depending on SBFD UL subband BW;

- The whole bandwidth of interfering signal is within transmission bandwidth configuration for SBFD DL subband.

Correspondingly, the interfering signal centre frequency offset is defined as:

Proposal 8: For in-channel adjacent subband blocking requirement,

- for SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband.

- for SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly.

Issue 3-2-3: Requirement Text Proposal

Based on the above analysis and proposal, the following text proposal is provided for in-channel adjacent subband blocking requirement:

Proposal 9: The following table is adopted for in-channel adjacent subband blocking requirement (taken 1-H as example).

Table 12.3.9.x-x: Base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency minimum offset (MHz) (Note 3) | Interfering signal centre frequency maximum offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±4.5 | ±(NRB,SBFD,DL *0.18 *2μ - 1.35) | 3 MHz OFDM NR signal 15 kHz SCS, 15 RBs |
|  |  |  | DUD | ±4.5 | -(NRB,SBFD,DL,1 *0.18 *2μ - 1.35) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 1.35) if interfering signal at NRB,SBFD,DL,2 |  |
| 5, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±7.5 | ±(NRB,SBFD,DL *0.18 *2μ – 2.25) | 5 MHz OFDM NR signal 15 kHz SCS, 25 RBs |
|  |  |  | DUD | ±7.5 | -(NRB,SBFD,DL,1 *0.18 *2μ – 2.25) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 2.25) if interfering signal at NRB,SBFD,DL,2 |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±30 | ±(NRB,SBFD,DL *0.18 *2μ - 9) | 20 MHz OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | ±30 | -(NRB,SBFD,DL,1 *0.18 *2μ – 9) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 9) if interfering signal at NRB,SBFD,DL,2 |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: For SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband. For SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly. |  |  |  |  |  |  |

# 4. Conclusion

In this contribution, we provided BS RF requirement for SBFD-capable BS, accordingly with the following observations and proposals obtained:

Transient period btw. SBFD and non-SBFD operation

Proposal 1: The following text proposal is adopted for clause 3.1 definitions:

- SBFD operation: SBFD-capable BS operation in non-SBFD symbols. - Non-SBFD operation: SBFD-capable BS operation in non-SBFD symbols.

Proposal 2: The y-axis title of the illustrative figure for transient period between SBFD and non-SBFD needs to be updated as “Transmitter output power in frequency regions for transition”

Proposal 3: The following text proposal is adopted for the clauses of transient period between SBFD and non-SBFD (take clause 12.2.8 for conducted TX requirement as example):

### 12.2.8 Transient period between SBFD and non-SBFD operation #### 12.2.8.1 General Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS. The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 12.2.8.1-1. Transmitter output power frequency regions for transition Time Transmitter ON period (DL transmission) Transmitter OFF period Transmitter OFF period Transient period between SBFD and non-SBFD OFF power level ON power level UL transmission GP or UL transmission Figure 12.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases: Table 12.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation Frequency regions for transition; BS transmitter behavior during transition. SBFD symbol/slot to DL symbol/slot; SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. SBFD symbol/slot to UL symbol/slot; SBFD DL subband; BS transmitter ON-to-OFF. DL symbol/slot to SBFD symbol/slot; SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. UL symbol/slot to SBFD symbol/slot; SBFD DL subband; BS transmitter OFF-to-ON. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. #### 12.2.8.2 Minimum requirement for BS type 1-H For BS type 1-H, the transmitter transient period between SBFD and non-SBFD operation shall be shorter than the values listed in the minimum requirement table 12.2.8.2-1. Table 12.2.8.2-1: Minimum requirement for the transmitter transient period between SBFD and non-SBFD operation for BS type 1-H Transition; Transient period length (µs). OFF to ON; 10. ON to OFF; 10.

In-channel adjacent subband requirements (leakage)

Proposal 4: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DU or UD operation shall be 3dB relaxation compared to corresponding ACLR limits, i.e.,

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DU or UD operation |
| --- | --- |
| Category A Wide Area BS | -10 dBm/MHz (3dB relaxation) |
| Category B Wide Area BS | -12 dBm/MHz (3dB relaxation) |
| Medium Range BS | -22 dBm/MHz (3dB relaxation) |
| Local Area BS | -32 dBm/MHz |

Proposal 5: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DUD operation shall be 8dB relaxation compared to corresponding ACLR limits, i.e.,

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DUD operation |
| --- | --- |
| Category A Wide Area BS | -5 dBm/MHz (8dB relaxation) |
| Category B Wide Area BS | -7 dBm/MHz (8dB relaxation) |
| Medium Range BS | -17 dBm/MHz (8dB relaxation) |
| Local Area BS | -32 dBm/MHz |

In-channel adjacent subband requirements (blocking)

Proposal 6: In-channel adjacent subband blocking requirement shall apply within transmission bandwidth configuration for SBFD DL subband(s), i.e., NRB,SBFD,DL for DU/UD case, and both NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for DUD case.

Proposal 7: For in-channel adjacent subband blocking requirement, the interfering signal centre frequency minimum and maximum offset shall be specified, to guarantee:

- The similar minimum offset as in-band general blocking requirement, depending on SBFD UL subband BW;

- The whole bandwidth of interfering signal is within transmission bandwidth configuration for SBFD DL subband.

Proposal 8: For in-channel adjacent subband blocking requirement,

- for SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband.

- for SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly.

- Proposal 9: The following table is adopted for in-channel adjacent subband blocking requirement (taken 1-H as example).

Table 12.3.9.x-x: Base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency minimum offset (MHz) (Note 3) | Interfering signal centre frequency maximum offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±4.5 | ±(NRB,SBFD,DL *0.18 *2μ - 1.35) | 3 MHz OFDM NR signal 15 kHz SCS, 15 RBs |
|  |  |  | DUD | ±4.5 | -(NRB,SBFD,DL,1 *0.18 *2μ - 1.35) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 1.35) if interfering signal at NRB,SBFD,DL,2 |  |
| 5, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±7.5 | ±(NRB,SBFD,DL *0.18 *2μ – 2.25) | 5 MHz OFDM NR signal 15 kHz SCS, 25 RBs |
|  |  |  | DUD | ±7.5 | -(NRB,SBFD,DL,1 *0.18 *2μ – 2.25) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 2.25) if interfering signal at NRB,SBFD,DL,2 |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±30 | ±(NRB,SBFD,DL *0.18 *2μ - 9) | 20 MHz OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | ±30 | -(NRB,SBFD,DL,1 *0.18 *2μ – 9) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 9) if interfering signal at NRB,SBFD,DL,2 |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: For SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband. For SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly. |  |  |  |  |  |  |

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis

[5] R4-2409765, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411640, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112.

[8] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[9] R4-2416498, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112bis.

[10] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[11] R4-2418098, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#113.

[12] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

[13] R4-2501968, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#114.

[14] R4-2502310, “Way Forward for [114][306] NR_duplex_evo_General”, Samsung, RAN4#114.

[15] R4-2504219, “On the new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#114bis.

[16] R4-2504751, “Way Forward for [114bis][306] NR_duplex_evo_General”, Samsung, RAN4#114bis.

[17] R4-2507483, “On the new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#115.

[18] R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, RAN4#115.