3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511253

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Remaining issues on modification of existing TX requirements for SBFD-capable BS

Agenda item:			7.21.3.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the follow-up RAN4 meetings, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10] [12] [14] [16].

In RAN4#115, the TX requirements for SBFD-capable BS has been endorsed in the big dCR [R4-2507714] and its contained individual endorsed dCRs. In this contribution, accordingly, we would continue to provide our text proposal related to the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing TX requirement for SBFD

## 2.1 Base Station output power and radiated transmit power

Based on the endorsed dCR [R4-2508712], we propose the following changes to clauses 12.2.2, 12.5.2 and 12.5.3 for Base station output power and radiated transmit power for SBFD.

Proposal 1: The following text proposals by including “non-SBFD symbols” and other corrections are adopted for clauses 12.2.2, 12.5.2 and 12.5.3 for Base station output power and radiated transmit power for SBFD:

=================== Start of Text Proposal ===================

### 12.2.2     Base station output power for SBFD

#### 12.2.2.1	General

The SBFD-capable BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power over SBFD symbols may be different from the rated carrier output powers over non-SBFD symbols as specified in clause 6.2.

#### 12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to SBFD-capable BS output power requirement for SBFD operation over SBFD symbols.

### 12.5.2     Radiated transmit power for SBFD

#### 12.5.2.1	General

The SBFD-capable BS radiated transmit power for SBFD operation is specified over SBFD symbols, in which the EIRP over SBFD symbols may be different from the EIRP over non-SBFD symbols as specified in clause 9.2.

#### 12.5.2.2	Minimum requirement for BS type 1-H and BS type 1-O

The same requirement as clause 9.2.2 is applicable to SBFD-capable BS radiated transmit power requirement for SBFD operation over SBFD symbols.

#### 12.5.2.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.2.3 is applicable to SBFD-capable BS radiated transmit power requirement for SBFD operation over SBFD symbols.

### 12.5.3     OTA base station output power for SBFD

#### 12.5.3.1	General

The OTA base station output power for SBFD operation is specified over SBFD symbols, in which the TRP for SBFD symbols may be different from the TRP over non-SBFD symbols as specified in clause 9.3.

#### 12.5.3.2	Minimum requirement for BS type 1-O

The same requirement as clause 9.3.2 is applicable to OTA base station output power requirement for SBFD operation over SBFD symbols.

#### 12.5.3.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.3.3 is applicable to OTA base station output power requirement for SBFD operation over SBFD symbols.

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we provided the text proposal related to the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS:

Proposal 1: The following text proposals by including “non-SBFD symbols” and other corrections are adopted for clauses 12.2.2, 12.5.2 and 12.5.3 for Base station output power and radiated transmit power for SBFD:

=================== Start of Text Proposal ===================

### 12.2.2     Base station output power for SBFD

#### 12.2.2.1	General

The SBFD-capable BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power over SBFD symbols may be different from the rated carrier output powers over non-SBFD symbols as specified in clause 6.2.

#### 12.2.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to SBFD-capable BS output power requirement for SBFD operation over SBFD symbols.

### 12.5.2     Radiated transmit power for SBFD

#### 12.5.2.1	General

The SBFD-capable BS radiated transmit power for SBFD operation is specified over SBFD symbols, in which the EIRP over SBFD symbols may be different from the EIRP over non-SBFD symbols as specified in clause 9.2.

#### 12.5.2.2	Minimum requirement for BS type 1-H and BS type 1-O

The same requirement as clause 9.2.2 is applicable to SBFD-capable BS radiated transmit power requirement for SBFD operation over SBFD symbols.

#### 12.5.2.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.2.3 is applicable to SBFD-capable BS radiated transmit power requirement for SBFD operation over SBFD symbols.

### 12.5.3     OTA base station output power for SBFD

#### 12.5.3.1	General

The OTA base station output power for SBFD operation is specified over SBFD symbols, in which the TRP for SBFD symbols may be different from the TRP over non-SBFD symbols as specified in clause 9.3.

#### 12.5.3.2	Minimum requirement for BS type 1-O

The same requirement as clause 9.3.2 is applicable to OTA base station output power requirement for SBFD operation over SBFD symbols.

#### 12.5.3.3	Minimum requirement for BS type 2-O

The same requirement as clause 9.3.3 is applicable to OTA base station output power requirement for SBFD operation over SBFD symbols.

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411638, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416499, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.

[11] R4-2418099, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#113.

[12] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113.

[13] R4-2501969, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#114.

[14] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114.

[15] R4-2504220, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#114bis.

[16] R4-2504752, “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114bis.

[17] R4-2507485, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#115.

[18] R4-2508713, “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, RAN4#115.