3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511254

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Remaining issues on modification of existing RX requirements for SBFD-capable BS

Agenda item:			7.21.3.3

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the follow-up RAN4 meetings, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10] [12] [14] [16] [18].

In this contribution, accordingly, we would like to provide our analysis and viewpoints on remaining issues related to modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS. In RAN4#115, the RX requirements for SBFD-capable BS has been endorsed in the big dCR [R4-2507714] and its contained individual endorsed dCRs. In this contribution, accordingly, we would continue to provide our text proposal related to the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing RX requirement for SBFD

### 2.1 ACS requirement

For the requirement of in-band selectivity (more specifically, adjacent channel selectivity (ACS)) and in-band blocking, the following agreements is achieved in the study phase [2]:

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

In RAN4#114bis, there is still deadlock for this topic, with at least the following four options on the table:

- FFS on this issue - Option 1: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Option 2: Prioritize discussing the scenarios under non-colocation case first. - The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Option 3: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Option 4: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Derive ACS requirement level based on the simulation results using 5% GS in SI. - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS

In RAN4#115, the following WF is approved:

- Way forward - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

Based on our understanding, for ACS requirement, by adopting the 9dB offset to the in-band blocking interference level is reasonable proposal, while the frequency configuration of the NR interference signal shall be reused. Furthermore, it should be noted that the offset shall be 12dB for band n104.

Proposal 1: In general, revised Option 1 shall be adopted:

- Option 1a: The requirement of ACS shall be applicable to Rel-19 SBFD-capable BS.

  - the interference level keeps 12dB offset for n104 and 9 dB offset for other bands to the general in-band blocking interference level defined for SBFD

  - no change on the ACS interferer frequency offset values specified in Table 7.4.1.2-2 for conducted requirement and Table 10.5.1.2-2 for OTA requirement.

### 2.2 In-band blocking requirement for FR1 WA BS

In RAN4#115, the following WF is approved for in-band blocking requirement for FR1 WA, which is captured in [18]:

## Issue 2-2: In-band blocking for FR1 WA BS - Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered

For the level of in-band blocking requirement, based on our co-existence simulation results, the adjacent channel gNB DL power level @99 percentile is -27.76dBm:

Furthermore, it is also identified that the percentage value of grid shifting plays an important role to determine the distributions of adjacent-channel and co-channel gNB DL received powers, since the increased percentage grid shifting leads to higher pathloss between the interfering gNB and victim gNB directly, which can be translated to the requirement assumptions as deployment guidance as provided in the WF.

Accordingly, we propose the following compromised solution can be adopted:

Proposal 2: For in-band blocking FR1 WA BS, the requirement and corresponding assumptions are adopted together:

- Adopt the value -28dBm, which is taken from co-existence simulation with 100% grid shift.

- The in-band blocking requirements assume a [90] dB coupling loss between interfering NR BS transmitter and NR SBFD BS receiver.

### 2.3 In-band blocking requirement for FR2-1 WA BS

For in-band blocking requirement, in on our analysis given in [R4-2411639], the levels of adjacent channel gNB DL power level for different scenario have been provided. It can be observed that at least for Scenario 6 (FR2 UMa-UMa), 99% of the levels of adjacent channel gNB DL power is smaller than the existing in-band blocking requirement. Therefore, we suggest the group to conclude with the following proposal firstly:

Proposal 3: For FR2 WA BS capable of SBFD operation, the existing in-band blocking requirement shall be reused.

### 2.4 In-band blocking requirement definition

For the in-band blocking requirement, the interference source should be the interference from BS operating at adjacent channel, therefore the type of interfering signal should be OFDM NR signals. Specifically, we suggest to follow the following existing in-band blocking requirement, as below table. For interference signal frequency location, the existing offset can still be followed.

Proposal 4: The type of interfering signal and corresponding CBW can be defined as follows, i.e, by following existing interference signal CBW and minimum offset, while using OFDM NR signal instead.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±4.5 | 3 MHz OFDM NR signal 15 kHz SCS, 15 RBs |
| 5, 10, 15, 20 | ±7.5 | 5 MHz OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | ±30 | 20 MHz OFDM NR signal 15 kHz SCS, 106 RBs |

### 2.5 In-channel selectivity

For in-channel selectivity requirement, the following agreements is achieved in the study phase [2]:

Regarding the receiver in-channel selectivity requirement, the requirement shall be studied based on that the wanted signal and UL interfering signal shall be located in the configured UL subband, and the wanted signal and interfering signal levels could be further studied in the WI phase.

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. It should be noted that ICS requirement is introduced to consider the interference at the receiving bandwidth. After SBFD feature is introduced, the receiver functionality and performance is guaranteed by the OTA sensitivity performance in which the subband DL self-interference is present and by in-band blocking requirement in which the subband DL interference from neighboring cells are present. Therefore we see no needs to consider SBFD DL subband in the in-channel selectivity testing.

Proposal 5: The existing in-channel selectivity (ICS) requirement shall be applicable to SBFD-capable BS, while the receiver functionality/performance is guaranteed by other requirements/tests for the presence of interference from DL subband.

# 3. General CR drafting issue

### 3.1 One guidance for CR drafting

In RAN4#115, the following WF is agreed to check the terminology for “SBFD slot”, “SBFD symbol”, “SBFD symbol/slot” or “SBFD period” to be used in specification drafting:

- Way forward - Check and align the terminology for “SBFD slot”, “SBFD symbol”, “SBFD symbol/slot” or “SBFD period” to be used in specification drafting. - The decided terminology should be clarified in clause 3.

In both approved RAN1 CRs to TS38.213 and 38.214 [R1-2504970, R1-2504993], the terms “SBFD symbol” and “non-SBFD symbol” are used, therefore we suggest RAN4 specification to be drafted accordingly.

Proposal 6: The terms “SBFD symbol” and “non-SBFD symbol” shall be used in RAN4 specification for SBFD-capable BS.

# 4. Conclusion

In this contribution, we provided the viewpoints on the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Modification on Existing RX requirement for SBFD

Proposal 1: In general, revised Option 1 shall be adopted:

- Option 1a: The requirement of ACS shall be applicable to Rel-19 SBFD-capable BS.

  - the interference level keeps 12dB offset for n104 and 9 dB offset for other bands to the general in-band blocking interference level defined for SBFD

  - no change on the ACS interferer frequency offset values specified in Table 7.4.1.2-2 for conducted requirement and Table 10.5.1.2-2 for OTA requirement.

Proposal 2: For in-band blocking FR1 WA BS, the requirement and corresponding assumptions are adopted together:

- Adopt the value -28dBm, which is taken from co-existence simulation with 100% grid shift.

- The in-band blocking requirements assume a [90] dB coupling loss between interfering NR BS transmitter and NR SBFD BS receiver.

Proposal 3: For FR2 WA BS capable of SBFD operation, the existing in-band blocking requirement shall be reused.

Proposal 4: The type of interfering signal and corresponding CBW can be defined as follows, i.e, by following existing interference signal CBW and minimum offset, while using OFDM NR signal instead.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 3 | ±4.5 | 3 MHz OFDM NR signal 15 kHz SCS, 15 RBs |
| 5, 10, 15, 20 | ±7.5 | 5 MHz OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | ±30 | 20 MHz OFDM NR signal 15 kHz SCS, 106 RBs |

Proposal 5: The existing in-channel selectivity (ICS) requirement shall be applicable to SBFD-capable BS, while the receiver functionality/performance is guaranteed by other requirements/tests for the presence of interference from DL subband.

General CR drafting issue

Proposal 6: The terms “SBFD symbol” and “non-SBFD symbol” shall be used in RAN4 specification for SBFD-capable BS.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411639, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416500, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.

[11] R4-2418100, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#113.

[12] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113.

[13] R4-2501970, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#114.

[14] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114.

[15] R4-2504221, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#114bis.

[16] R4-2504752, “Way Forward for [114bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114bis.

[17] R4-2507486, “On the modification of existing RX requirements for SBFD-capable BS”, Samsung, RAN4#115.

[18] R4-2508713, “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, RAN4#115.