3GPP TSG-RAN WG4 Meeting #116	R4-2511261

# Bangaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5984 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RRM performance requirements of NR channel BW less than 5MHz for FR1 phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correction on test cases to appropriately point RMC SR.1.2 and SR.1.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on test cases to appropriately point RMC SR.1.2 and SR.1.3 in test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3.17, A.6.5.11.2, A.6.6.1.15, A.6.6.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

#### A.6.5.3.17	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle with less than 5MHz BW

##### A.6.5.3.17.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation, and the SCell operate with a bandwidth of less than 5 MHz. Supported test configurations are specified in Table A.6.5.3.17.1-1 below. General test parameters for known Scell activation case are specified in Table A.6.5.3.1.1-2. Supported Cell specific test parameters for NR PCell specified in Table A.6.5.3.1.1-3. Supported Cell specific test parameters for NR SCell specified in Table A.6.5.3.1.1-4 apply except those specified in Table A.6.5.3.17.1-2.

The test procedure specified in A.6.5.3.1 applies to this test.

Table A.6.5.3.17.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Table A.6.5.3.17.1-2: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| BWchannel | ConfigSCell 1 | MHz | 3 MHz |  |  |
| BWoccupied | ConfigSCell 1 | RB | 12 |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.3 FDD |  |  |
| SSB Configuration | ConfigSCell 1 |  | SSB.13 FR1 |  |  |
| Io Note3 | ConfigSCell 1 | dBm/ 2.7 MHz | -60.31 | -58.58 | -58.58 |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themeselves. |  |  |  |  |  |

### <<End of Change1>>

### <<Start of Change2>>

#### A.6.5.11.2	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz

##### A.6.5.11.2.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delay under NR-DC with a bandwidth of less than 5 MHz meets the requirements defined in Clauses 8.9.2 and 8.9.3, where the PSCell is unknown to the UE at the time of addition.

Supported test configurations are shown in Table A.6.5.11.2.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 and radio channel 2 in FR1, respectively. Test parameters are given in Table A.6.5.11.1.1-2. The cell-specific parameters as specified in Table A.6.5.11.1.1-3 with config 1 apply except those specified in Table A.6.5.11.2.1-2 .

The test procedure specified in A.6.5.11.1 applies to this test.

Table A.6.5.11.2.1-1: Supported test configurations for FR1 PSCell Addition and Release with less than 5 MHz

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 3 MHz (12 PRB), FDD |

Table A.6.5.11.2.1-2: Cell Specific Parameters for FR1 PSCell Addition and Release with less than 5 MHz

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 | T4 |
| BWchannel | MHz | 1 |  | 3: NPRB,c = 12 |  |  |  |  |
| Data PRBs allocated |  | 1 |  | 12 |  |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 |  | SR.1.3 FDD |  |  |  |  |
| RMSI CORESET parameters |  | 1 |  | CR.1.2 FDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 |  | CCR.1.6 FDD |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 |  | 15 |  |  |  |  |
| SSB configuration |  | 1 |  | SSB.13 FR1 |  |  |  |  |
| IoNote | dBm/2.7MHz | 1 |  | N/A |  | –63.4 dBm |  |  |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

### <<End of Change2>>

### <<Start of Change3>>

##### A.6.6.1.15.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on Carrier 1, FR1 Neighbouring cell (Cell 2) and a FR1 SCell (Cell 3) on Carrier 2. Carrier 2 is on a different frequency than the PCell.

The test parameters for PCell and neighbour cell are given in table A.6.6.1.15.2-1, A.6.6.1.15.2-2 and A.6.6.1.15.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and the SCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.15.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1 with 12 PRB SSB for SCell

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| PDSCH RMC configuration |  |  | SR.1.1 FDD |  | SR.1.3 FDD |  |  |  |
| RMSI CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | CCR.1.1 FDD |  | CCR.1.7 FDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 FDD |  |  |  |
| IInitial BWP configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |
| Active DL BWP configuration |  |  | DLBWP.1.1 |  |  |  |  |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |  |  |  |  |
| RLM-RS |  |  | SSB |  |  |  |  |  |
| Note 2 | dBm/SCS |  | -98 |  |  |  |  |  |
|  |  |  | -98 |  |  |  |  |  |
| Note 2 | dBm/15 kHz |  | -98 |  |  |  |  |  |
|  | dB |  | 4 | -1.46 | -Infinity | -1.46 |  |  |
|  |  |  |  | s |  |  |  |  |
|  | dB |  | 4 | 4 | -Infinity | 4 |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz |  | -94 | -94 | -Infinity | -94 | -94 | -94 |
|  |  |  | -94 | -94 | -Infinity | -94 | -94 | -94 |
| Io | dBm/9.36 MHz |  | -64.60 | -62.25 | --64.60 | -62.25 |  |  |
|  | dBm/9.36 MHz |  | -64.60 | -62.25 | --64.60 | -62.25 |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  |
|  | NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.6.1.15.3	Test requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1120 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### <<End of Change3>>