3GPP TSG-RAN WG4 Meeting #116                                                     R4- 2511270

Bengaluru, India, August 25th – 29th, 2025

Source:		Nokia

Title:	IoT-NTN HPUE RF requirements for Tx

Agenda item:		7.6.3.3.1

Document for:	Discussion

# Introduction

Based on the agreement in the WF from RAN4 #115 meeting [1], in this paper, we’d like to present our view in IoT NTN HPUE SAR related issues.

# SAR Related issues ### SAR Related issues Issue 2-1-1: Whether and how to define SAR scheme other than already agreed P-MPR (e.g. duty cycle related scheme): - Background (for information): - P-MPR was already agreed to be the baseline SAR solution for all NTN HPUE cases. - The current status for duty-cycle based SAR scheme in TN - NR: - Defined in MOP section; - Applied in ΔPPowerClass in configured transmitted power clause; - whether can apply to FDD band has some different understanding. - eMTC: - Not defined in MOP section; - Reference general LTE configurated transmitted power clause, no separate definition - For general LTE, the duty cycle related power backoff explicitly defined only for TDD band whose frame configuration is 0 or 6 - NB-Iot: - Not defined in MOP section; - Not defined in configured transmitted power clause which use dedicated equations (Not reference LTE general equation) - Options for Selection: - Option 1: Keep the same status as TN for respective base RAT - NR-NTN: Reuse NR TN, - Including ΔPPowerClass in configured transmitted power clause - eMTC based Iot-NTN: Reuse eMTC - Not precluding ΔPPowerClass in configurated transmitted power clause - NB-Iot based Iot-NTN: Reusing NB-Iot - No duty cycle related scheme defined - Option 2: Introduce further revision / new method - NR-NTN sub-options: - A. Introduce clarification for FDD band support; (Nokia, Qualcomm) - B. changing the ΔPPowerClass from mandatory to optional;(Qualcomm) - C. UE voluntarily dropping uplink transmissions by self-assessment to control the SAR (vivo) - D. differentiate UE working status in terms of whether SAR is required in a time duration. (OPPO) - E. To have the network to notify/signal its upper bound of uplink ratio for a certain time duration (Samsung) - F. Energy headroom reporting (Qualcomm) - G. ΔPPowerClass and P-MPR reporting (Ericsson, SONY, Nokia) - H. Send information LS to RAN2 support PC1 NTN HPUE for FDD use cases. (Nokia) - I. inform the RAN that it is close to the SAR limit (ViaSet, Inmarsat) - Iot-NTN (including eMTC & NB-Iot based): - Introduce NR like scheme - Introduce other revision / new method as proposed for NR-NTN - Option3: Do not introduce for NTN - NR NTN : Explicitly preclude ΔPPowerClass and no other duty cycle related reference - eMTC-based Iot-NTN: Explicitly preclude ΔPPowerClass and no other duty cycle related reference - NB-Iot based Iot-NTN: No duty cycle or other new scheme defined

# Discussion

  - SAR related issue

There has been discussion and different options of solving SAR issues by duty cycle. Our view is that combining P-MPR and legacy duty cycle can be an optimal way to solve this issue as in previously submitted Tdoc [2]. Therefore, we suggest introducing duty cycle in NTN related high power devices.

As in the WF, the duty cycle is not only a solution for NTN HPUE, but also a solution for SAR issues in TN NB IoT and TN NR use cases. Since it is now a common understanding and there is no restrict to TDD or FDD to support duty cycle for NTN/TN by the “maxUplinkDutyCycle-x" capabilities in the current 38.306, it is RAN4 consensus if the duty cycle can be introduced and benefit different type of user devices.

In the current standard, there is no duty cycle capability defined for PC1 in the current TS 38.306. So, a new release independent capability to support PC1 can be created and inform RAN2.

Send information LS to RAN2 for the definition of duty cycle to support PC1 NTN HPUE for FDD use cases.

Approve the LS in the attachment

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

- Send information LS to RAN2 for the definition of duty cycle to support PC1 NTN HPUE for FDD use cases.

- Approve the LS in the attachment

# References

- R4-2508122, WF_Iot_NTN_HPUE Part2

- R4-2507092, IoT-NTN HPUE RF requirements for Tx

# Annex (LS to RAN2)

3GPP TSG-RAN WG4 Meeting #116                                                        R4-25xxxx

Bengaluru, IN, 25th – 29th Aug 2025, 2025

Title:	LS on duty cycle configuration for NTN HPUE

Response to:	-

Release:	Rel-19

Work Item:	NR_IoT_NTN_req_test_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Yi Tan

yi.1.tan@nokia.com

# 1	Overall description

The SAR issue is common for high-power handheld devices, and duty cycle is an effective method to control SAR issues.

In the current TS 38.306, there is no “maxUplinkDutyCycle-x" capabilities to address the duty cycle capability for NTN PC1 UEs. Therefore, RAN4 would like to create a release independent capability “maxUplinkDutyCycle-PC1” to indicate UE’s maximum tolerated percentage of scheduled uplink symbols to ensure the SAR compliance in NTN for FDD configurations, for rel-19 and earlier releases.

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN2 to consider the above information.

# 3	Dates of next TSG RAN WG 4 meetings

3GPPRAN4#116-bis		        	13 – 17 Oct. 2025					Prague, CZ

3GPPRAN4#117		        		17 – 21 Nov. 2025				Dallas, US