3GPP TSG-RAN WG4 Meeting #116                                                      R4-2511272

# Bengaluru, IN, 25th – 29th August, 2025

Source:		Nokia

Title:	Reply LS R1-2504839 on maximum transmission power for STxMP

Agenda item:		9.3

Document for:	Discussion

# Introduction

Based on the LS TCI state k=0,1 or k=1,2 from RAN1 to RAN4 in RAN4 #115 meeting [1], we’d like to present our view for related issues.

1. Overall Description: In 38.101-2 (Table 1), the maximum transmission power PCMAX,f,c,k for STxMP is defined, with k=0 for the first indicated TCI state and k=1 for the second indicated TCI state. While, according to 38.213 (Table 2), PCMAX,f,c,k is applied for the k-th TCI state, which implies k=1 for the first indicated TCI state and k=2 for the second indicated TCI state. In addition, in 38.321 (Table 3), it is also assumed k=1 for the first indicated TCI state and k=2 for the second indicated TCI state. In other words, The description of PCMAX,f,c,k in RAN1 and RAN2 specifications are aligned, but not aligned with the description of PCMAX,f,c,k in RAN4 specification. Table 1. Definition of maximum transmission power for STxMP in 38.101-2. ### 6.2K.4 Configured transmitted power for simultaneous transmission to multiple directions A UE configured for simultaneous transmission to multiple directions can configure two maximum output powers. The configured UE maximum output power PCMAX,f,c,k for each of joint/UL TCI states k (k=0,1) indicated for simultaneous transmission to multiple directions of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax. Table 2. Description of maximum transmission power for STxMP in 38.213 ### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l - if the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is configured with multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel, and the UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in PUSCH transmission occasion i, the UE determines the PUSCH transmission power PPUSCH,b,f,c, k(i,j,qd,l) for the k-th indicated TCI-State or TCI-UL-State as PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] - else, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] where, - PCMAX,f,c,k(i) is the UE configured maximum output power for the k-th indicated TCI-State or TCI-UL-State defined in [8-2, TS 38.101-2] for carrier f of serving cell c in PUSCH transmission occasion i . - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5] for carrier f of serving cell c in PUSCH transmission occasion i.. Table 3. Description of maximum transmission power for STxMP in 38.321 #### 6.1.3.81 Enhanced Single Entry PHR for multiple TRP STx2P MAC CE The Enhanced Single Entry PHR for multiple TRP STx2P MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2b. The two PHs together with two PCMAX,f,c,k for the Serving Cell are reported if UE is configured with twoPHRMode and multipanelSchemeSDM or multipanelSchemeSFN, as specified in clause 5.4.6. It has a fixed size and consists of four octets defined as follows (Figure 6.1.3.81-1): - R: Reserved bit, set to 0; - Power Headroom k (PH k): This field indicates the power headroom level for k = 1, 2, where PH 1 is associated with the first TCI-State or TCI-UL-State for a real or reference PUSCH transmission and PH 2 is associated with the second TCI-State or TCI-UL-State for a real or reference PUSCH transmission, as specified in TS 38.213 clause 7.7.1 [6]. PH fields for a Serving Cell are included in ascending order based on k. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB are specified in TS 38.133 [11]);. RAN1 has discussed the above issue. Instead of changing both RAN1 and RAN2 specification, RAN1 thinks it may be simper to update RAN4 spec, so that RAN1, RAN2 and RAN4 specifications are aligned. 2. Actions: To RAN2 ACTION: RAN1 respectfully suggests RAN4 to change the value range of k in PCMAX,f,c,k to {1, 2}.

# Discussion

Regarding the definition of k in PCmax,f,c,k as {0, 1} or {1, 2} mentioned in the LS, this was a new definition in RAN4 for STxMP scenarios. It looks like RAN1 and RAN4 had not been synchronized in advance in the past. It is valid consideration from RAN1 that the definition of k should be the same in RAN1 and RAN4 standards.

From our point of view, there is no conflict observed in RAN4 standard changing the k definition from {0, 1} to {1, 2}. Therefore, it is possible to raise a CR to make changes.

Observation 1: The STxMP scenario is new for both RAN1 and RAN4 for the definition of k in PCmax equation, it looks like RAN1 and RAN4 had not been synchronized in advance in the past.

RAN4 can raise a CR to change the k definition from {0, 1} to {1, 2}, if there is no conflict in RAN4 standard based on RAN4 consensus.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: The STxMP scenario is new for both RAN1 and RAN4, for the definition of k in PCcmax, it looks like RAN1 and RAN4 had not been synchronized in advance in the past.

- RAN4 can raise a CR to change the k definition from {0, 1} to {1, 2}, if there is no conflict in RAN4 standard based on RAN4 consensus.

# References

- R1-2504839, TCI state k=0,1 or k=1,2