3GPP TSG-RAN WG4 Meeting #116	R4-2511275

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Performance parameters for Less than 5 MHz test cases in NTN

Agenda item:	7.6.4.5

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Throughout the first meetings in this WI good progress was achieved by reusing most of the requirements from the less than 5 MHz WI for terrestrial networks with the necessary adaptation for NTN. The current paper aims at discussing the performance requirements for less than 5 MHz operation.

In the present paper we provide our views on the issues open for discussion related to the performance requirements for the operation in less than 5 MHz in NTN.

Discussion

One of the open issues presented in the WF of the previous meeting [3] regards the editorialization of the specification. Companies are discussing the best manner to capture the test cases to be introduced for cell reselection in this WI for the less than 5 MHz operation in NTN. There are currently two options being considered:

### Issue 2-2-2: Test cases to verify the core requirements reusing NTN Agreement: The new test cases of cell reselection for NTN less than 5MHz can be defined. And the detailed testing configuration and requirements (e.g. TSI-NR) can be further discussed: - Option 1: captured in the new sub clause in NTN clause - Option 2: an additional test configuration in the exsiting sub clause for NTN

It is a common and straightforward understanding that the test cases for less than 5 MHz operation shall be applicable only for UEs that support the features. In this case, both options achieve the main goal of differentiating the test configurations. However, in order to maintain the current structure and harmony of the specification, it is preferred that the same solution is applied to all test cases for less than 5 MHz and, if possible, also re-using the same principles for TN and NTN testing. Hence, based on such specification principles, we prefer that the test cases for cell reselection operation with less than 5 MHz, are presented in a different subclause, as all other tests in this WI.

Additionally, this approach has also the advantage of making it easier for the reader to identify the list of test cases applicable for less than 5MHz operation in NTN. Therefore, we propose:

Captured the cell reselection test cases for less than 5 MHz in a new sub-clause in NTN clause.

Another open issue related to the test cases to be specified relates to the parametrization of the test cases and coverage of different scenarios. The test cases shall be developed such that the largest number of scenarios is covered for testing. On the other hand, this WI has only introduced operation for less than 5MHz, it is then, assumed that the legacy tests (for regular BW) covers most of the scenarios to be discussed.

Some of the test cases left for further discussion in issue 1.1.3 in [3] are:

- For CHO, whether the test cases are to be performed with or without L3 measurements.

- For satellite switch with resynchronization: whether RACH based configuration is to be tested.

- For RLM tests (RLM-1 and RLM-2) the UE is to be tested under DRX or non-DRX conditions.

- For intra and inter-frequency requirements whether measurement gap is needed and whether the UE is to be tested under DRX or non-DRX conditions.

In this document we discuss the test configurations above.

- For CHO, the goal of the test case, is to verify that the UE can complete the measurements using reduced bandwidth in due time. The CHO without L3 measurement is a feature that depends on an independent capability from Release 18. It is possible that a Rel-19 UE that supports less than 5 MHz operation does not support this capability, and the test will not be performed. On the other hand, if the UE does support this feature, the execution of a CHO without L3 measurement will already be performed as part of the legacy tests. Therefore, intuitively, it is more advantageous if the test case for the less than 5 MHz operation is performed with L3 measurements. Therefore:

For CHO test cases, prioritize the case with L3 measurements.

- For the satellite switch with resynchronization, there is further discussion to be held whether the test case will be RACH-less or RACH based. The feature was introduced to allow for smooth transition between satellites under high mobility and the RACH-less functionality is the most intended functionality.

For satellite switching with resynchronization adopt the RACH-less configuration to be tested.

- There are two RLM tests (one for in-sync and another for out-of-sync). For the sake of coverage, and to simplify the agreements, it is possible to adopt both configuration (DRX and non-DRX) by attributing one to each of the test cases.

For RLM test cases, use DRX mode in the out-of-sync test case and use non-DRX mode for the in-sync test case.

- For intra and inter-frequency measurements, it is important to decide whether measurement gaps are needed and whether the UE is to be measured in DRX or non-DRX conditions. From our point of view, it is important that at least one of the cases is tested with measurement gap. It is more straightforward if the measurement gap is configured in the inter-frequency test case. For coverage it is also important to verify that the UE is capable of completing the measurements with less than 5 MHz SSB in inter-satellite measurements. As, depending on UE capabilities, measurement caps are required for inter-satellite measurements, we propose that the inter-frequency measurements test case shall be performed with inter-satellite configuration.

For intra-frequency measurements, use the following configuration parameters: non-DRX, no measurement gap, intra-satellite scenario.

For inter-frequency measurements, use the following configuration parameters: DRX, measurement gap configured, inter-satellite scenario.

Conclusion

In this paper, we provided our views on the performance test cases of Less than 5 MHz operation in NTN by means of the following proposals:

Proposal 1: Captured the cell reselection test cases for less than 5 MHz in a new sub-clause in NTN clause.

Proposal 2: For CHO test cases, prioritize the case with L3 measurements.

Proposal 3: For satellite switching with resynchronization adopt the RACH-less configuration to be tested.

Proposal 4: For RLM test cases, use DRX mode in the out-of-sync test case and use non-DRX mode for the in-sync test case.

Proposal 5: For intra-frequency measurements, use the following configuration parameters: non-DRX, no measurement gap, intra-satellite scenario.

Proposal 6: For inter-frequency measurements, use the following configuration parameters: DRX, measurement gap configured, inter-satellite scenario.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- RP-241985, " Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105

- R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi), Malta, MT, 19th – 23rd May, 2025