3GPP TSG-RAN WG4 Meeting #116	R4-2511277

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Requirements for NTN UEs on Ku band operation

Agenda item:	7.7.7

Document for:	Discussion

Introduction

In the last RAN4 meeting, the discussions for the implementation of RRM requirements related to Ku Band have started. The initial discussions, open issues and agreement were captured under the WF [1]. Most of the problems described therein are related to the RRM requirements that need to be defined/updated for VSAT type UEs and how to create requirements for FR1-NTN for VSAT Type UES.

In the present document we provide our views on the following list of open issues on the previous WF:

- Issue 1-2: Assumption for defining RRM requirements in Ku-band NTN

- Issue 1-3: satellite switch requirements

- Issue 1-5: inter-satellite cell reselection requirements (section 4.2C).

- Issue 1-8: RLM (section 8.1C)

Discussion

In the present paper we will discuss some of the open issues presented in the previous WF [1].

Assumption for defining RRM requirements in Ku-band NTN

In our point of view, issues 1-2 , 1-3 and 1-13 on the WF are intertwined and impact each other. We would like to discuss both of them combined.

Issue 1-2: Assumption for defining RRM requirements in Ku-band NTN Candidate options: - Option 1: (LGE) - The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies; - Single SAN Tx beam per radio cell in DL - For inter satellite HO, only blind HO and no neighbor cell measurement - Intra satellite neighbor cell measurement is needed - Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping) - Option 2: other options are not precluded.

The discussion above reflects the overall scenario to be considered for Ku band operation. Option 1 indicates that some companies have a preference for replicating in the Ku band the simplified mobility scenario created for FR2-NTN operation, which consists in a “directional lock” between UE and satellite and the assumption that UEs are incapable of measuring neighbor satellites while connected to the main serving satellite. This is reflected, in the “only blind HO” proposals.

Although we believe inter-satellite measurements are an integral part of an efficient NTN system, we understand that in Release 18, for the sake of time and simplification, this feature was not added as part of the UE capabilities. Mantaining such limitation for VSAT UEs in later releases is a compromise that aims smaller changes at the cost of less efficient mobility features.

If no modification is done to the VSAT speficications, VSAT UEs will only be capable of blind inter satellite HO.

However, it is our understanding in this WI that some companies believe that the soft satellite switching is necessary for the evolution of the feature to the market. A UE that is capable of performing soft satellite switching shall be capable of measure two different satellites in parallel.

A VSAT UE that is assumed to be capable to perform soft satellite switching shall also be capable of performing inter satellite measurements for HO.

Based on the discussion above we propose:

Introduce soft satellite switching requirements for VSAT UEs, as long as VSAT UEs that support this feature do also support non-blind inter-satellite HO.

For hard-satellite switch the interruption time is akin to that observed for FR1 numerologies, assuming the UEs are capable to track the new satellite transmission, there is no need to modify the definition of the Tsearch, Tprocessing, Tmargin and TΔ. And the value of Tsat_beam might be maintained constantly compared to the value used for HO between two FR2-NTN cells. .

For hard-satellite switching, maintain the same values of Tsearch, Tprocessing, Tmargin and TΔ used for FR1-NTN hard-satellite switching.

For hard-satellite switching use the same value for Tsat_beam used for HO between two FR2-NTN Cells executed by a UE with electronically switched beams.

Inter satellite cell reselection requirements

This subject was discussed in the previous meeting, but some companies requested more time to review this issue. We reiterate our point of view regarding this issue, we believe that it could be beneficial for the network whether proper requirements for inter-satellite requirements were put into place.

For cell reselection requirements, they are similar for FR1 and VSAT FR2-NTN, except that no inter-satellite measurements are enabled for VSAT Type of UEs. There is no fundamental reason that precludes VSAT UEs to perform inter-satellite measurements, however, this is a more complex scenario to be analyzed. During Rel-18 WI, when the VSAT requirements were first created for specifications, there was limited time for discussing all the potential scenarios to develop the RRM requirements, and RAN plenary took the decision to limit the scope of NTN applicability.

We believe that, in this WI, specially with the introduction of scenarios where mobile VSAT UEs are connected to LEO, faster reaction times is paramount, and RAN4 has a new chance to create inter-satellite requirements.

RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19

RLM requirements

In the previous meeting, the RLM requirements were discussed as well as possible configurations for UE mesaurements. The following item was left for further discussion:

Issue 1-8: RLM (section 8.1C) Candidate options: - Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. - Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

Lmax is not a parameter that depends only on RAN4 discussions and definitions. According to RAN1 specification, Lmax denotes the number of different SSB positions in a frame and is left for NW implementation which one(s) are to be used for a particular cell. For FR2 numerology (and as a consequence for FR2-NTN numerology) this number is equal to 64. If RAN4 defines Lmax to be equal to 8, the follow up question would be which 8 out of the original 64 are to be considered valid, and that may create an unnecessary discussion that seems to provide no improvement for the UEs performance or efficiency. Besides, for other VSAT UEs (in the Ka band) the value of Lmax is already considered to be 64. Therefore, we propose to maintain the definitions of Lmax.

LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.

Conclusion

In this paper we provided our views on the NTN operation in the KU band. We focused our discussion in open issues of the previous WF, and based on our analysis we provided the following list of proposals:

Observation 1: If no modification is done to the VSAT speficications, VSAT UEs will only be capable of blind inter satellite HO.

Observation 2: A VSAT UE that is assumed to be capable to perform soft satellite switching shall also be capable of performing inter satellite measurements for HO.

Proposal 1: Introduce soft satellite switching requirements for VSAT UEs, as long as VSAT UEs that support this feature do also support non-blind inter-satellite HO.

Proposal 2: For hard-satellite switching, maintain the same values of Tsearch, Tprocessing, Tmargin and TΔ used for FR1-NTN hard-satellite switching.

Proposal 3: For hard-satellite switching use the same value for Tsat_beam used for HO between two FR2-NTN Cells executed by a UE with electronically switched beams.

Proposal 4: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19

Proposal 5: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology.

- References

- R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, Apple, 3GPP TSG-RAN WG4 Meeting #114-bis