3GPP TSG-RAN WG4 Meeting #116	R4-2511279

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On test cases for CB-MSG3 in TS 36.133

Agenda item:	7.28.5

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) to introduce the contention-based message 3 (CB-MSG3), where the UE skips the Msg1 and Msg2 in the Random Access procedure, for faster data dump. This was indicated by RAN2 in their LS to RAN4 (R2-2405769).

RAN4 has decided to include this new feature in RAN4 specifications, such that the correct UE behavior is captured when skipping RACH Msg1/2. In this document we discuss the inclusion of the test cases for this feature.

Discussion

RAN2 has agreed upon the introduction of CB-MSG3 and has reached several agreements related to this feature and RAN4 has agreed to capture in specification the right behavior of the UE. Following up with the work, RAN4 has decided to include performance evaluation of the feature, by introducing test case(s). The following agreement was made in the previous meeting:

Agreement: - Define test cases for random access for CB-msg3-EDT, and the correct behaviour and transmit timing can be verified in the same test case. - Further discuss whether to down-select the test cases: Random Access (A.13.3.2); A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT. A.13.3.2.X2 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage for CB-msg3-EDT. A.13.3.2.X3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage for CB-msg3-EDT.

From our point of view, it is mostly necessary to check whether the UE presents the right behaviour, by skipping Msg1/2. Whether the UE is capable of selecting enhanced or normal coverage modes should be implicit by the UE succeeding on the previous RACH test cases (where Msg1/2 is not skipped). Likewise, the non-anchor carrier RACH operation is also tested by mean of legacy tests. That said, in order to minimize the scope of test cases, we propose to downselect only one of the proposed list of test cases.

Include one test case for UEs in standalone mode in normal coverage for CB-MSG3-EDT.

Conclusion

In this paper we analyzed the need for updates on the RAN4 requirements to introduce performance requirements related to CB-MSG3 feature and the following proposal was made:

Proposal 1: Include one test case for UEs in standalone mode in normal coverage for CB-MSG3-EDT.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”,

- R1-2407548 “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0