# 3GPP TSG-RAN WG4 Meeting #116	R4-2511285

# Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	7.21.3.2

Document for:	Discussion

1 Introduction

During RAN4#115, BS RF requirements for SBFD have been discussed [13]. In the contribution we provide discussion on existing TX requirements.

2 Discussion

2.1 Summary of the status on the TX requirements

Per WF in [12], the following way forward was reached.

  - Issue 1-1: Whether SBFD-SBFD co-ex case should be in Rel-19 scope

- Way forward

  - RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

  - Issue 1-2: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

- Way forward

  - The testing process should be based on a well-defined test model, in which flexible symbols used for guard period and uplink symbols are not included.

The status on existing TX RF requirements is summarized in the following table.

| ## Requirements | ## Agreements | ## Completed or open | ## Still open for single operator case? | ## Still open for inter-operator case? |
| --- | --- | --- | --- | --- |
| ## Base Station output power and radiated transmit power | ## R4-2409958 o PSD scaling for normal and SBFD slots/symbols: § No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols § Vendors can declare different TX power values for normal and SBFD slots/symbols | ## Completed | ## N | ## N |
| ## Output power dynamics | ## R4-2413515 • Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement. R4-2419908 l The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. | ## Completed | ## N | ## N |
| ## Transmit ON/OFF power | ## R4-2413515 • Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol. | ## Completed | ## N | ## N |
| ## Transmitted signal quality | ## R4-2416585 • Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. • Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. R4-2502380 Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity. R4-2508713 • The testing process should be based on a well-defined test model, in which flexible symbols used for guard period and uplink symbols are not included. | ## Completed | ## N | ## N |
| ## BS OBW | ## Rel-18 SI conclusion in 38.858 the existing OBW requirement shall be applied for the whole BS channel bandwidth in SBFD symbols/slots instead of DL sub-band. | ## Completed | ## N | ## N |
| ## ACLR requirement | ## R4-2413515 • When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. | ## Completed for Rel-19 | ## N | ## N for Rel-19 |
| ## OBUE requirement | ## R4-2413515 • When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. R4-2508713 • RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20. | ## Completed for Rel-19 | ## N | ## N for Rel-19 |
| ## transmitter spurious emission | ## Rel-18 SI conclusion in 38.858 all the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols/slots. The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation. | ## Completed | ## N | ## N |
| ## inter-band co-location and co-existence requirement | ## Rel-18 SI conclusion in 38.858 it was agreed not to update on existing inter-band co-location requirements and manufacturer will declare whether support co-location requirements in SBFD symbols/slots. | ## Completed | ## N | ## N |
| ## Transmitter intermodulation | ## R4-2409958 The transmitter intermodulation requirement is applicable in SBFD slots/symbols: § Follow the existing requirement § No receiver requirement is specified. | ## Completed | ## N | ## N |

It can be found that for existing TX requirements, the requirements are completed for Rel-19 and no open issue is left.

Observation: for existing TX requirements, the requirements are completed for Rel-19 and no open issue is left.

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD.

Observation: for existing TX requirements, the requirements are completed for Rel-19 and no open issue is left.

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2419244, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2501856, On modification of existing RX RF requirements for SBFD

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

- R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

[13] R4-2507683, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon