3GPP TSG-RAN WG4 Meeting #116 	R4-2511286

# 2025 Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	7.21.3.3

Document for:	Approval

1 Introduction

During RAN4#115, BS RF requirements for SBFD have been discussed [12]. In this contribution, we would like to share our views regarding the open issues.

2 Discussion

2.1 Summary of the status on the RX requirements

The status on existing RX RF requirements is summarized as in the following table.

| Requirements | Agreements | Complete or open | Still open for inter-operator case in Rel-19? |
| --- | --- | --- | --- |
| Reference sensitivity level and OTA sensitivity | R4-2419908 l 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity. | Completed | N |
| Dynamic range | n R4-2502380 The dynamic range requirements of FR1 can reuse existing requirement. n For FR2, no dynamic range requirement is needed. | Complete | N |
| ACS | R4-2416585 • The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. | Open | Y |
| In-band blocking | R4-2413515 Agreement: • On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. • RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. • For co-existence study, n The reference point for the power level which should be before array gain R4-2416585 • For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. | Open | Y |
| Out-of-band blocking | Rel-18 SI conclusion in 38.858 the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account. | Complete | N |
| Receiver spurious emissions | Rel-18 SI conclusion in 38.858 apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots. | Complete | N |
| Receiver intermodulation |  | Open | Y |
| In-channel selectivity |  | Open | Y |

2.2 In-band blocking for FR1 WA BS

Per WF in [12],

## Issue 2-2: In-band blocking for FR1 WA BS - Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered

For Rel-19, it was agreed to not to introduce the specific requirements for SBFD-SBFD co-ex case which can be considered in Rel-20, and for SBFD-TDD adjacent operator scenario it was agreed to take the range [-28, -16.7] as starting point which are coming from the co-existence simulation based on 100% grid shift.

Table x: FR1 Blocking level(dBm) @99% CDF

| Scenario | Grid shift | ZTE | Huawei | Samsung | Ericsson case1 | Ericsson case2 | Nokia | CATT | Tejas Networks[baseline] | Tejas Networks[Reduced blocker] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100% | -25.3301 | -25.8 | -27.76 | -23.4 | -20.4 | -16.7 | -25.4 | -28.05 | -62.38 |

The average value -25.2 dBm is obtained after the maximum and minimum values are removed. It is about 18 dB higher than existing in-band blocking limits (-43 dBm). And considering that the actual ACLR/emission could only be several dB better than that in the specification. -25 dBm blocking is already marginal.

Proposal 1-1: It is proposed to adopt -25 dBm as the in-band blocking level for Macro SBFD-TDD adjacent operator scenario. A 6dB offset from OTA reference sensitivity for SBFD is used for the wanted signal.

For interference signal CBW and interference signal frequency, it is proposed to reuse existing definition for BS channel bandwidth larger than 20 MHz

    - Interference signal CBW: 20 MHz OFDM NR signal

    - Interference signal frequency offset: +/- 30 MHz

      - For lowest/highest carrier is configured in DU/UD SBFD operation, the interference applies adjacent to the DL subband.

      - Otherwise +/- 30 MHz

  - For requirements assumption as deployment guidance, FFS on the following options

    - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive

    - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values

    - Option 3: One note can be added to generally describe the deployment restriction.

Regarding to the requirements assumption to be included as deployment guidance, we propose to adopt Option 1. From the baseline assumption, BS transmitter power is 49 dBm, so the coupling loss assumption can be set as 74 dB. In addition,  the interfering channel bandwidth is 20 MHz, 49 dBm is more suitable.

Proposal 1-2: it is proposed to include the following requirements assumption in the specification.

    - For FR1 WA BS, the in-band blocking requirements assume a 74 dB coupling loss between interfering NR BS transmitter and NR SBFD BS receiver

.3 In-band blocking for FR1 Micro BS

Per WF in [12],

- Issue 2-4: In-band blocking for FR1 Micro BS - Way forward - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point - Companies to check the blocking level in the range of [-20, -32] dBm

It was agreed to use the same grid shift assumption as FR1 Macro BS as starting point and it was agreed to check the range [-20, -32] dBm.

Table x: FR1 Blocking level(dBm) @99% CDF

| Scenario | Grid shift | ZTE | Ericsson case1 | Ericsson case2 | Nokia | CATT | Tejas Networks[baseline] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 100% | -38.9889 | -39.4 | -31.5 | -24.6 | -32.1 | -32.30 |

The average value -35 dBm is obtained after the maximum and minimum values are removed. It is already lower that the lower bound -32 dBm. And it is about 6 dB higher than existing in-band blocking limits (-38 dBm) and should be feasible for implementation.

Proposal 2: it is proposed to adopt -32 dBm as the in-band blocking level for Micro SBFD-TDD adjacent operator scenario. And a 6dB offset from OTA reference sensitivity for SBFD is used for the wanted signal.

2.4 In-band blocking for FR2-1 WA BS

Per WF in [12],

WF:

- Encourage companies to have offline discussion for the simulation assumptions before next meeting.

In last meeting there are two options under discussion.

  - Option 1 (Nokia, Samsung, ZTE, CATT): The current FR2-1 WA BS in-band blocking requirements can be re-used

  - Option 2 (Ericsson): Define in-band blocking interferer signal power to -23 dBm

For FR1 the in-band blocking level has been analysis on a statistical basis based on the 99% probability of an interferer being possible. For FR2, the probability of the blocking signal level and also the probability of simultaneous low wanted signal and high blocking signal were both considered. And the requirements are defined as relative value. In addition, there is no high-performance filter can be implemented in the FR2 AAS architecture. Hence it is proposed to adopt option 1 for FR2-1 WA BS.

Proposal 3: The current FR2-1 WA BS in-band blocking requirements can be re-used

2.5 ACS requirement

Per WF in [6],

WF:

- FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

For ACS and in-band blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. ACS and in-band blocking are defined with different interferer frequency offsets at adjacent channel(s). And value of the interferer level is 9 dB offset. This 9 dB comes from the difference of the metric used in the simulation. ACS is derived from the ACIR simulation and blocking results from CDF curve. And the 9 dB applies for all BS types including FR1 and FR2. Hence it is proposed to keep it also for SBFD.

Meanwhile we reran the RAN4 co-ex simulation and find that If the grid shift is set to 50%, around 16 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

| 100% grid shift UL throughput loss |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile | 42.5 (baseline) | 45 | 47.5 | 50 | 52.5 | 55 | 57.5 |
| @5% (cell-edge) | 100% | 100% | 100% | 11.35% | 8.75% | 5.15% | 2.91% |
| @50% (cell-average) | 15.41% | 11.30% | 8.32% | 6.12% | 4.55% | 3.19% | 1.87% |

The existing ACS blocking is -52 dBm, 13 dB improvement means require -39 dBm which is lower than proposed blocking level by 14 dB, and it is even lower than the proposed in-band blocking level (-31 dBm) by 8 dB. From this angle, the ACS is not needed.

Proposal 4: it is proposed to discuss and decide on the following options.

Option 1: The interference level keeps 9 dB offset to the general in-band blocking interference level, i.e. -34 dBm dBm for FR1 Macro, and -41 dBm for FR1 Micro

Option 2: no need to define ACS for SBFD

2.6 Receiver intermodulation

Per WF in [12],

- Issue 2-5: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal - Way forward - Down-select to one of the below two options in the next meeting - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case - Interfering signal 1: CW - Interfering signal 2: TBD MHz NR - Power level: the final agreed in-band blocking level -9 dB

For general in-band blocking case as discussed in clause 2.2 and 2.4, the stronger intermodulation product from closer in-band blocking blocker and self sub-band DL TX is already covered. For example, the interfering signal for existing intermodulation is -52 dBm, while the in-band blocking blocker/ACS blocker is -25 dBm/ -33 dBm, and the self-interference from DL sub-band is ~ -27~-37 dBm. Below figure shows the ACS test as an example. The power level for either in-band blocking or self-interference are much higher than existing -52 dBm. And both interferences are on during the in-band blocking /ACS test. Hence there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s).

Proposal 5: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case

2.7 In-channel selectivity

Per WF in [12],

WF:

  - Check whether existing ICS requirements can be reused

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density, within the UL channel bandwidth. For SBFD, it was agreed to adopt the already defined channel bandwidth for UL sub-band. Hence the ICS requirement within the sub-band can reuse the existing ICS requirements.

Proposal 6: Existing ICS requirements is reused for SBFD.

3 Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Proposal 1-1: It is proposed to adopt -25 dBm as the in-band blocking level for Macro SBFD-TDD adjacent operator scenario. A 6dB offset from OTA reference sensitivity for SBFD is used for the wanted signal.

    - Interference signal CBW: 20 MHz DFT-s-OFDM NR signal

    - Interference signal frequency:

      - For lowest/highest carrier is configured in DU/UD SBFD operation, the interference applies adjacent to the DL subband.

Proposal 1-2: it is proposed to include the following requirements assumption in the specification.

    - For FR1 WA BS, the in-band blocking requirements assume a 74 dB coupling loss between interfering NR BS transmitter and NR SBFD BS receiver

Proposal 2: it is proposed to adopt -32 dBm as the in-band blocking level for Micro SBFD-TDD adjacent operator scenario. And a 6dB offset from OTA reference sensitivity for SBFD is used for the wanted signal.

Proposal 3: The current FR2-1 WA BS in-band blocking requirements can be re-used

Proposal 4: it is proposed to discuss and decide on the following options.

Option 1: The interference level keeps 9 dB offset to the general in-band blocking interference level, i.e. -34 dBm dBm for FR1 Macro, and -41 dBm for FR1 Micro

Option 2: no need to define ACS for SBFD

Proposal 5: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case

Proposal 6: Existing ICS requirements is reused for SBFD.

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958, Way Forward for [111][313] NR_duplex_evo,	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416528, Topic summary for [112bis][307] NR_duplex_evo_BSRF, Moderator (Huawei)

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2419245, On modification of existing RX RF requirements for SBFD, Huawei

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung

- R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2507683, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon