3GPP TSG-RAN WG4 Meeting #116 	R4-2511289

Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	Further discussion on RF requirements for CW

Agenda Item:	7.22.3.3

Document for:	Approval

1 Introduction

At last RAN4#115 meeting, the CW RF requirements have been discussed and agreements can be found in [2].

In this paper we provide further discussion and proposals for open issues.

2 Discussion

Per Study Item discussion, output power, frequency error, unwanted emission including spurious emission are potentially needed. Some progress was made at RAN4#115, the progress is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not (outcome of SI) | WF in R4-2502859 at RAN4#114 | R4-2505097 at RAN4#114bis | R4-2508101 at RAN4#115 |
| --- | --- | --- | --- | --- |
| Output power | YES | CW output power [33] dBm | CW output power <=33dBm, and can be declared by manufacture. |  |
| ON/OFF power | TBD in WI phase | FFS for OFF power | Define CW OFF power Value TBD | OFF power is -50dBm, MBW is FFS |
| Frequency error | YES | Reuse Micro BS frequency error ±0.1 ppm for A-IoT CW |  |  |
| Phase noise | No for inside topology TBD in WI phase for outside topology | Further discuss phase noise requirements | FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI. | o For BS with phase noise cancellation capability, no phase noise requirement is specified. o For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS. |
| Unwanted emission | YES | Specify emission requirements, values FFS | FFS on unwanted emission boundary and corresponding values: |  |
| Spurious emissions | YES | NR spurious emission requirement can be considered as baseline for R19 AIoT CW node | FFS on spurious emissions boundary and corresponding values | 1. The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz 2. General BS spurious emission limits are reused. |
| Intermodulation | TBD in WI phase | FFS |  | No need |

Issue 3-1: CW Output power

- Agreement @RAN4#114bis:

  - CW output power <=33dBm, and can be declared by manufacture.

The text proposal for TS 38.914 can be drafted based on the agreements. The text proposal can be found in companion paper in [x]

Issue 3-2-1: ON/OFF power

- Agreement:

  - OFF power is -50dBm, MBW is FFS

-50 dBm OFF power was agreed and the measurement bandwidth is FFS. The transmitter is considered OFF when the CW is not allowed to transmit. The NR UE transmit OFF power is -50 dBm per channel bandwidth. If channel bandwidth for CW is defined, we can reuse the UE transmit OFF requirement, otherwise measurement bandwidth can be set as 1MHz as NR UE co-existence requirement. CW is single tone signal which may not be suitable to define channel bandwidth. Hence our preference is that the CW OFF power is defined as -50 dBm/MHz

Proposal 1: the CW OFF power is -50 dBm/MHz.

Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

Based on the technical analysis on Phase Noise requirements in SI phase, for inside topology, no explicit phase noise requirement is required. Rationale behind is that the internal CW signal enables convenient single tone signal and phase noise cancellation through calibration mechanisms, eliminating the need for stringent phase noise requirements. And for outside topology, phase noise requirements become solution-dependent and must align with system implementation. For example, if direct RF calibration cable exist between the CW node and Base Station node for real-time cancellation, phase noise impacts on uplink demodulation are significantly mitigated, which is the same situation as the case for inside topology. In such an implementation, there is no need to define phase noise requirements.

Observation 1: phase noise requirements become solution-dependent

Figure 2-1: example block diagram of the A-IoT indoor coverage solution

Some examples are discussed in TR 38.769 [4]. In the case of separated carrier-wave transmitter and D2R receiver in D1T1-B, some discussion considering noise limited scenario was presented in previous meetings. In the real indoor scenario, the received wanted power is normally higher than the noise floor (-122.2dBm/15kHz). From the simulation in SI, the minimum received wanted D2R power is about -110 dBm when CW is transmitting at 23 dBm. If CW is transmitting at 33 dBm, the minimum received D2R power will be -100 dBm. In [], it is proposed -96 dBm/15 KHz as minimum sensitivity limits for 1 Rx. Assuming the residual power of CW power at reader is -38 dBm, the phase noise impact to the SINR can be found in Table 2-2.

Table 2-2: the phase noise impact to the SINR

| Residual integral phase noise(dBc) | Wanted signal | Noise floor （dBm） | phase noise interference level | I+N | SINR |
| --- | --- | --- | --- | --- | --- |
| 50 | -96 | -122 | -88 | -88 | -8 |
| 55 | -96 | -122 | -93 | -93 | -3 |
| 60 | -96 | -122 | -98 | -98 | 2 |
| 65 | -96 | -122 | -103 | -102.9 | 6.9 |
| 70 | -96 | -122 | -108 | -107.8 | 11.8 |

It can be found that 55 dBc residual integral phase noise (DSB) can fulfill the required SINR with some margin.

Table 2-3: CW phase noise limit

| Δf (kHz) | Phase noise emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 60 | [-25] | [7.5 kHz] |
| 120 | [-28] | [7.5 kHz] |

In Rel-19 for D1T1, it is likely the CW node and BS node belongs to the same vendor, and phase noise cancellation will be implemented, which can be considered as a whole solution. In last meeting we agreed that for BS with phase noise cancellation capability no phase noise requirement is specified. If there is no agreement reached in RAN4#116, it is proposed that for Rel-19 phase noise is not specified for D1T1.

Proposal 2: it is proposed to discuss the following two options and make a decision at RAN4#116

Option 1: For BS without phase noise cancellation capability, it is proposed to define the CW phase noise limit as in Table 2-3.

Option 2: If there is no agreement reached in RAN4#116, for Rel-19 phase noise is not specified for D1T1

Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

CW node transmits singe tone signal. Except the spurs in the discrete spectrum, the emission in the frequency range immediately outside the CW is phase noise dominate. Table 2.2-1 show one example phase noise performance with carrier frequency 900 MHz. Assuming the CW occupies one PRB, the integrate noise phase at the adjacent PRB is less than -25 dBm/30 kHz, which is already close to the GSM MS emission level (-27 dBm/30 kHz) at 400 kHz offset.

Table 2.2-1: example phase noise performance @ 900 MHz

| Offset | Phase Noise |
| --- | --- |
| 300Hz | -78.5dBc/Hz |
| 500Hz | -83.2dBc/Hz |
| 1kHz | -91dBc/Hz |
| 10kHz | -101dBc/Hz |
| 100kHz | -103dBc/Hz |
| 1MHz | -115dBc/Hz |

Table 2.2-2: GSM emission mask

If referring to GSM emission mask, and the Δf (foffset) for CW is defined from the center of the CW transmission, and the CW max transmission power is 33 dBm, the spectrum emission mask the emission mask is translating as in defined in Table 2.2-3.

Table 2.2-3: GSM-like spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 100 | N/A |  |
| 200 | 3 | 30 kHz |
| 250 | 0 | 30 kHz |
| 400 | -27 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

And if referring to NB-IOT BS mask as shown below, changing the definition of Δf (foffset) for CW and using CW transmission power equals to 33 dBm, the spectrum emission mask can be found in Table 2.2-3. It can be found the NB-IOT BS mask are more stringent than the GSM like mask in most frequency offsets.

Table 6.6.3.2H-1: Standalone NB-IoT medium range BS operating band unwanted emission limits, BS maximum output power 31 < Prated,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2, 3) | Measurement bandwidth (Note 8) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz (Note 1) | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 6) | 1.015MHz f_offset < 1.5 MHz | Prated,c - 65 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Prated,c - 52 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Prated,c - 52 dB, -15dBm) | 1 MHz |
| 5 MHz f fmax | 5.5 MHz f_offset < f_offsetmax | Prated,c - 56 dB | 1 MHz |
| NOTE 1: The limits in this table only apply for operation with a standalone NB-IoT carrier adjacent to the Base Station RF Bandwidth edge. NOTE 2: For BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 3: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 2.2-3: NB-IOT BS spectrum emission mask: Prated,c= 33 dBm

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -5 | 30 kHz |
| 250 | -8 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~fmax | -23 | 1 MHz |

Based on the discussion above, we propose to take NB-IOT BS mask as baseline. Furthermore, according the close-in phase noise capability, the limits at  200 kHz and  250 kHz can be set as -18 dBm and -20 dBm respectively.

Proposal 3：It is proposed to adopt the following proposal for CW node.

The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask applies from 200 kHz to fmax. The spectrum emission mask for CW node is defined as in Table 2.2-3

Table 2.2-3: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~fmax | -23 | 1 MHz |

Issue 3-x: Frequency entries

On R2D raster, it was agreed to define 10 KHz channel raster to align with NR enhanced channel raster. For CW transmit frequency entries it is proposed to reuse the NR entries for enhanced channel raster

Table  2.2-4: Applicable NR-ARFCN for n8 for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |

Proposal 4：It is proposed to reuse the NR enhanced channel raster for CW transmit frequency entries.

3 Conclusion

In this paper we provide discussion and proposals for CW RF requirements.

Proposal 1: the CW OFF power is -50 dBm/MHz.

Proposal 2: it is proposed to discuss the following two options and make a decision at RAN4#116

Option 1: For BS without phase noise cancellation capability, it is proposed to define the CW phase noise limit as in Table 2-3.

Table 2-3: CW phase noise limit

| Δf (kHz) | Phase noise emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 60 | [-25] | [7.5 kHz] |
| 120 | [-28] | [7.5 kHz] |

Option 2: If there is no agreement reached in RAN4#116, for Rel-19 phase noise is not specified for D1T1

Proposal 3：It is proposed to adopt the following proposal for CW node as in Table 2.2-3.

Table 2.2-3: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~fmax | -23 | 1 MHz |

Proposal 4：It is proposed to reuse the NR enhanced channel raster for CW transmit frequency entries.

References

[1]	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2]	R4-2508101, WF on requirements for A-IoT BS and CW, Moderator (Huawei)

[3] PLL Performance, Simulation, and Design

[4] TR 38.769

8	A-IoT CW transmitter characteristics

8.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the CW node with a single or multiple transmit antenna(s). The CW waveform for D2R backscattering is a single-tone unmodulated sinusoid.

8.2	CW Output power

### 8.2.1	General

Output power of the CW node is the mean power of the single-tone signal delivered to a load with resistance equal to the nominal load impedance of the transmitter.

Rated total output power (Prated) of the CW node is the mean power of the single-tone signal that the manufacturer has declared to be available at the antenna connector during the transmitter ON period. The rated output power, Prated,, of the CW node shall be than or equal to +33 dBm.

Maximum output power (Pmax) of the base station is the mean power level of the single-tone signal measured at the antenna connector during the transmitter ON period in a specified reference condition.

### 8.2.1	Minimum requirement

In normal conditions, the CW maximum output power, Pmax, shall remain within +2 dB and -2 dB of the rated output power, Prated, declared by the manufacturer.

In extreme conditions, the base station maximum output power, Pmax, shall remain within +2.5 dB and -2.5 dB of the rated output power, Prated, declared by the manufacturer.

In certain regions, the minimum requirement for normal conditions may apply also for some conditions outside the range of conditions defined as normal.

8.3	Frequency error

### 8.3.1	General

The requirements in clause 8.3 apply to the transmitter ON period.

Frequency error is the measure of the difference between the actual CW transmit frequency and the assigned frequency.

### 8.2.1	Minimum requirement

The frequency error requirement of CW node shall be accurate to within ±0.1 ppm

8.4	Transmit OFF power

### 8.4.1	General

Transmit OFF power is defined as the mean power when the transmitter is OFF.

### 8.4.1	Minimum requirement

The transmit OFF power shall not exceed -50 dBm/MHz

8.5	Unwanted emission

8.5.1	General

Unwanted emissions consist of operating band unwanted emissions and spurious emissions.

The maximum offset of the operating band unwanted emissions mask from the operating band edge is 10 MHz. The Operating band unwanted emissions define all unwanted emissions in each supported uplink operating band plus the frequency ranges 10 MHz above and 10 MHz below each band. Unwanted emissions outside of this frequency range are limited by a spurious emissions requirement.

Unless otherwise stated, all requirements are measured as mean power (RMS).

### 8.5.2	Operating band unwanted emissions

#### 8.5.2.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits in FR1 are defined from 10 MHz below the lowest frequency of each supported uplink operating band up to 10 MHz above the highest frequency of each supported uplink operating band.

Emissions shall not exceed the maximum levels specified in the tables below, where:

-	f is the separation between the assigned transmission frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

#### 8.5.2.2	Minimum requirement

The operating band unwanted emissions shall not exceed the maximum levels specified in the Table 8.5.2.2-1. The spectrum emission limit between each Δf is linearly interpolated.

Table 8.5.2.2-1: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 200 | -18 | 30 kHz |
| 250 | -20 | 30 kHz |
| 350 | -25 | 30 kHz |
| 800 | -26 | 30 kHz |
| 1200 | -19 | 1 MHz |
| 5200~fmax | -23 | 1 MHz |

### 8.5.3	Transmitter spurious emissions

#### 8.5.3.1	General

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from 10MHz below the lowest frequency of each supported uplink operating band, up to 10 MHz above the highest frequency of each supported uplink operating band.

#### 8.5.3.2	Minimum requirement

The power of any spurious emission shall not exceed the limits in Table 8.5.3.2-1.

Table 8.5.3.2-1: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |